VTS19/Ops/WP5

## Draft IALA Guidelines on Symbology

Recommendations on the Integration and Display of AIS and other information in a VTS Centre are set out in the IALA Recommendation V125 (June 2003)- Recommendation on the Integration and Display of AIS and other Information at a VTS Centre.

V-125 recommends that the onboard standard for AIS and general symbology should be used as the basis from which shore based VTS symbology requirements may be developed but recognises that there may be a need to adapt symbology for their own more detailed needs. It further identifies that any divergence from the on-board   symbology should be carefully considered and justified, and that symbology already identified for existing on- board use should not be assigned a different meaning.

This guideline identifies some of the limitations of AIS and general symbology and the issues that VTS Authorities may wish to consider in deciding to adapt standard symbology. Whilst the recommendation V-125 allows VTS Centres to adapt symbology to meet their own needs, it would be advantageous if variations in presentation are minimised. Where appropriate, therefore, this guideline identifies areas of commonality that may be considered.

# AIS Symbology Limitations

# S52 Symbology Limitations

The standards laid down in S52 for presentation of geographical information is designed for use on-board a vessel on an Electronic Chart and Information Display System (ECDIS) for use in navigation. Examples of these symbols are attached at Annex 1 and Annex 2.

The S52 standard may be limited and therefore reduce some of the flexibility needed in a VTS Centre. For example, there may be a requirement to differentiate between the type of vessel or its cargo  or whether it carries a pilot through the use of variations in colour, shape or other attributes.

Local conditions and national regulations can lead to a need for a larger flexibility.

The capability of a VTS Operator to carry out a particular task can be reduced if the symbology available does not correspond to the requirement and therefore present the VTS Operator with a suitable tool to enable the task to be completed satisfactorily.

For example the standard scale on the symbology may be in conflict with the scale on the map or/and the need for use of zooming a picture. The size of an AIS target symbol may cause difficulties in areas of high traffic density due to the shear number and size of these targets. However, without specialist software there is no means of suppressing the constant display of these targets.

In the S52 standard the presentation of map (land and sea) is restricted in terms of the choice of colours available for differentiating between land and sea, as well as in the choice of colours available for the combination of sea and land. VTS Operators may prefer a dark background with lighter colour symbols. However, in the standard there is a lack of contrast between the various backgrounds.

The human/machine interface in the VTS environment is different that that required for navigational purposes on- board and therefore may also result in different needs. For example, the VTS Operator is working in an “office type” environment where there is a need to look at a computer screen for many hours in the watch. The colours available in the S52 standard have not necessarily been designed with this in mind. The ability to change the colours on the screen can considerably help to reduce VTS Operator fatigue and therefore any signs of visual stress.

Annex 1

Extract from Section A.4.1 page 63 of the s52 Standard

Point Symbols & Centred area symbols (SY)

Annex 2

Extract from Section A.6.1 page 85 of the s52 Standard

Day- Bright