VTS and the ISPS Code

- Council at its meeting on 25 May charged the VTS Committee with reviewing the need for an IALA Recommendation in respect of the ISPS Code and its potential impact on IALA members.

- This task was undertaken during VTS 21, following which the VTS Committee concluded that the ISPS Code has limited direct impact for VTS authorities and centres except in the following ancillary areas:

        - The Code addresses the security arrangements onboard vessels and at “port facilities”.  The action required to be taken at the latter is decided by means of a security assessment to be conducted by Contracting Governments.  Where relevant, security measures may require that the “approaches, anchorages, manoeuvring and berthing areas” be monitored. This may be achieved in a variety of methods, including the use of VTS sensors and data.  Where the surveillance capabilities of a VTS are used to perform this monitoring function, it should not be considered a core VTS function, but an allied service for security purposes.  Before accepting that a VTS centre is capable of fulfilling such a role, careful consideration will need to be given to the capacity of VTS staff to accommodate security functions without detriment to the safety of shipping, as well as to the additional training that might be required.

        - Whilst the Code does not require VTS authorities to integrate AIS into their systems, it does advance the date by which vessels over 300 tonnes are required to be fitted with AIS to 31 December 2004.  Thus if the incorporation of AIS into a VTS is considered necessary, be it for safety, efficiency or security purposes, VTS authorities will wish to note this change in the fitting programme for ships.

        - Where national security arrangements envisage the need to respond to a security incident onboard a vessel within a VTS area, the VTS authority may be required to address in their security plans the co-ordination of marine units involved in any subsequent escort or intervention operation.  It will also need to keep in mind the need to control other vessel traffic in the locality.

        - Where following a security assessment, a VTS centre or remote sensor site are assessed to be important assets or infrastructure requiring protection, additional security enhancements may be required.  These may include site surveillance and the control of personnel and vehicles.

        - VTS will continue to play a pivotal role in the management of any port emergency arising from a security incident or breach.

- In summary, VTS has the potential to contribute significantly to the security posture adopted within a port or VTS area.  This contribution, however, will normally take the form of an allied service for security purposes, and will largely be determined by national security considerations.  It is considered, therefore, that an IALA Recommendation addressing the specific role of VTS authorities in the context of the ISPS Code would be inappropriate.  It is recommended, however, that the ISPS Code and the ancillary considerations that potentially arise for VTS authorities, be addressed in an article in the IALA Bulletin.