# CHAPTER 17:     VTS OPERATIONAL PROCEDURES

## 1701 	Introduction

Operational Procedures are an integral part of a verifiable safety management system for VTS.  A properly implemented quality control system, approved by the competent authority, can ensure that the standards set for the type and level of service are consistently maintained and that the service is delivered safely and effectively.

The development and maintenance of VTS centre specific operational procedures is a continuous process.  To ensure the safe and efficient management of the service it is critical that:

- VTS Staff are made aware of changes and amendments.

- Auditable and documented processes are developed that enable the early and effective update of operational procedures

Best practice indicates that new or changed procedures should be communicated at the Watch Handover and incorporated into the operational procedures handbooks/manual. This chapter summarises the key considerations in developing operational procedures for VTS Centres.

## 1702 	Overview

IALA Recommendation V-127 Operational Procedures for VTS (Edition 1.0 – June 2004) has been prepared to assist VTS authorities in identifying key aspects that should be considered when developing operational procedures for a VTS Centre.

## 1703	 Communications and VTS Procedures

Recommendation V-127 provides a checklist for preparing Operational Procedures and this chapter provides guidance in developing recommended procedures.  To assist a common and consistent approach to the development of Operational Procedures, key terminology utilised by the maritime sector has been defined.  The key terminology includes:

- Result Oriented Messages

- Standard Phrases

- Types of Communication Messages and Message Markers

## 1704	 Result-Oriented Messages

A fundamental principle of VTS communications is that advice and instructions should be “result-oriented” only; leaving the execution to the vessel.  The execution, such as courses to be steered or engine manoeuvres to be ordered, remain the responsibility of the person on board accountable for navigational decision making at that time.

The interpretation of “result-oriented” will depend on the situation and context.  Phrases that are used for vessel conning, such as, “Stop Engine”, “Hard to Starboard” or “Steer Course “XXX”” should not be used.

## 1705	 Standard Phrases

Guidance on maritime communications can be found in:

- Standard phrases for ship-to-shore communications are defined in IMO Resolution A.918(22) Standard Marine Communication Phrases.

- Standard Reporting Procedures, IMO Resolution A.851(20) General Principles for Ship Reporting Systems and Ship Reporting Requirements.

## 1706 	  Types of Communication Messages and Message Markers

To facilitate shore-to-ship and ship-to-shore communication in a VTS environment, one of the following eight message markers should be used to increase the probability of the purpose of the message being properly understood.  It is at the discretion of the shore personnel or the ship's officer whether to use one of the message markers and, if so, which marker is applicable to the situation. If used, the message marker is to be spoken preceding the message or the corresponding part of the message. The contents of all messages directed to a vessel should be clear; IMO Standard Marine Communication Phrases should be used where practicable.

## 1707	  Message Markers

There are eight types of Communication Messages that are frequently used in VTS (Link to SMCP Reference) each with its own discrete message marker.  These are:

| Information | Warning | Advice | Instruction |
| --- | --- | --- | --- |
| Question | Answer | Request | Intention |

- Information: Marker “INFORMATION”

This Marker is used to convey observed or predicted facts and situations; it is generally used for navigational and traffic information. The action taken by the recipient is at their discretion.  Any decision taken onboard using this message could be influenced by additional information

Example 1.	“INFORMATION, The vessel “No Name” predicted to overtake to the west of you in the vicinity of Buoy “XXXX”.

Example 2.	“INFORMATION, Next high water at Port “YY” predicted at “XX:XX” at a height of “ZZ” metres.

which may not be available to the VTS centre.

- Warning:   Marker “ WARNING.”

This Marker is used to convey potentially dangerous situations or observed developing situations. The contents of a Warning Message should be immediately assessed in conjunction with any additional information which may not be available to the VTS centre and corrective action taken where necessary.

The outcome of any action taken based on the warning is the responsibility of the recipient.

Example 1.	WARNING,  Vessel ‘Piero, VTS radar indicates you are heading towards shallow water”.

Example 2.	“WARNING,  Vessel ‘Piero, Information available to the VTS suggests you are dragging your anchor.  Check your condition and confirm your intentions”.

- Advice:   Marker  “ADVICE”

This Marker conveys that the message contains a recommendation that should be considered by the recipient in making navigation related decisions.

Advice Messages must not contain extraneous information.  As circumstances warrant, VTS shall update information provided to mariners to ensure that the most recent information is available on which to base navigation related decisions.

The duty VTSO should use their professional judgement and all available information in deciding when interaction to provide an Advice Message is appropriate.  When interaction to

Example 1.	“ADVICE, Motor Vessel Nonesuch, the Large Container Vessel ‘LCV Europa’, has passed the “x” Buoy inward bound and will be taking her tugs at “y” location.  Wide berth requested”.

Example 2.	“ADVICE, By my display you are leaving the channel.  Recommend you transit within the channel”.

provide an Advice Message is appropriate, the message should be concise and relevant.

- Instruction : Marker “INSTRUCTION”

This Marker conveys that the message is a direction given by the VTS centre under the provisions of a statutory regulation.

The sender must have delegated authority to send such a message. The recipient has a legal obligation to comply with this message unless contradictory safety reasons exist, which then have to be reported immediately to the sender.

Generally masters of vessels will respond promptly and carry out instructions given by a VTS.  However, it should be recognised that there may be occasions when an instruction by a VTS is disregarded because the master has additional information not available to the VTS Centre and he decides on another course of action.  For example, a vessel not visible to the VTS may be a contributing factor to the navigational situation.

Example 1.	"INSTRUCTION. Vessel “No Name” Do not cross the fairway."

Example 2.	“INSTRUCTION. The large Container Vessel ‘LCV Europa’, is experiencing some difficulty swinging in the fairway off your berth.  Do not leave your berth without further instructions.”

- Query : Marker “QUESTION”

This Marker indicates that the following message is of an interrogative character and calls for a reply.  The use of this marker removes any doubt as to whether a question is being asked or a statement is being made, especially when interrogatives such as what, where, why, who, how are additionally used at the beginning of the question. The recipient is expected to reply with an answer.

Example.		"QUESTION. Vessel “Tatsua Maru(What is) your present maximum draft?"

- Response: Marker “ANSWER”

This Marker indicates that the following message is the reply to a previous question.  It should be noted that an answer should not contain another question.

Example.			"ANSWER. The present visibility is XXXX metres."

- Request for Action: Marker “REQUEST”

This Marker indicates that the following message is asking for action from others with respect to the vessel. The use of this marker is intended to signal: “I want something to be arranged or provided.”  For example, ship’s stores requirements, tugs, permission to proceed.

Example.			"REQUEST. Please stand by on this channel."

- Statement of Intent: Marker “INTENTION”

This Marker indicates that the following message informs others about immediate navigational action intended to be taken. The use of this message marker is logically restricted to messages announcing navigational actions by the vessel sending this message.

Example.	"INTENTION. I will reduce my speed."

## 1708	Developing Operational Procedures

Operational procedures should be drawn together into an Operations Manual available to all VTS Staff.  Consideration should be given to including information relating to Policy, Organisation and Administration, linked to Chapter 14, to provide guidance for VTSOs in appreciating their role within the agency providing the service either as a part of the Operations Manual or as a stand alone linked document.

The Recommendation V-127 provides a reference list to assist VTS authorities to identify the key requirements that should be considered when developing operational procedures. The list is neither mandatory nor exhaustive and should be adapted to suit individual needs.

The Recommendation V-127 recognises that:

- The nature of the tasks and activities to be performed will depend on the capability of the VTS, the VTS area and the type and level of services to be provided. In general, these tasks and activities all involve collecting, processing, evaluating and disseminating information. The collection and dissemination of this information will involve both internal and external communications, while information will be processed within the VTS Centre itself.  The level of decision-making that can be taken within the VTS centre should be clearly identified and promulgated.

- The objectives of the VTS can only be met through co-operation and trust among users of the service, VTS personnel and allied services. This can only be achieved through the reliability of the VTS information, which is dependent on the assured availability, continuity and quality of the service provided to all stakeholders.

IALA Recommendation V-127 identifies a distinction between Internal and External Operational Procedures:

Internal Procedures – procedures that cover the day-to-day running of a VTS centre or sub-centre, including the operation of systems and sensors, interactions among the staff and the internal management of data.

External Procedures – procedures that govern the interaction with participating vessels and allied services (defined as services actively involved in the safe and efficient passage of the vessel through the VTS area).

A further distinction is made between routine procedures and those related to incidents such as search and rescue and environmental protection. The latter are generally referred to as emergency procedures.  A summary of the procedures that need to be considered are at Annex A to this chapter.

ANNEX A:  EXAMPLES OF VTS OPERATIONAL PROCEDURES

| Internal Procedures | External Procedures |
| --- | --- |
| Routine Procedures - Gathering and Recording of information - Operational staff - Equipment operation, maintenance, calibration and updating - interaction with allied services - Public Relations - Security - Training - Watch handover - Vessel handover - Maintenance of marine publications | # Routine Procedures Pre-Arrival Information - Vessels Entering VTS Area - Vessels Transiting VTS Area - Vessels at Anchor - Vessels at Berth - Vessels Departing the VTS Area - Transition between Adjacent VTS Areas - Adverse environmental conditions |
| Emergency Procedures - System Failure - Internal emergencies, for example fire and flood - Forced evacuation of VTS centre - Personnel medical emergencies - Security incidents. | Emergency Procedures - Collision, Capsize, Sinking, Grounding, Fire On Vessel, Man Overboard - Pollution - Places of Refuge - Medical Emergency - Vessel Not Under Command (NUC) - Security incident - Protest Action - Natural Disaster |