Criteria for determining type of VTS services (Task 7)

Introduction

There are well-defined procedures and criteria to be used in implementing a VTS contained in the VTS Manual, Rec. V119 and other guidance.  This guidance contains definitions of the different types of services the VTS can provide (NAS, TOS, INS), but they do not contain specific guidance to assist in determining what type of service the specific VTS that is being established should provide.

This working paper addresses VTS Committee work programme Task 7, to develop criteria to assist in making this determination.  It is recognised there is linkage with Task 8 (NAS) [depends on outcome of group working on Task 8].  In working on this task, IALA Recommendation V119 on the Implementation of Vessel Traffic Services was used as starting point (including review of comments from Geoff Eades) to familiarise ourselves with the process for establishing a VTS to identify the appropriate place in that process where the types of service should be determined.   The definitions and functions of the types of VTS services contained in the VTS Manual and IMO A.857(20) were also reviewed to help focus in on specific criteria.

Process to determine whether a VTS is needed

The VTS Manual includes the following steps to be followed in determining the need for and implementing a VTS.  These steps are also included in IALA Recommendation V119 on the Implementation of Vessel Traffic Services.    The description of these steps in V119 is unclear; the same terms are much better defined in the VTS Manual (2008).  V119 should be reviewed and brought into alignment with the VTS Manual.  In the Manual, these steps are listed several times and there are inconsistencies between the listings.  These should be resolved in future review of the Manual.

- Inception (includes Preliminary Risk Assessment) – evaluation of local conditions, hazards and consequences.  Basic mitigation identification – i.e., general type of mitigation (changes to buoyage, TSS, dredging, VTS).  Development of more specific mitigation measures, identification of all possible mitigation measures (brainstorming) that address the identified risks without regard to cost or feasibility.

- Feasibility– narrowing the possible mitigations developed during the inception phase – what is technically possible,  available resources (people, funding, equipment).   This is the point when the type of VTS services to be provided should be determined

- Design – identify functional requirements and equipment technical specifications.

- Formal Risk Assessment – evaluates that the measures designed actually reduce the identified risks.  This will provide feedback to the Design and Feasibility; this may affect the determination of the specific services to be provided by the VTS.

- Cost/Benefit analysis –identify whether the expected reduction in risk is justified in terms of the level of investment required; that the expected cost and effort will return adequate benefits (e.g., “80/20 rule:” 20% of the effort will return 80% of the desired results; 80% effort will be required for the remaining 20%)

- Implementation – final design and execution of the mitigation (VTS)

- Evaluation - At a reasonable time after implementation, follow-on risk assessment should be completed to assess the effectiveness of the VTS.

The VTS Manual places determination of types of service in the inception phase (pg. 75, figure 7.2); we believe it is more appropriately placed in the Feasibility phase.  The determination of type of service follows the preliminary risk assessment in the Inception phase, and will influence the Design phase.  Feasibility considerations will also influence the types of service provided (noted in the specific criteria below).

Criteria:

The following criteria should be considered in determining the proper services to be provided when establishing a VTS.  These criteria should be included in Recommendation V119, the VTS Manual or as a new recommendation.

- Rules for vessel traffic – may apply to certain vessels (such as hazardous materials carriers), during certain environmental conditions – may require specific TOS procedures and mandatory participation in specific services (NAS).

  - Examples:

    - One-way traffic zones for certain vessels

    - Limitations for vessels carrying hazardous cargo during restricted visibility

    - …etc.

- Need for allocation of waterway space or usage of infrastructure (locks, bridge or narrow waterway passage) – may indicate need for TOS

  - Examples:

    - Restricted zone around LNG vessel

    - Lock procession order

    - One-way traffic through bridges for certain size vessels

    - Coordination of vessel traffic with drawbridge openings

- Presence of mariners unfamiliar with local conditions, configuration or hazards or not required to carry a pilot may indicate need for NAS (may be mandatory).   Level/amount of INS may be higher for these vessels.

  - Examples:

    - Ability of the VTS Operator and mariner to communicate clearly

    - Great Belt NAS provided to vessel standing into shallow water

    - Vessels in Strait of Juan De Fuca missing turn in TSS

- Location of VTS:  port, coastal or river – may influence the types of services provided.  More information (INS) needed in a port VTS, more NAS needed in a coastal VTS, more TOS needed in a river VTS.

  - Examples:

    - Coastal:  Assistance to vessels before boarding a pilot

    - Port:  Coordination with port services – berth, tugs, etc.

    - River:  Tidal windows for deep draft vessels; coordination of transits at high water/swift current

- Possible need for additional or higher level/amount of VTS services during special or changed circumstances. (NAS, TOS, INS)

  - Examples:

    - Maritime incident (SAR, casualty, oil spill)

    - Loss of other navigational aids, such as following a storm

    - Higher level of security measures

    - Unanticipated changes to port infrastructure or increase in vessel traffic volume

    - Limitations on vessel manoeuvrability, such as wind effects on vessels with large sail area

- Existence of areas that may affect navigation.  May cause need for INS, NAS or TOS.

  - Examples:

    - Environmentally sensitive areas, such as endangered whale locations (INS)

    - Navigation hazards, such as shallow water or reefs (INS, NAS)

    - Protection of infrastructure, such as bridges (NAS, TOS)

    - Existence of traffic routeing measures, such as TSS, recommended routes, areas to be avoided (INS, NAS)

- Equipment available to the VTS – sensors, communications will limit the specific services the VTS can provide; also services that the VTS will provide will determine the types of equipment that is required.  As new capabilities become available the potential services that may be provided should be assessed.

  - Examples:

    - New types of cameras may assist in providing more information to vessels.

    - Use of AIS has expanded the information available to VTSs and the ability to track vessels outside of radar coverage areas.

- Communications and access to data outside the VTS will determine the extent of INS a VTS can provide.

  - Examples:

    - Port operations information, such as tug arrangements, lock passage planning, berth availability

- Availability of services.

  - Examples:

    - Services may only be available during limited hours

    - Certain services may only be provided in a limited geographic area

    - Services may only be provided to a limited extent

- The authority that the VTS has been granted by local or national regulations and legislation can affect what type and the extent of services the VTS may provide.

  - Examples:

    - A VTS may not be authorised to provide courses or order vessel speeds.

    - The VTS may not have well-developed procedures for providing certain services

- Resources available can affect the services provided

  - Examples:

    - Funding based on risk assessment and cost-benefit analysis

    - Operator capabilities – training, experience, procedures

    - Equipment capabilities, availability such as cameras that can see in various conditions

The following diagram summarizes the criteria and which services they affect:

| Criteria | NAS | TOS | INS |
| --- | --- | --- | --- |
| Rules | X | X |  |
| Waterway allocation |  | X |  |
| Unfamiliar mariners | X |  | X |
| VTS location | X | X | X |
| Special circumstances | X | X | X |
| Areas affecting navigation | X | X | X |
| Available equipment | X | X | X |
| Access to information |  |  | X |
| Availability of services | X | X | X |
| Authority | X | X |  |
| Resources available | X | X | X |

Considerations:

The services provided and the amount of those services provided may vary between VTSs and within a VTS (level/amount of INS, NAS, TOS may vary).  This variance may be geographic or time based.  These variances should be made very clear to the VTS and its users.

Provision of VTS services may develop over time as waterway usage, infrastructure, configuration and other factors change.  These should be assessed through an iterative process of risk assessment to determine the appropriate mitigation measures.  This will require development of new procedures, possible expansion of required equipment and training of VTS operators.  The addition of these services will need to be advertised to waterway users.

Specific services available and rules that must be followed should be made clear to vessels.  Currently there is good knowledge of the rules and requirements that must be followed by vessels.  There needs to be better advertisement of the services available to vessels, including the specific types of services available, and any limitations on these services (e.g., times or geographic areas, limited information sources).

Notes:

- Training and competence of operators – make higher visibility – emphasize more in the criteria.

- Need to determine where this information should be placed – V119, revision to VTS Manual or as a standalone guideline

- Ensure services to be provided, area of coverage, level of services and any limitations by a VTS are made clear to VTS Operators and are included in VTS training.