VTS29/17/2

20 January, 2009

Submitted by the European Maritime Safety Agency (EMSA)

VTS data exchange and sharing

1. Introduction

The need for VTS data exchange should be properly justified and regulated. A mandate has been achieved at the European level through agreed:

- Use of (and agreed interpretation of for data exchange purposes) information currently contained in (IMO-based) international systems of reporting; and

- business rules incorporating a centralised system (SafeSeaNet), with further assurance of performance by the system and compliance by Member States with the performance rules.

Subject to the EU regulatory framework, the range of users of the VTS data have been identified but not limited to a range of shore-based authorities, including the search and rescue authorities, counter-pollution organisations, MAS services, maritime security authorities, port authorities, Port State Control authorities, flag Administrations and what are described as “shore-based authorities” in terms of IMO guidelines.

EMSA recognises the valuable work the VTS Committee has made in the area of exchanging VTS information between shore-based authorities. This paper recommends additional effort, using the experience of SafeSeaNet as a model, including the lessons learnt through the system’s implementation, linking 24 States.

This paper aims at defining the VTS message contents and the request and response mechanism for the exchange of data.

2. The message content

EMSA recalls the submission VTS.28/14/1 in which a number of important questions were raised. The European legislation depends heavily upon global standards as frameworks defining the message contents such as IMO Resolution A.851(20). These are transposed and standardised into discrete messages for electronic exchanges via XML, as was explained in EMSA’s previous paper to IALA at VTS28.

EMSA would like to see the message content defined by IALA not to change substantially from what already applied at EU level. This does not prevent the use of the network for the exchange of other additional information (e.g. weather, tide conditions etc.) between two or more users at national or international level.

3. Use of a centralised system (the hub and spoke model)

Existing protocols for the exchange of information are limited to one-to-one exchange relationships. A fuller exchange of authoritative data on a world or world regional scale will require a centralised system to constantly monitor the quality of the data and the maintenance of the business rules. Central monitoring of the business rules, the identification of users and the access rights to specific message content as and when it is required are integral, essential features and functions. The concept of the hub and spoke is followed by SafeSeaNet and is illustrated below:

Bilateral   exchanges eliminated/ minimised

X

Shore-based Authority (e.g. VTS)

Centralised data Exchange (spoke)

X

X

Central Hub (SSN)

Hub and spoke model

Precautions are taken to protect the system and network from external intrusions, also verifying the identity of users. The appropriateness of the contents of messages and users access rights also ensures that the information notified, reported only reached those groups authorised to do so under the circumstances they are needed. Hence the content and purposes of information are critical to this function, e.g. port authorities are authorised to access information on ships about to enter or leave that port, plus some emergency information.

Security standards in SafeSeaNet require that every user must be authenticated. The authentication ensures that data access is granted only to those whose identification is documented. SafeSeaNet defines the user as a Member State‘s authority playing a single SSN role (Port Authority, Port State Control, etc.). All users must be identified and all messages exchanged within the system must clearly indicate who the generator is. Therefore, XML message must refer to the user ID of the responsible authority that generated the message.

4. Action requested

The IALA VTS Committee is recommended to consider the message contents and the “hub and spoke” model as a request and response mechanism for the exchange of data.