IALA Legal Advisory Panel

LAP5/4/1

Response to request form VTS Committee VTS28

Background

The VTS Committee has been directed to provide guidance on navigational assistance services and has requested that the Legal Advisory Panel comments on the application of guidance materials, specifically:

- SOLAS Regulation V 12 - Vessel Traffic services

- IMO Resolution A.857(2) – Guidelines for Vessel Traffic Services

- IMO resolution A.918(22) – IMO Standard Marine Communication Phrases and

- IALA Vessel Traffic Services Manual, 4th Edition (2008)

to the provision of a course to steer by a VTS.

Status of documents

- SOLAS Regulation V -12

.	Required to be given effect by Contracting Governments; and

.	this Regulation specifically requires that Contracting Governments follow, where possible, the guidelines developed by IMO, in particular Resolution A.857(20).

- Resolution A.857(20) adopted November 1997

.	Contracting Governments should take account of the Guidelines provided in Appendix 1 of that Resolution; and

.	 those guidelines should be used in conjunction with Resolution MSC.43(64) (Guidelines and Criteria for Ship Reporting Systems – adopted December 1994) which call upon the Standard Marine Communication Phrases (which were under development when this MSC Resolution was passed) and the IALA Manual.

- Resolution A.918(22) – Adopted November 2001

.	Standard Marine Communication Phrases adopted for use in communications from ship to shore and vice versa.

- IALA Vessel Traffic Services Manual, 4th Edition (2008)

.	updated 2002 edition

.	manuals are third in the hierarchy of IALA documents, behind Recommendations and Guidelines and provide an overall view of a large subject area and reference other IALA documents as an indicator for further study.

Commentary

SOLAS V-12 requires that A.857(20) be followed, where possible.

A.857(20) indicates that VTS may consist of an information service, a navigational assistance service and/or a traffic organisation service (1.1.9) depending on the objectives of the VTS (2.2.2.5).

A navigational assistance service is a service to assist on-board navigational decision making and to monitor its effects (1.1.9.2).  Further, a navigation assistance service is especially important in difficult navigational circumstances or in cases of defects or deficiencies and may be instigated by either the vessel or VTS (2.3.2).

All VTS messages directed to a vessel should make it clear whether the message contains information, advice (where a professional opinion is included (2.5.2.3)), warning or an instruction (2.4.2) and when the VTS is authorised to issue instructions, these should be result-oriented only and should be issued with care not to encroach upon the master’s responsibility for safe navigation (2.3.4, 2.6.2).

VTS messages should use IMO Standard Marine Communications Phrases (SMCP) (2.4.1).

A.918(22) suggests that the SMCP was drafted as a simplified version of English in order to reduce grammatical, lexical and idiomatic variety so as to reduce misunderstanding in safety-related verbal communication ( p12, 4).

There are eight message markers that may be used to increase the probability of the purpose of the message being understood, including Information, Warning, Advice and Instruction (A1/6).  It is at the discretion of the shore personnel or ship’s officer to decide when to use message markers, and if used, which one is appropriate (A1/6).

No reference is made to any requirement for messages contained within message markers to be results oriented.  The description for both Instruction and Advice indicates that the intention of the sender is to ‘influence’ – the former using the force of national regulation and the other by recommendation without regulatory support (A1/6(i) and (ii)).

Examples of phrases for use in navigation assistance services are provided at A1/6.2.2 (p49).  The phrases include those related to Course and at A1/6.2.2.3.4 (p52) the following can be found: “Course to make good is …. Degrees and at A1/6.2.2.3.5.1 “Advise you ~ a new course of  … degrees”.

This implies that the giving of a course to steer is acceptable both in regard to a navigational assistance service and in giving advice.

However, in the VTS Manual, 4th Edition this position is contradicted at 1704 (p156) by the following statement: “a fundamental principle of VTS communication is that advice and instructions should be “results oriented” only; leaving the execution to the vessel.  The execution, such as courses to be steered or engine manoeuvres to be ordered, remains the responsibility of the person accountable for navigational decision making at that time”.

The position with regard to results oriented messages was different in the VTS Manual 3rd Edition, which reads: “A VTS … has responsibility for managing the traffic in the area.  While the master of a vessel, with his knowledge of its behaviours and his professional skills, has responsibility for the safety of the vessel.  Taking into consideration these different but related responsibilities, any instructions from a VTS to a vessel should be “result oriented” only, leaving the details of execution to the master, officer of the watch or pilot on board the vessel” (2.4.3, p 31).

There does not appear to have been any change to the higher precedent SOLAS Regulation V-12, Resolution A.857(20) or Resolution A.918(22) giving rise to this change.

Conclusion

The only document that supposes that Advice given by VTS services should be ‘result oriented’ is the VTS Manual 4th Edition (2008).  There is no basis for this position in the other documents.

As the giving of a course to steer is acceptable both in regard to a navigational assistance service and in giving advice, one option is to amend the VTS Manual 4th Edition at the next review.