e-NAV Shoreside User Requirements

The purpose of IALA is to ensure that seafarers are provided with effective and harmonised marine aids to navigation services worldwide to assist in safe navigation of shipping and protection of the environment.  Comprised of navigation authorities, manufacturers and consultants, IALA’s role is to harmonize aids to navigation worldwide and to ensure that the movements of vessels are safe, expeditious and cost effective and at the same time protect the environment.

IALA represents aids to navigation authorities that will be responsible in the future for the provision of e-navigation services to the mariner.  These services are comprised of both operational and technical e-Navigation services.

Many of the shoreside user requirements of e-navigation will come from vessel traffic services.  The following VTS levels of service and list of VTS equipment identifies some of those requirements.  The same approach should be done from the shipboard perspective.

Types of service at a VTS

An authorized VTS will be capable of offering one or more of the following service types: Information Service, Traffic Organisation Service, Navigational Assistance Service

Information Service (INS)

Maintain a traffic image and interact with traffic and respond to developing traffic situations. Assist the on-board decision-making process, by providing:

- Position, identity, intention and destination of vessels;

- Amendments and changes in the VTS area such as boundaries, procedures, radio frequencies, reporting points;

- Mandatory reporting of movements;

- Meteorological and hydrological conditions, notices to mariners, status of aids to navigation; limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances.

Traffic Organisation Service (TOS)

Provide essential and timely information to assist the onboard decision-making process.

May involve the provision of information, advice and instructions.

Plan movements to maintain vessel safety and to achieve efficiency.

Service may include:

- Position, identity, intention and destination of vessels;

- Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

- Mandatory reporting of movements;

- Meteorological and hydrological conditions, notices to mariners, status of aids to navigation;

- Traffic congestion and special vessels with limited manoeuvrability which may impose restrictions on the navigation of other vessels or any other potential hindrances;

- Allocation of waterspace;

- Establish and operate a system of traffic clearances - all or certain classes of vessels may be required to participate in this service and shall not proceed without clearance;

- Establish routes to be followed and speed limits to be observed and other measures

- Traffic congestion and advice about vessels within the VTS area

- Co-ordinating VTS sailing/route plans, including intended route, ETA in VTS area, departure from berth or anchorage. May include general flow of traffic, efficiency and co-ordination with allied services.

Navigational Assistance Service (NAS)

Provide navigational information, advice, or instruction to assist in the on-board navigational decision-making process.

Provided at the request of a vessel or when a navigational situation is observed and intervention by VTS is deemed necessary. Requires positive identification and continuous communication throughout the process.

Agreed between the vessel and VTS providing the service.

Provision of information, such as:

- Course and speed made good by a vessel;

- Position relative to fairway axis, navigational features and/or way-points;

- Proximity to navigational hazards;

- Positions, identities, intentions and any restrictions of surrounding traffic.

Provision of advice and/or instruction, and may include or require:

- An assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

- A review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

- An assessment of the environmental conditions;

- An assessment of the implications of the cargo carried;

- A review of the proposed sailing plan;

- Recommendations on measures to maintain the sailing plan noting that any advice on courses and speeds should be result orientated;

- The use of message markers;

- The use of a dedicated frequency;

- Restriction of other traffic movement.

Along with the above VTS service types, the following equipment/services/systems can be considered to form the basis of the shoreside user requirements for e-navigation.  The following list of VTS equipment is extensive and the requirements for each sensor or system are voluminous, so all of those requirements cannot be replicated here.  However, an attempt was made to capture the most basic requirements (i.e., functional requirements or the data elements output by the equipment) of each below.  The same approach should be done from the shipboard perspective.

If shoreside user requirements are supported by e-navigation, then one can expect a benefit to VTS operators and other shoreside authorities.  Better information from added sensors will serve to enhance the benefit.  Through the e-navigation integration of this equipment, one can anticipate a potential reduction in equipment and hence, a cost savings to those VTS’ and aids to navigation authorities.

Shoreside Sensors and Communications Equipment

Communications

Communicate with shipping and with other operators or allied services.

Control of available communication equipment to be integrated into each VTS Operators console.

Communication with Shipping in the VTS Area

VHF using telephony or data transmissions.

Between shore and participating vessels using appropriate international VHF channels on simplex or duplex channels.

With duplex channels, re-transmission from shore if the information received is of interest to other vessels to enable them better to comprehend the traffic situation.

Private channels when available for use between VTS Centres and local service craft. Local service craft should be able to communicate with shipping on the appropriate international VHF channel.

Number of VHF channels required for specific VTS usage assessed and arranged with the National Telecommunication Authorities.

Number of channels dependent upon number of sub-areas and sectors that will be used and the overall traffic density.

One or more general reporting or emergency frequencies.

VHF Channel 16 for emergencies and for calling ships not participating in the VTS.

Record communications with vessels for accident investigation, disputes, etc.

Very High Frequency (VHF) Radio Communication

VTS often has its own independent VHF network for the use within specifically designated VHF Channels.

Network may be comprised of one or more VHF Channels in different sectors of the VTS Area.

VTS Authority may require specific VHF Channels to be designated by the National Telecommunications Authority for specific types of operations.

VHF equipment must comply with national and international regulations

Long Range Communication

Normal maritime communication systems (e.g., MF, HF, Inmarsat) where a VTS Authority requires pre-arrival information from beyond VHF coverage.

Communication with Allied Services

Ability to communicate with allied services by the use of reliable and secure communication networks.

Typically, VTS Centre equipped with point-to-point line with Maritime Rescue Coordination Centres (MRCCs).

However, point-to-point lines have been found to be inadequate and unnecessarily expensive to establish and maintain. Recommended that VTS Centres should be equipped with a digital switched network, with caller identification.

Other Communications

If more than one VTS Centre, communicate easily with operators in other VTS Centres so that specific traffic information can be passed from one operator to another with the minimum of delay.

The following general types of communication may be required by a VTS:

- Between VTSOs in the same VTS centre

- Where appropriate, between VTSOs in different VTS centres belonging to the same VTS

- Between VTSOs of any adjacent VTS

- With vessels about to enter a VTS area

- With Pilotage, tugboats, SAR, Port Authorities and other Authorities.

These types of communication can be performed by public or private telephone lines, telex, facsimile, radio telephony and microwave links, or automated data transfer systems.

Where there is a need for communication between a VTS and a shore authority to have a high degree of operational importance, provide direct telephone lines to increase the reliability and decrease the time to connect.

VTS Radar System

VTS radars will in most cases need to operate simultaneously on short and long range, without the need for operator adjustments. Weather-related phenomena such as sea clutter and ducting will influence shore-based radars more than ships’ radars.

Radar Functions

| Parameters / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Path, time and track prediction, |  |  | X |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| Anchor watch, |  |  | X |
| Vessels vector, | X | X | X |
| Course, speed and label/identity, | X | X | X |
| Collision alerts. | X | X | X |

Automatic Identification System (AIS)

Littoral states obtain information about ship and cargo; as a VTS tool for traffic management

Ships provide information autonomously and continuously to a competent authority and ships without involvement of ship’s personnel

Receive and process information from other sources

Respond to high priority and safety related calls with a minimum of delay

Provide positional and maneuvering information at a data rate adequate to facilitate accurate tracking

Provide static, dynamic, voyage-related, and short safety-related messages

AIS should exchange data from ship-to-ship and with shore-based facilities.

Identify vessels; assist in target tracking; simplify information exchange (i.e. reduce ship reporting using radiotelephony); and provide additional information to assist situational awareness.

Automatic and immediate provision of vessel identity (MMSI or call sign) which, where necessary, helps to facilitate rapid radio communication

Monitor and track ships from suitably equipped ships, and shore stations.

AIS data consists of shipboard information, such as: position, time, course over ground (COG), speed over ground (SOG) and heading.

Binary messaging capabilities

Receive information from AIS equipped ships within VHF range.

Provide AIS information from one or several base stations to users. In addition to vessel data, provide status on AIS equipment and management functions for the control of the AIS network.

Where applicable, support regional Vessel Traffic Services between adjoining VTS Centres.

View AIS data on electronic chart, either separately or combined, with the other data sources including radar.

Have the ability to filter the displayed information.

Presented to the VTSO on an integrated display.

The user may have the option to display the input from each sensor with two different symbols.

Identify which sensor(s) are used to derive the target position.

Symbols to depict different types and sizes of vessels, vessels with pilots embarked, other information.

AIS symbols to be used in VTS Centres is choice of the VTS Authority

Radio Direction Finder (RDF)

Receive information on the position and assist in identification of vessels, using their radio emission.

Two or more separate RDF stations is required.

All bearings automatically displayed on the chosen screen when signal received after a delay of no more than 3 seconds.

Bearings remain visible on the chosen screen as long as the vessel is transmitting a signal.

Recording or instant replay capability.

The recommended bearing accuracy for the types of capability are: ± 2.5º for Basic;  ± 1.5º for Standard; and  ± 1.0º for Advanced RDFs.

Hydrological/Metrological Equipment

Local Hydro/meteo information relevant to the VTS Area(s) available to VTS

Ability to disseminate this information to users and allied services.

Where a VTS Authority establishes its own monitoring stations, the individual VTS Authorities determine the accuracy and availability requirements for each VTS Centre.

Typical meteo variables include air temperature and humidity, wind velocity and direction, and visibility.

Hydro variables such as tidal level, tidal stream/current direction and velocity.

Data obtained through sensors or available in tables/databases from national authorities.

Communicate sensor data to a VTS Centre via a telecommunications link.

Present graphical and/or numeric information for use by the VTS Operators.

Integrate hydrological and meteorological information into VTS applications to provide the VTS Operator a real-time assessment of the environmental situation in the VTS area of responsibility.

Provide information to ships to assist in assessing the waterway conditions.

Tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.

| Parameter | Minimum Accuracy |
| --- | --- |
| Height of Tide |  ± 0.10 m |
| Rate of Tidal Stream/ Current |  ± 0.5 knots |
| Direction of Tidal Stream/Current |  ± 10 Degrees |
| Wave height |  ± 10% of the height |
| Wave Direction |  ± 20 Degrees |
| Wind speed |  ± 1m per sec |
| Wind Direction |  ± 10 Degrees |
| Visibility |  ± 10% of the distance |
| Air Temperature |  ± 10C |
| Air Humidity |  ± 5 % |
| Air Pressure |  ± 2 hPa |
| Sea Surface Temperature |  ± 10C |

Closed circuit TV (CCTV) Cameras

Integrated into VTS applications

Identify type of vessel at a minimum range of 3 nm with nominal visibility in excess of 10 nm

Identify a vessel by shape, colour and other features at a minimum range of 1 nm with nominal visibility in excess of 10 nm

Use in low traffic density areas as an alternative to radar or in conjunction with radar as an additional sensor

Automatically track a vessel manually selected by the VTS Operator via the VTS Screen

Manually de-select the automatically acquired target and manually select another target and/ or area of the acquired target

Where more than one camera is installed to cover a VTS area, the output from both should be fused together to provide one composite picture.

Information Management

Collect, process and correlate information from different sources to present an integrated image of traffic, its environment and maintain situational awareness. This may include:

Communications, internal and external;

Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS;

Shipping information data, i.e. vessel and cargo data, including vessel movement information;

Meteorological and hydrological data; and

Data from other sources if relevant.

Record and store data for later use.

Operator Interface

Display the traffic image, including the traffic situation and corresponding traffic information.

Traffic Situation Display

Display mapping graphics, analytical graphics and overlay information provided to enable a VTS Operator to have a concise picture of the geographical features, waterways and navigational lanes.

Ability to suppress radar video echoes of the coastline, making this area of the display available for other synthetic information.

If VHF RDF included in the system, bearing lines should be shown on the VTS display, with an option for the operator to suppress.

IALA Recommendation V-125 on the use and presentation of symbology at a VTS Centre (including AIS).

Traffic Information Display

In addition to the traffic situation display, include display of textual information in the form of tables etc.

List of participating vessels, including static and dynamic information concerning the vessel, such as vessel name, Call Sign, IMO Number, MMSI, ETA, ETD, Course, Speed and Position

Work Environment - VTS Operator Positions

A console provided at every VTSO position with the equipment integrated in ergonomic arrangement.

Illumination of the operator position such that all relevant equipment (VTS monitor, communication equipment, target analyzer etc.) can be monitored effectively and administrative tasks can be carried out at the same time.

Daylight displays for radar equipment

Access to data base information at VTSO position for reference purposes and for providing information to shipping.

Operational conditions, Redundancy and emergency precautions

Account for extremes of temperature or precipitation, wind force, or earthquake resistance where appropriate.

Maintenance and availability of remote sensors and adequate redundancy provided.

All operational VTS communication services to have back-up systems with a power source independent from the normal power supply.

Redundant sensors or alternative site for the VTS Centre where appropriate.

Where provision is made for an alternative site for the VTS Centre, operations should be capable of being easily transferred to the secondary location in the event of an emergency or maintenance situation that causes the temporary closure of the primary VTS Centre.

When determining the position and range of radar equipment, malfunction of a radar should be taken into account, and where practicable arrangements should enable another radar, or AIS where appropriate, to provide cover for the sub-area or sector affected.

Spare VTSO consoles should also be considered to:

Substitute for any console that is unserviceable;

Be used for On-the-Job training;

Where a need exists, be used by a VTS Supervisor;

Where appropriate, be used in the co-ordination of emergencies.

VTS should not be entirely dependent on synthetic images of traffic.

Combination of radar video and extracted information is recommended.

Means to verify track positions and classify individual target on the basis of the radar video.

Flexible and easily updateable during routine operations without the need for interrupting these VTS operations.

r