Dear Mr. Cherif

Thank you for your letter of April 10, 2008 (Ref: E 4/56) asking for IALA’s comments on the proposed new ICAO Standards and Recommended Practices (SARPs) on visual aids for marking wind turbines.

According to IALA recommendation O-117 on The Marking of Offshore Wind Farms (attached as appendix 1) Aeronautical obstruction warning lights fitted on top of wind generators should not be visible below the horizontal plane, as far as practicable.  This, of course, is to avoid confusion to the mariners between the lights used at the water level, which mark the limits of the wind farms and any other lights marking the top of the structure for aviation purposes.

However, IALA also understand the needs for accurate and reliable light signals below the horizontal plane surrounding wind farms, mainly for helicopters and low flying airplanes. IALA is therefore suggesting that the section 6.4 on Wind turbines be revised to indicate that flashing red lights with medium intensity should be used in such cases.  The Manual of Standards Part 139 – Aerodromes, issued by the Australian Government Civil Aviation Safety Authority, chapter 9.4.2.3 and 9.4.2.4 (attached as appendix 2) provides guidance for the appropriate selection of those lights.  The use of flashing red lights only would also make easier the distinction between lights for mariners and lights for aviators.  In addition, on occasions in very good visibility the aeronautical beacon can be mistaken for a navigational light or a vessel side light.  The use of automatic sensors to reduce intensity should be considered.