ANM12/10/4

Formerly EEP12-output-20

From EEP12

To: ANM Committee

Liaison Note

Aids to Navigation Guide (NAVGUIDE) Updates

The EEP Committee has updated the relevant sections of the NAVGUIDE.  Please note that no changes were made to the Sound Signal section 4.3.

EEP12-output-20 contains the relevant changes and is attached at Annex 1.

EEP also notes ANM response to EEP request for clarification on the type and numbers of pictures requested for the NAVGUIDE and has advised its committee members to forward pictures in the required file format to the NAVGUIDE co-ordinator Gerry Brine (gerry.brine@amsa.gov.au) by March 6, 2009.

Annex 1 – EEP12 Navguide Updates

### Section 3.1.12

### Fixed Aids to Navigation

The IALA International Dictionary of Aids to Marine Navigation defines a beacon as “a fixed artificial navigation mark” that can be recognised by its shape, colour, pattern, topmark or light character, or a combination of these. While this functional definition includes lighthouses and other fixed aids to navigation, the terms lighthouse and beacon are used more specifically to indicate importance and size.

#### Definitions

Photo courtesy of Australian Maritime Safety Authority

Lighthouse:  A lighthouse is generally considered to be:

A large conspicuous structure (visual mark) on land, close to the shoreline or in the water that:

- acts as a daymark, and;

- provides a platform generally for higher range marine AtoN signal lights.

Other aids to navigation such as audible signals and radio aids to navigation may be located on or near the lighthouse;

A Lighthouse may be a staffed or automated facility, although the staffing of Lighthouses is becoming less common.

An automated lighthouse will often be remotely monitored and in some cases remotely controlled.

Beacon: Visual characteristics of a beacon are often defined by daymarks, topmarks, and by numbers.  A marine signalling light, if fitted, would generally be of a lower range than lighthouses.

In navigable channels a pile beacon may be used as an alternative to a buoy.

#### Purpose of Lighthouses and Beacons

A lighthouse or beacon may perform one or more of the following navigational functions:

- mark a landfall position;

- mark an obstruction or a danger;

- indicate the lateral limits of a channel or navigable waterway;

- indicate a turning point or a junction in a waterway;

- mark the entrance of a Traffic Separation Scheme (TSS);

- form part of a leading (range) line;

- mark an area;

- provide a reference for mariners to take a bearing or line of position (LOP).

Other purposes for which a lighthouse can be used include:

- base for AIS equipment;

- base for racon

- platform for radar

- base for radionavigation systems

- reference station for DGNSS

- coast watch or coastguard functions;

- VTS functions;

- base for audible (fog) signals;

- collection of meteorological and oceanographic data;

- radio and telecommunication facilities;

- tourist facilities.

### Floating Aids to Navigation

A floating aid to navigation serves a similar purpose to a beacon or lighthouse.  However the floating aid to navigation is normally associated with locations where:

- it would be impractical due to water depth, seabed conditions or cost to establish a fixed aid;

- the hazard shifts over time (eg. sand banks, an unstable wreck, etc.);

- the aid is at high risk of damage or loss from ice flows or ship impacts and as a consequence is treated as expendable;

- a temporary mark is required.

#### Buoys

Buoys are defined as minor floating aids and whilst it is normal that they are lit there are instances where no light is installed.

These types of aids to navigation are specifically covered by the IALA Maritime Buoyage System and tend to have circular hull forms in range of 1 to 3 m diameter.  Buoys may be fitted with sound signals.

In addition, due to limitations of the structure, the following may apply:

- where lights are exhibited they are usually solar or single cycle battery powered, however gas powered buoys are still in operation;

- where lights are exhibited, due to power limitations, light ranges are restricted to typically 2 to 5 nautical miles, although much higher ranges are used in some applications;

- due to limited power on the buoy, additional services are restricted but Racon and AIS units are sometimes deployed in addition to a light.

- Electric fog signals are used on buoys in some countries

#### Lightvesels, Lightfloats and Large Navigational Buoys (LANBYS)

Photo of Lanby, courtesy of Commissioners of Irish Lights

Lightvessels, lightfloats and lanbys (or LNB) are defined as major floating aids and may carry one or more of a racon, AIS AtoN, sound signal, and in some cases, a radio beacon in addition to the aid to navigation light.  A lightvessel may also display a white riding light to signify a vessel at anchor.

These types of aids to navigation:

- generally have high operating costs;

- are only deployed at critical locations;

- are often assigned an availability target that is higher than for a buoy;

- are not specifically covered by the IALA Maritime Buoyage System.

Some lightvessels continue to be manned, but the trend is towards automation, often with remote monitoring and control.

Refer to IALA publication:

IALA Recommendation O-104 for ‘Off Station’ signals for major floating aids to navigation

#### IALA Maritime Buoyage System (MBS)

The Maritime Buoyage System represents one of IALA’s major contributions to enhancing the safety of navigation.  As recently as 1976 there were more than thirty buoyage systems in use world wide and conflicting sets of rules applied.  In 1980 Lighthouse Authorities from fifty countries and representatives from nine international organisations reached agreement on the rules for a single system.

The IALA Maritime Buoyage System uses 5 types of marks, which may be used in combination.  The mariner can distinguish between these marks by identifiable characteristics.  The system includes Cardinal and Lateral marks.  The Lateral marks differ between Buoyage Regions A and B.  In response to concerns from members over the marking of new, dangerous, wrecks, IALA has produced specific guidelines and recommendations, including the trial introduction of an emergency wreck marking buoy.

Refer to IALA publications:

IALA Maritime Buoyage System (with supporting guidelines)

IALA Guideline 1046 on a Response Plan for the Marking of New Wrecks

IALA Recommendation O-133 Emergency Wreck Marking Buoy

The content of the General Principles and Rules of the IALA Maritime Buoyage System can be found in Annex A.

IALA also produces recommendations and guidelines for marking areas for specific navigational needs such as wind farms, fish farms and offshore structures.

#### Performance Criteria for Floating Aids

Availability is defined as:

The probability that an aid to navigation or a system of aids to navigation as defined by the Competent Authority is performing its specified function at any randomly chosen time. This is expressed as a percentage of total time that an aid to navigation or a system of aids to navigation should be performing their specified function

The availability of a floating aid is the principal measure of performance determined by IALA.  The recommended availability targets are indicated in Table 1.

Table 1 – Availability Targets

| Type of Aid (examples only) | Availability Target |  |
| --- | --- | --- |
| Floating aids to navigation that are considered to be of primary navigational significance. | Category 1 | at least 99.8% |
| Floating aids to navigation that are considered to be of navigational significance | Category 2 | at least 99% |
| Floating aids to navigation that are considered to be of less navigational significance than Category 1 or 2. | Category 3 | at least 97% |

Note

The availability objective assigned to floating aids to navigation conforming to the IALA Maritime Buoyage System should also apply to the topmark.

Refer to IALA publications:

IALA Recommendation O-130 on Categorisation and Availability Objectives for Short Range Aids to Navigation

IALA Guideline 1035 on Availability and Reliability of Aids to Navigation

#### Technical Considerations for Floating Aids to Navigation

There are various technical considerations that should be taken into account, including: cost; design factors; positioning and markings.

##### Cost

The cost of establishing a floating aid at a given location will generally be less than for a fixed structure.  The cost difference increases with increasing water depth and exposure to wind and waves.

In contrast, the maintenance cost of floating aids to navigation tends to be high relative to the capital value.  This has caused many authorities to critically examine the potential for savings through design changes, use of alternative materials, alternate service deliveries (contracting out) and amending maintenance practices, generally with the aim of extending maintenance intervals.

Where an authority operates a large number of floating aids, it may become practicable to operate a dedicated buoy tender vessel with specialised equipment to minimise buoy change-out times and to improve occupational safety.

Refer to IALA publication:

IALA Guideline 1047 on Cost Comparison Methodology of Buoy Technologies

##### Floating Aid Design

The process of designing a buoy to meet specific requirements is a specialised task.  It involves, but is not limited to:

- defining the operational performance characteristics;

- defining the equipment, power requirements and power source(s);

- defining the type and capabilities of the vessels that will be used to service the buoy;

- selecting the initial type proportions and mooring for the buoy;

- integrating of equipment and power supply;

- considering of the maintenance requirements;

- identifying deployment and recovery techniques;

- protecting equipment from damage;

- providing the ability to rectify faults without having to lift the buoy;

- determining the buoy response to the wave, wind and current conditions at the site(s);

- optimising design.

Refer to IALA publications:

IALA MBS and Guidelines

IALA Guideline 1006 on Plastic Buoys

IALA Guideline 1011 on a standard method for defining and calculating the load profile of aids to navigation

IALA Guideline 1036 on Environmental Considerations in Aids to Navigation Engineering.

IALA Guideline 1037 on Data Collection for Aids to Navigation Performance Calculation

IALA Guideline 1039 on Designing Solar Power Systems for Aids to Navigation (with excel file)

IALA Guideline 1040 on the Maintenance of Buoys and Small Aids to Navigation Structures

IALA Guideline 1042 on Power Systems for Aids to Navigation

IALA Guideline 1043 on Light Sources used in Visual Aids to Navigation

##### Mooring Design and Swing Radius

The mooring system for a floating aid to navigation is the sum of the components that keep the aid within a nominated area.  These components have to withstand the forces of wind, wave and current on the floating aid and drag on the mooring line.  Methods for determining the forces are covered in the IALA  Guideline  1059 on Floating Aid Moorings .  The basic assumptions made are that the:

- mooring system is tangential to the sea bed under all conditions of current and wind at the site;

- buoy axis is vertical under the most common conditions of current and wind;

- ratio of the breaking stress of the mooring system to the calculated stress is not less than 5 under the most unfavourable conditions of current and wind;

- reserve buoyancy of the fully equipped floating aid is greater than the combined loads of current and wind under the most unfavourable conditions.

###### Swing Radius

Reference can be made to the IALA Guideline 1059 on Floating Aid Moorings on the design of normal moorings for the formula for the maximum swing radius. May 1998 indicates that the maximum swing radius (watch circle radius) of a floating aid is:

rm  = L2 − H2

Where:

The recommended minimum length of mooring line is:

- Lmin = 23H for depths less than 50 metres;

- Lmin = 1.52H for depths greater than 50 metres;(depending on local sea conditions)

- Lmin = 7H for shallow moorings where breaking waves occur.

| rm = | Maximum swing radius (m) |
| --- | --- |
| L = | Length of mooring line (m) |
| H= | Depth of water (m) (This is defined as the maximum depth of water and includes the highest tide level and half the maximum wave height at the particular site.) |

Refer to IALA publications:

IALA Practical Notes on the use of mooring chains for floating aids to navigation (June 1989)

IALA Recommendation E-107 on the design of normal moorings

IALA Guideline 1024 on synthetic 1059 on Floating Aid Moorings lines

##### Positioning of Floating Aids

The charted position of a floating aid defines the nominal (or true) position for the anchor/sinker.

With most floating aids there is potential for the mooring anchor/sinker to be moved off-station during storms or for positional errors to occur while laying the anchors/sinkers.

Anchors/sinkers have traditionally been laid while taking cross bearings and/or horizontal sextant angles from fixed visual marks.  When out of sight of land the process may have relied on radionavigation or radio-positioning aids.  While some authorities may still use these procedures, the use of DGPS position fixing is increasingly seen as the preferred method.  The benefits of DGPS position fixing are: convenience, accuracy and repeatability.  A buoy tender using DGPS can generally be brought to within 10 metres of the nominal buoy position at the time of releasing the anchor/sinker.

If the anchor/sinker is allowed to free-fall, its final resting position will depend on the prevailing current, water depth, shape of the anchor/sinker and the nature of the seabed.  Controlling the decent of the anchor/sinker may serve to improve the positional accuracy of the buoy.

#### Markings and Topmarks

##### Markings

Floating aids to navigation are often identified by names, abbreviations of names, letters and/or numbers.  Authorities should ensure that the actual marking is identical to the List of Lights reference and the charted marking.

##### Topmarks

The type, colour and arrangement of topmarks on a buoy are shown in the IALA Maritime Buoyage System, extracts of which are shown in Annex A.

Topmarks anc can be conical, cylindrical, spherical or diagonal cross.

Conical topmarks (for lateral and cardinal marks):

- The vertical height of a cone from base to apex should be about 90% of the base diameter;

- For cardinal marks, the separation distance between cones should be about 50% of the base diameter of the cone;

- The vertical clear space between the lowest point of the topmark and all other parts of the mark should be at least 35% of the base diameter of the cone;

- In the case of a buoy, the base diameter should be 25%-30% of the diameter of the buoy at waterline.

Cylindrical (can) topmarks (for lateral marks):

- The vertical height of a cylinder should be 1 to 1.5 times the base diameter;

- The vertical clear space between the lowest part of the cylinder and all other parts of the mark should be at least 35% of the diameter of the cylinder;

- In the case of a buoy, the base diameter of the cylinder should be 25%-30% of the diameter of the buoy at the waterline.

Spherical topmarks (for isolated danger and safe water marks):

- In the case of a buoy, the diameter of the sphere(s) should be at least 20% of the diameter of the buoy at the waterline;

- For isolated danger marks the separation distance between spheres should be about 50% of their diameter;

- The vertical space between the lowest part of the sphere(s) and all other parts of the mark should be at least 35% of the diameter of the sphere(s).

'X' (Diagonal Cross) topmarks (for special marks):

- In the case of a buoy, the arms of the 'X' should be diagonally contained within a square with length of side of about 33% of the buoy diameter at the waterline. The width of the arms of the 'X' should be about 15% of the length of side of the square.

Section 3.1.14

### Sector Lights and Leading (Range) Lines

Note – Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner.  Bearings may carry a suffix ‘TBS” or True Bearing from Seaward as confirmation.

#### Sector Lights [Update in light of new technologies eg LED]

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs.

A common means of creating a sector is to fit a coloured filter in front of the main light. However, sector lights with LED light sources are being introduced to the market thereby reducing the need for filters as they produce the coloured light.  A sector can be produced by filtering or by using a secondary light (or several lights) on the same structure.  The secondary light can take any of the following forms:

- Range (directional) light;

- Beacon with a coloured lens, masked to achieve the sector angle;

- Beacon fitted with internal or external filter panels;

- Beacon or beacons with different coloured light sources, masked to achieve the sector angle;

- Precision Direction Light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source, fading of colours or changing rhythms between adjacent sectors.

For a beacon fitted with coloured filter panels, the reason for the lack of a precise transition at the sector boundary is readily apparent from Figure 1 which shows the light source, lens and filter geometry.  The transition zone is defined by an "angle of uncertainty".

A similar geometry exists with multiple coloured beacons and masking.

Figure 1 – Angle of Uncertainty

It can also be noted that:

- the observed angle of uncertainty is generally less than the geometric angle due to the relative intensities of sector colours (ie. colour mixing) as the observer passes through the transition zone;

- if space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.25 with this type of sector arrangement;

- the angle of uncertainty can be reduced by decreasing the physical width of the light source or by increasing the radial distance to the coloured filter;

- in situations where the main light has a large projected area, such as a rotating lens or reflector array, it is generally preferable to use a separate sector light rather than installing a coloured filter in front of the main light.

From time to time specialised sector lights have been developed to exhibit different rhythms over different sector bearings.  This capability is found in some Precision Direction Lights (PDL).

A PDL is a specialised form of sector light that can generate sharply defined sector boundaries.  This feature is particularly useful for applications that require one or several narrow sectors or high precision boundaries. The PDL may use a white light source with coloured filter, but newer designs are utilising LED and possibly laser as a light source.

PDL Sector Lights are very precise, providing a complete colour change at a sector boundary occurs over an angle of less than 1 minute of arc in most models.

##### Applications

The design of sector lights can be a complex task.  The process should be carried out with reference to an accurate chart of the area.  In some cases good local knowledge is also required.

A sector light may indicate one or more of the following:

- boundaries of a navigable waterway;

- change of course position;

- shoals, banks, etc.;

- an area or position (eg. an anchorage);

- the deepest part of a waterway;

- position checks for floating aids.

A Precision Direction Light (PDL) allows for further applications that include the ability to:

- produce narrow sectors with an angles of uncertainty down to approximately one minute of arc;

- define the central zone of a channel;

- accurately mark one side of a straight channel.  (A pair of PDLs can cover the permutations of converging, diverging and parallel channels);

- define different rhythms over adjacent sectors.

##### Examples

Some examples of sector lights applications are illustrated in Figure 2 and Figure 3.

Figure 2 – Sector Light Application

This illustration follows the IALA Maritime Buoyage System colour convention for Region A (‘red to port when approaching the aid from seaward’).  The white sector should, if possible, be wide enough to provide a margin of safety for a vessel that inadvertently leaves the white sector. Curves C and D indicate depth contours or limiting dangers that dictate the boundaries of sectors.

Figure 3 – Shows several applications for sector lights.

The function of each light is described below:

- Light 1 is a coastal white light with a red sector indicating a danger.

- Light 2 is a sector light obscured over the shore, with two white sectors indicating a safe channel.  When sailing towards the sector light it shows red to port and green to starboard following the IALA Maritime Buoyage System colour convention for Region A.  The boundary between the red and the green sector also indicates the position of a buoy.

- Light 3 is a sector light with a red light and 4 white sectors indicating four anchorage positions.  It is obscured over the shore.

- Light 4 is a sector light with a white sector indicating a safe channel.

##### Design Considerations for Sector Lights

Where a single sector light defines a navigable channel the following points should be considered:

- Lateral position:  There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached.  This may cause a problem in channels subject to a strong cross current.  For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary;

- Safety margin:  Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards.  If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately.

- Angle of uncertainty:  Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary;

- Vessel size:  The design process for a sector light needs to consider the draught and manoeuvrability of the largest vessels likely to utilise the sector, how quickly they can respond once they cross a sector boundary and the situations that may develop when other vessels are in the vicinity;

- Lights and filters: When using incandescent light source the  sector design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material as this will affect the resultant colour and intensity of the light exhibited.  The process should also check for potential glare problems;

- Flash characteristic:  The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary;

- Sector colours:  A white light is normally the first preference for a lighthouse or beacon.  If a single coloured sector is added, red is often used.  If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits.  In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System;

- Multiple sectors:  Multiple sectors can be used to provide a better indication of a vessel’s lateral position within the channel but at the expense of complexity for both the system designer and navigators.

- Lamp position and type:  The position of the light source within the optical system is critical for the correct alignment of the sectors. When replacing lamps or using lampchangers, it is important to ensure that the light source (e.g. filament) position is identical. If a lampchanger is incorporated, the sector system should be designed for the widest light source used in the lampchanger.

Refer to IALA publication:

IALA Guideline 1041 on sector lights

#### Transits / Leading Lines (Range Lines)

A transit is defined as the alignment of two or more marks.

A Leading (or Range) light is a specialised application of a transit.

A simple transit can be used to:

- Provide a turning reference;

- Define a clearing line for the limits of safe navigation;

- Provide a distance mark along a waterway.

##### Leading Lines (Range Lines)

A leading line is an aid to navigation system that comprises two separated structures with marks or lights that, when viewed from the centreline or deepest route along a straight section of channel, are aligned.

In a two station leading line, the structures lie along an extension of the centreline of the nominated channel.  The rear structure must have a greater elevation than the front structure to enable both marks or lights to be viewed simultaneously.

A leading line provides a vessel with a heading reference and a visual indication of the size and direction of any cross track error.

Figure 4 – Leading Line (photo courtesy of Canadian Coast Guard)

##### Purposes of Leading Lines

A leading line may be used to:

- indicate the centreline of a straight section of a navigable channel;

- indicate to deep draught vessels the deepest part of the waterway;

- indicate the navigable channel where fixed and floating aids to navigation are not available or do not satisfy the accuracy requirements for safe navigation;

- define a safe approach bearing to a harbour or river entrance, particularly where there are cross currents;

- separate two-way traffic (i.e. when passing a bridge).

##### Design Considerations for Leading Lines

A well-designed leading line will enable vessels of the type and size that commonly use the channel to:

- Identify the marks or lights when the ship is at the inner and outer sections of the channel and readily detect cross track position errors from the centreline of the channel;

- Detect cross track position errors with sufficient sensitivity that the channel can be utilised without abrupt changes to the vessels heading and speed;

- Observe both lights together, by selection of leading light character rhythms that appropriately overlap in their free running condition. In some situations it may be preferable to provide additional equipment to synchronise the light characters.

- Observe the lights in all ambient conditions for which they are designed to be used without glare, if lights are to be used for both day and night operations light intensities will need to be varied.

The characters of rhythmic leading lights should be selected so that the front and rear lights, in their free running states, can generally be observed together.  In some situations it may be preferable to provide additional equipment to synchronise the light characters;

If lights are to be used both day and night, the light intensities should be adapted for each situation to avoid glare at night.

Radar transponders (RACONs) may be used as leading line markers.

Refer to IALA publications:

IALA Recommendation E-112 for leading lights (including excel program)

IALA Guideline 1023 for the design of leading lines

IALA Recommendation for a definition of nominal daytime range of maritime signal lights intended for guidance of shipping by day, (April 1974)

Section 4.3 to 5.6.2

## Sound Signals

Although sound signals still exist, it has been IALA policy since 1985 that these devices should only be used as a hazard warning.

The National Authority shall determine whether a hazard requires a sound signal and the level of reduced visibility per year that justifies its installation (ex: 10 days of visibility under 1nm/year). Certain man-made structures such as offshore structures, bridges, breakwaters and isolated AtoN may be considered a hazard requiring a sound signal.

Refer to IALA publications:

IALA Recommendation E-109 for the calculation of the range of a sound signal;

IALA Recommendation O-113 for the marking of fixed bridges over navigable waters

IALA Recommendation O-114 for the marking of offshore structures

### Considerations on Sound Signals and their use

IALA Recommendation O-114 establishes that on offshore structures, sound signals shall be operated when the meteorological visibility is two miles or less.  There are a number of considerations to be taken into account with regards to sound signals and their use:

- Sound propagates in the atmosphere in a variable manner, making the perception of direction and distance to the emitter difficult.  It may be very difficult to estimate the location of a danger.

- A linear increase in the perception of a sound corresponds to an exponential power increase in the sound source.

- Background noise level on board vessels may prevent recognition of a sound signal.

- Occasionally, sound propagation is such that a signal may be almost inaudible close to the source, but of the expected level further away from the source.

- The identification of the sound signal characteristics may not be reliable as a result of fluctuations in propagation causing interruption of reception.

- A sound signal may be considered a nuisance by the local community.

- In some situations, there is the need to combine two or more sound sources or to install a baffle device to avoid the propagation of sound in a certain direction. In both cases, care must be taken to avoid the sound of one source being cancelled by the sound of the other or by the reflected sound.

- Sound signals are normally operated automatically using a fog detector.

### Considerations on Fog Detectors

Until approx. 30 years ago, sound signals were operated by lighthouse keepers observing local visibility and turning on the signal as required. At present, automatic fog detectors that emit an infrared beam, measure the reflection from the water particles in the air, and activate the sound signal at certain visibility thresholds.

Reliable remote visibility meters, developed for use at remote meteorological stations, are used as fog detectors.  These may be activated by heavy rain or snow, as well as fog.

### Sound Signals in the World

Some countries, including Finland, Iceland, Australia and Norway have abolished the use of sound signals.

Other countries continue to use fog signals, usually 2 mile range electric signals, on offshore lighthouses and light vessels.  In countries with high incidence of fog, some large compressed air powered signals are still in use.

### Range of a sound signal

Nominal Range: the distance at which, in fog, a lookout positioned in the wing of the bridge has a probability of 90% of hearing the signal when subjected to a noise as defined by IALA as being equal to or in excess of that found in 84% of large merchant vessels, the propagation between the sound signal emitter and the listener occurring during relatively calm weather and with no intervening obstacles.

Table 2 – Nominal Range

| Sound Pressure Level |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pn(NM) f(Hz) | 0.5 | 1 | 1,5 | 2 |
| 25 | 162 | 172 | 176 | 178 |
| 50 | 149 | 161 | 165 | 168 |
| 100 | 138 | 150 | 154 | 157 |
| 200 | 130 | 142 | 147 | 150 |
| 400 | 122 | 135 | 140 | 144 |
| 800 | 115 | 130 | 137 | 142 |
| 1000 | 113 | 129 | 137 | 144 |
| 1250 | 112 | 129 | 138 | 146 |
| 1600 | 110 | 130 | 140 | 150 |
| 2000 | 109 | 132 | 145 | 156 |
| 2500 | 108 | 136 | 151 | 166 |
| 3150 | 107 | 141 | 160 | 179 |
| 4000 | 109 | 150 | 177 | 199 |

Pn	– Nominal Range in sea miles

f	– Frequency of the sound in Hz

Nr 	– Sound Pressure Level, in decibels, of the sound emitted by the SOUND SIGNAL at the reference distance of 1 meter in the direction concerned.

Usual Range: the distance at which, in foggy weather, a lookout positioned in the wing of the bridge has a probability of 50% of hearing the signal when subjected to a noise as defined by IALA as being equal to or in excess of that found in 50% of large merchant vessels, the propagation between the sound signal emitter and the listener occurring during relatively calm weather and with no intervening obstacles.

Table 3 – Usual Range

| Sound Pressure Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pu(nm) f(Hz) |  | 0.5 |  | 1 |  | 1,5 |  | 2 |  |
| 25 | 155 |  | 162 |  | 165 |  | 168 |  |  |
| 50 | 144 |  | 150 |  | 154 |  | 157 |  |  |
| 100 | 132 |  | 139 |  | 143 |  | 146 |  |  |
| 200 | 125 |  | 132 |  | 136 |  | 140 |  |  |
| 400 | 117 |  | 125 |  | 130 |  | 135 |  |  |
| 800 | 112 |  | 121 |  | 128 |  | 134 |  |  |
| 1000 | 110 |  | 121 |  | 128 |  | 135 |  |  |
| 1250 | 109 |  | 121 |  | 129 |  | 137 |  |  |
| 1600 | 109 |  | 122 |  | 132 |  | 141 |  |  |
| 2000 | 108 |  | 123 |  | 136 |  | 148 |  |  |
| 2500 | 109 |  | 127 |  | 142 |  | 157 |  |  |
| 3150 | 110 |  | 132 |  | 152 |  | 170 |  |  |
| 4000 | 112 |  | 142 |  | 168 |  | 193 |  |  |

Pu	– Usual Range in sea miles

f	– Frequency of the sound in Hz

Nr 	– Sound Pressure Level, in decibels, of the sound emitted by the SOUND SIGNAL at the reference distance of 1 meter in the direction concerned.

- Power Supplies

## Types

A wide range of power systems and energy sources have been used or contemplated for operating lighthouses and floating aids.  Everything from clockwork to radio-active isotopes have been used.  Some of the more common types are listed in .

Table 4 – Power sources for operating lighted aids to navigation.

| Electric Energy Sources | Non-Electric Energy Sources |
| --- | --- |
| Primary cells | Acetylene |
| Commercial power supply | Propane |
| Diesel and petrol engine driven generators | Butane |
| Photovoltaic solar modules | Kerosene |
| Wind generators |  |
| Wave activated generators |  |
| Fuel cells using alcohol or hydrogen |  |

There is a general trend away from gas, using mains utility electricity where available and photovoltaic solar power where mains is not available.

## Non-Electric Energy Sources

There are various non-electric power supplies, acetylene and propane are described in this section.

### Acetylene

Acetylene (C2H2) has been used to operate lights on buoys and unattended aids to navigation for many years.  Acetylene can explode if compressed directly, but can be safely contained under low pressure in special cylinders when dissolved in acetone.

The manufacture of acetylene, standards for the cylinders and the filling process are usually controlled by government regulations.

Acetylene has been a convenient and reliable energy source for aids to navigation. However appropriate attention should be given to:

- safe handling of cylinders;

- the broad range of explosive mixtures with air (between 3 and 82% acetylene).

- the purity of the gas, and;

- minimizing leaks in pipe work and fittings.

### Propane

Propane gas (CH3 CH2 CH3) has been used as an alternative fuel to acetylene, particularly in buoys.  Although propane has to be consumed in an incandescent mantle burner to provide a white light, it has several advantages over acetylene:

- it is a by-product in oil refining processes;

- its abundance and low cost;

- propane liquefies at a pressure of 6 atmospheres at 17ºC, and can be transported in quite light and low cost gas bottles;

- propane will maintain a positive pressure down to -40ºC and will not freeze in conditions likely to be encountered at sea;

- placing the bottles in pockets in the buoy or by filling it directly into the body of an buoy, or pressure vessel;

- the comparable containers are the 20 kg propane bottle with gross weight of 48 kg and the 7,000 litre acetylene cylinder, weighing 105 kg;

- furthermore the cost of the propane bottle is only about one-third of that of a acetylene cylinder;

- propane is a particularly safe gas, as only some 6% of all its possible mixtures with air are explosive against a figure of 80% for acetylene;

- burns cleanly without the risk of sooting that can occur with a poorly adjusted acetylene burner.

Refer to IALA publication:

‘Practical Notes’ for the safe handling of gases, October 1993

Refer also to national standards for the safe handling of gases.

## Electric Energy Sources- Non-Renewable Sources

IALA has created a series of documents to assist in the selection of electrical power systems for aids to navigation.

Refer to IALA publications:

IALA Guideline 1042-0 on Selection of Power Systems for Aids to Navigation and Associated Equipment

IALA Guideline 1042-1 on The Total Electrical Loads of Aids to Navigation

IALA Guideline 1042-2 on [Electrical] Power Sources

IALA Guideline 1042-3 on Electrical Energy Storage for Aids to Navigation

### Primary Cells

Primary cells provide electrical energy by a non-reversible chemical process.  They were used in large numbers up until the 1980s to operate buoys and automatic beacon lights.  The usage of primary cells has declined sharply since commercial solar power (photovoltaic) modules have become available.  A related issue that hastened the decline of primary cells was the tightening environmental standards in a number of countries that required cells to be recovered from site for disposal in an approved manner.  Disposal compliance costs, and occupational health and safety aspects of the frequent change-out of primary cells have worked in favour of converting to renewable energy sources (e.g. solar, wind and wave generators).

#### Zinc-Air Cell

The zinc-air primary cell was a common energy source for operating buoy and beacon applications.  The cell uses a porous carbon block to supply oxygen from the air through an alkaline electrolyte to oxidize a zinc anode.  Individual primary cells have an open circuit voltage of about 1.2 volts and can supply 1000 to 2000 Ah at a maximum rate of about 1 ampere.

#### Lithium-Thionyl Chloride Cell

Another type of primary cell in use in buoy applications is the lithium-thionyl chloride cell.  This has a higher energy density and a longer shelf life than the zinc-air cell.

#### Sealed Alkaline Battery

Is commonly used in some countries, and has the benefits of good low temperature performance.

#### Sea-Water Cells

The sea-water cell developed for buoy applications in Norway is a primary cell that uses a magnesium anode and a largely inert copper cathode.  The sea water acts both as an electrolyte and the provider of dissolved oxygen for the cathode.

A single cell is installed as part of the buoy tail tube.  The motion of the buoy has a beneficial effect in agitating the water to provide an oxygen-rich flow through the cell and carry away the reaction products.

Copper was selected for the cathode material because of its inherent antifouling properties.  A magnesium anode was considered environmentally acceptable because it is a naturally occurring element of sea water.  The cell produces a voltage of 0.8 to 1 volt under load.  Components of the cells under evaluated have been sized to provide around 35,000 watt hours of energy.

Since it is impractical to use more than one cell due to the current leakage that would occur, an AC-DC converter is used to raise the voltage to the level required by the load.

### Internal Combustion Engine/Generators

#### Diesel Generators

Diesel engine driven generators are often used as the primary source of electrical power where the location of an aid to navigation is too remote to be supplied from a mains electricity grid.  Diesel generators are also used to provide emergency or backup power.

The generator capacity to support the operational and domestic loads of a standard lighthouse is in the range of 10 to 30 kW.  Diesel generators of this size are expected to consume around 0.4 litres/kWh.

The requirement for diesel generators in lighthouses is decreasing as a result of:

- lighthouse automation (destaffing), and because;

- new beacon and lamp technology that enables a light with a nominal range 18 to 20 nautical miles to be operated from a renewable energy source.

#### Petrol engine generators

Petrol engine generators are a useful source of power for maintenance work, but are less common in permanent installations due to:

- fuel storage and transportation safety issues;

- maintenance requirements on the spark-ignition system, and;

- the petrol engine generally being regarded as less durable than a diesel.

#### Thermo-electric generator

This is a solid-state generator in which a heat source, commonly from a propane burner, is directed onto a thermopile (i.e. an array of thermocouple type elements).  Since each thermocouple only produces a voltage of around 0.5 volts, a number are connected in series;

This type of generator has a low thermal efficiency (around 5%) and is rarely used.

#### Stirling engine driven generator

The Stirling engine is an external combustion engine that can be operated on gas or diesel fuel.  Packaged generator sets are available that could operate a lighthouse.  Typical generators produce 1kW electrical output and 5kW of heat.  The heat output could be a useful by-product for maintaining a constant temperature in a lighthouse.

#### Fuel cell

This is a solid-state device that uses a catalytic process to oxidise fuel to generate an electrical current. A common fuel is hydrogen, or hydrogen rich fuels.  It can be thought of as a continuously fed battery.

The commercial fuel cell is still an emerging technology and at this stage is an expensive power source.  Aids to navigation applications are likely to be limited to situations where solar energy (photovoltaic) is impractical due to limited insolation or icing conditions.

There is some interest in the use of fuel cells in hybrid power systems with wind energy or solar energy. These systems are still in development.

## Electric - Renewable Energy Sources

### Solar Power (Photovoltaic cell)

Solar power is an ideal power source for many aids to navigation applications.  It offers:

- a power source with no moving parts;

- no maintenance requirements other than being cleaned;

- negligible deterioration in power output over its life;

- low life-cycle costs;

Photo courtesy of Commissioners of Irish Lights Lights

When used to power a light, the battery recharging process is separated from the operation of the light so that the recharge voltage can be optimized without detriment to lamp life.

The potential difficulties associated with solar power are:

- finding ways to minimise bird fouling;

- mounting solar modules vertically is probably the best long-term solution for buoys;

- sizing arrays to operate at high latitudes;

- protecting solar modules from;

- wave damage on buoys;

- vandalism and theft

- lightning.

Aids to navigation exposed to icing conditions are perhaps the only applications unsuited to the use of solar modules.

Refer to IALA publications:

IALA Guideline 1012 for the protection of lighthouses and aids to navigation against damage from lightning

IALA Guideline 1039 on Designing Solar Power Systems for Aids to Navigation (with excel program)

IALA Guideline 1042 on Power Systems for Aids to Navigation

#### Types

The three common technologies employed in the manufacture of silicon based, solar modules are listed in Table 5:

Table 5 – Silicon solar cell technology.

| Type (Technology) | Comments |
| --- | --- |
| Monocrystaline cells | Are made from a thin slice cut from a single large crystal of silicon, usually produced as a circular section rod. Generally have the highest efficiency of the three technologies. If circular wafers of silicon are used the module fill factor is significantly less than with polycrystalline cells. It is now usual for the cells to be trimmed to approximate a square. |
| Polycrystaline cells | Are made from a thin slice cut from a large cast billet of silicon comprising many crystals. Are slightly less efficient than the mono crystalline cell but they can be shaped to completely fill the module. |
| Amorphous module | Are made by depositing thin films of silicon directly onto a glass or stainless steel substrate a thin slice cut from a single large crystal of silicon. The cell has a lower efficiency than either of other technologies but can be multi-layered for enhanced performance. Problems have been found with lifetime of these cells. |

In addition to the silicon cell technologies, there are two optional module configurations based on the numbers of series connected cells.  The standard module normally has 36 cells in series to give an open circuit voltage of around 20 volts.  For 12 volt battery charging applications, a voltage (charge) regulator is considered essential.

A self-regulating solar module was promoted as a means of eliminating the voltage regulator that had frequently been found to be the least reliable component in a solar power supply.  The self-regulating module typically has 32 cells in series to give an open circuit voltage of around 18 volts (and a maximum on load voltage of around 15 volts). In a self-regulating solar module, the charging rate is determined by the interaction between the electrical characteristics of the battery and solar module.

Both concepts have worked well work well.

#### Module or Array Orientation

In the northern hemisphere, solar modules are normally installed facing south (southern hemisphere facing north) and inclined at an angle to the horizontal that is related to the latitude of the site.

The inclination angle for solar modules is often optimised for the particular site as part of the sizing calculations.

One of the main problems experienced with solar powered aids to navigation has been bird fouling.  Numerous ingenious solutions have been trialed, generally with mixed results.  Generally solar modules mounted at an angle or vertically benefit from self washing from rain.

The cost of additional solar modules needed for a vertical installation may be largely off-set by the savings that result from simplifying the mounting arrangements or framework.

### Wind Energy

#### Aids to Navigation Applications

Wind generators (or wind turbines) have been used by a number of IALA Members to power aids to navigation.  The most popular type were horizontal axis machines with a two bladed (propeller type) turbine. The maintenance requirements arising from the moving parts design of wind generators and susceptibility to storm damage has limited the use of wind generators.

Installations

Wind generator installations at aids to navigation sites pose a number of problems:

- wind generators tend to require a lot of maintenance if operated in turbulent air flows;

- if the wind generator is installed on a separate mast some distance from the aid to navigation, consideration has to be given to cable voltage drop;

- the size of wind generator likely to be considered for operating aids to navigation is at considerable risk of damage if there are populations of birds at the site.

#### Wind Generator Types

A comparison of the typical performance of different types of wind generators is shown in Figure 5.

Figure 5 – Comparison of Performance of Wind Generator Types

### Wave Energy

The wave activated generator (WAG) was developed in Japan and has been successfully used to power lighted buoys.  The interaction between the buoy and wave motions acts as a simple air pump that is used to drive an air turbine and generator.  The WAG is mounted on an extension of a hollow tail tube that passes through the buoy hull.  With wave heights of 0.5 metres, the power output is almost 100 watts.  WAG’s have limited life and current systems suffer from excessive wear.

Site conditions will determine the rate at which the tail tube of the buoy accumulates weed and other forms of fouling, and these aspects need to be taken into consideration when developing the maintenance regime for the WAG. WAG’s can also be very susceptible to storm damage.

## Rechargeable Batteries

### Principal types

There are two main types of storage battery technologies applied to aids to navigation – lead acid and nickel cadmium.

The lead acid type is generally preferred because of its lower cost and higher energy exchange efficiency (95% vs. 80%) than the nickel cadmium battery.  However, the nickel cadmium battery can operate in lower temperatures and for a greater number of deep discharge cycles.

Recently, new secondary battery technologies have appeared, including lithium batteries, nickel-metal-hydride (Ni-MH) batteries and lithium-iron-phosphate (LiFePO4) batteries. The last offers lower weight and longer life span from more charge-discharge cycles.

#### Lead Acid

The basic form of this battery uses a lead dioxide positive plate and a lead immersed in an electrolyte of dilute sulphuric acid.  These were originally wet or flooded cells.  However in recent years various forms of “sealed” cell batteries have become available and are quite common in aids to navigation applications.

Lead acid batteries are available in two main designs, flooded lead acid, and valve regulated (VRLA). The VRLA comes in two types, absorbed glass-mat (that use a micro glass separator system to absorb the electrolyte), and gel batteries, that use a gelified electrolyte and polymeric separators to prevent short circuits between the positive and negative plates.

##### Flooded Lead-Acid Cells

There are three main types of flooded lead acid cell in general use:

- Plate Cells:  that have a pure lead positive plate with a large surface area, in which active material is formed from the lead of the plate itself.

- Tubular Plate Cells:  that use a tubular plate for the positive electrode.  The active material is contained in permeable insulating tubes through which the electrolyte can diffuse.

- Pasted Plate Cells:  in which the positive electrode is a pasted plate consisting of active material formed from oxides or salts of lead pressed into a lead alloy plate of grid form.

##### Valve Regulated Lead-Acid Cell (VRLA)

The VRLA comes in two types:

- AGM: (absorbed glass-mat) that use a micro glass separator system to absorb the electrolyte.

- Gel Batteries: that use a gelified electrolyte and polymeric separators to prevent short circuits between the positive and negative plates.

Both types have no “free electrolyte” and the batteries theoretically can be used in any position without acid leakage.  It is recommended that feature be confirmed before system designs are finalised.

The batteries use an oxygen recombination process so that normally there is no gassing. However, a safety valve is provided to release any excess pressure.

Operational characteristics of the battery include:

- a 5-10 year life on float-charge applications;

- tolerance to deep discharge cycles (i.e. 80%);

- gradual loss of capacity in cycling applications compared with a flooded cell.

#### Nickel Alkaline Battery

These batteries use compounds of nickel and, generally, cadmium with a solution of potassium hydroxide as the electrolyte.

Nickel-cadmium cells use perforated steel plates that hold the active material, mainly a nickel hydroxide in the positive plate and a cadmium compound in the negative plate.  The construction is generally referred to as a "pocket plate” cell.

A range of valve regulated nickel-cadmium batteries that use a recombination process now complements the traditional flooded cell design.  Under normal float charging conditions any gas produced is recombined within the battery and water loss is negligible.  However if the battery is overcharged it will vent but water can be added if necessary.

### Technical Considerations

#### Flooded cell or “Sealed”

The traditional lead acid and nickel cadmium flooded cell technologies have provided good service in many lighthouses and other fixed aids to navigation.  Large capacity flooded cell batteries (or “wet cells”) have a considerable cost advantage for large installations.  However, there are some important advantages of sealed batteries.  These advantages include the fact that sealed batteries:

- are suitable for both floating and fixed aids;

- facilitate standardisation;

- reduce on-site maintenance (work, time and cost);

- are safer to transport and to handle;

- reduce risk of occupational injuries from electrolyte spills;

- encourage recovery for appropriate disposal;

- are a less hazardous cargo for helicopters and small boats.

#### Technical Comparison

Table 6shows some of the technical differences between the lead-acid and nickel cadmium batteries.

Table 6 – A comparison of the performance characteristics of lead-acid and nickel cadmium batteries.

| Type (Technology) | Operating Temperature oC | Self-Discharge Rate Per Month % | Cycle Life to 80% DoD at 40ºC | Ah Charge Efficiency from 20% Discharged to 100% at 25ºC | Relative Cost (1990) |
| --- | --- | --- | --- | --- | --- |
| Tubular-plate flooded | -15 to 55 | 2 | 1800 | 83 | 1 |
| Flat plate flooded | -15 to 55 | 2 | 1200 | 80 | 1 |
| Tubular-plate VRLA | -30 to 50 | 3 | 1000 | >90 | 1.6 |
| Flat plate VRLA | 5 to 50 | 1 | 500 | >90 | 2 |
| Pocket plate VR | -40 to 50 | 2 | 2000 | 71 | 3.9 |
| Pocket plate flooded | -40 to 50 | 4 | 2500 | 71 | 3.9 |

### Battery Disposal

A number of countries now have standards and regulations relating to the safe and environmentally acceptable methods of disposing or recycling of batteries.

Electrical Loads and Lightning Protection

### Electrical Loads

IALA has prepared a standard methodology for calculating and defining the load profile of electric aids.  This covers the loads for:

- lights

- racons

- AIS AtoN

- electric sound signals

- fog detectors

- monitoring and telemetry systems

- charge controllers

- signal control equipment

Refer to IALA publication:

IALA Guideline 1042-1 on Total Electrical Loads of Aids to Navigation

### Lightning Protection

To assist those engaged in the design of aids to navigation, IALA has produced Guidelines to describe practical methods for the design, installation, inspection and testing of lightning protection systems.  The information covers lightning protection for aids to navigation structures, equipment and systems.

Refer to IALA publication:

IALA Guideline 1012 for the protection of lighthouses and aids to navigation against damage from lightning

### ENVIRONMENT/HAZARDOUS MATERIALS/MERCURY

# Mercury

A number of historic lighthouses still utilise rotating glass lenses and mercury float pedestals.  This was a clever method for providing a heavy lens with an almost frictionless bearing so that it could be turned by a clockwork mechanism.  However, given the toxic and corrosive properties of mercury, the following information may assist Authorities to implement appropriate safety procedures.

The mercury float pedestal for a first-order rotating lens contains about 13 litres of mercury.  Quantities of mercury may also be found in the electrical slip-ring units in rotating lamp array lighting equipment, some tilt-action switches, high current contact breakers, manometers and thermometers.

Physical Properties

Mercury is a heavy metal that has the unusual property of remaining liquid at normal temperatures ( above – 38 degC ).

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with "uncontained" mercury as a result of accidental spills.  Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber.  When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly.  Minute droplets will adhere readily to dust and can form particles that can be inhaled.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

- Vapour inhalation: Some vaporization from a free mercury surface will occur at normal room temperature and this is the most likely first contact that lighthouse personnel will have with mercury.  Unless the mercury vapour levels have been measured, personnel are unlikely to be aware of the hazard.  If the work-space around lighthouse equipment containing mercury is not well ventilated, the concentration levels can rise above recommended limits and there is potential for mercury poisoning:

- mercury vapour is heavier than air and in still air will tend to concentrate in low parts of the work-space. Well designed ventilation will allow such concentrations to disperse.

- Ingestion: can lead to acute mercurial poisoning;

- Absorption through the skin:- mercury is not easily absorbed through the skin.

Precautions

It is essential for the Authority to have detailed and strictly managed working procedures for all personnel working with, or in close proximity to mercury.

Staff must be trained in these work procedures and regularly medically monitored to ensure that they do not become contaminated with mercury.

The working procedures must follow national health and safety regulations and should be written by an expert in this field.

For work on optics the procedure will cover emptying, cleaning and re-filling the optic bath. Clean-up procedures will detail methods to recover all visible particles of mercury and the use of chemicals to neutralise smaller spills.

Personal protective equipment must be supplied that is specifically designed for use with mercury. This will include overalls, gloves, overshoes, respirator and eye protection. Procedures for the safe storage and disposal of this equipment must be in place.

A mercury vapour meter must be available to monitor the working environment and procedures in place for regular testing and calibration.

Consignment

Mercury is a hazardous substance and the relevant national and international regulations must be followed with regard to the type of container to be used, the packaging of this container for transport and the marking of this packaging.

Note:

Both IMO and the International Air Transport Association (IATA) have regulations covering the transportation of mercury.

Section 6.3

IALA Aids to Navigation Manual – IALA NAVGUIDE

NAVGUIDE 176 Edition 5 2006

UPDATE FOR 2010

Note for Editor:   Working Group 2 on Conservation, Heritage & Civil Engineering of EEP would request that the Lighthouse Conservation section of the Navguide is renumbered to give it a higher level of significance under the Chapter on Management Issues i.e. a first level number rather than second level as now e.g. 6.3. We have amended the text to suit current thinking.

6.3 Lighthouse Conservation

The IALA Advisory Panel on the Preservation of Lighthouses, Aids to Navigation,

and Related Equipment of Historic Interest (PHL) was established by the IALA

Council in 1996 in response to membership interest in the heritage value of

lighthouses. In 2002, this Panel, as Working Group 2, became part of the IALA Committee on Engineering, Environment, and Preservation of Historic Lighthouses (EEP)

Refer to IALA Publications

- IALA Lighthouse Conservation Manual

- Guideline on Agreements for Complementary use of lighthouse property

Objectives:

• to encourage deeper commitment by members to preserve historic values;

• to encourage member countries to see the conservation of their own lighthouses in

an international context;

• to share information on the subject between both members and non-members, with

particular attention being given to the complementary use of lighthouses.

• to research and document strategies on the conservation of historic lighthouses, particularly in relation to changes in technology and working practices.

Examples of work accomplished to date:

• the creation of the format for an IALA database for recording details of historic

lighthouses;

• a book, titled “Lighthouses of the World” was published in 1998 with English,

French, German and Spanish and Japanese versions, featuring over 180 historic lighthouses from around the world;

• Workshop in Kristiansand, Norway on “The Alternative Use of Historic Lighthouses” in 2000

• Seminar on the “Practical Aspects of Lighthouse Preservation” in 2005 in Gothenburg, Sweden

• the IALA Lighthouse Conservation Manual was published in 2006 to provide guidance to members on many aspects of Historic Lighthouse Conservation.

• Seminar on the “Heritage Issues of Introducing New Technologies in Aids to Navigation” in Santander, Spain in 2009

The purpose of the working group was reaffirmed in the conclusions of the 16th IALA Conference in Shanghai, China in 2006 as follows:

IALA should continue to play a role in the conservation of historic lighthouses, associated aids to navigation and equipment.

Regarding the conservation of historic lighthouses, IALA should continue to:

o Develop documentation;

o Promote international cooperation programmes; and

o Provide advice on lighthouse conservation

Note to Editor:  The working group also considers that existing sections 6.2.2.1 Lens Size and Terminology should be retained as 6.3.1 and 6.2.2.2 Third Party Access to Aids to Navigation Sites should be retained as 6.3.2

In these situations the beacon will generally show a colour scheme and topmarks in accordance with the IALA Maritime Buoyage System.

as adapted from the IALA Guidelines on Availability and Reliability of Aids to Navigation, Theory and Examples (Edition 2, December 2004).

See also IALA Recommendation E 110, The Rhythmic Character of Lights on Aids to Navigation.

For example, in waterways where the aid may be drifting or destroyed due to ice conditions.

The chemistry of the sea-water cell and the prototype light buoys using this cell have been described in papers for the 1990 IALA Conference and IALABATT 2 and 3.

Refer to IALABatt 3  “Fuel cells for aids to navigation”.

Table adapted from IALABATT 2 Overview of PV Battery Charging, Hyperion Energy Systems Ltd, Ireland.

The quantity of mercury used in higher order optics is shown in Section Error: Reference source not found.