ANM14/14/5

| From: ANM 13 | Formerly unnumbered ANM13/output/ |
| --- | --- |
| To: e-NAV Committee | April 2009 |

Liaison Note

Discussion paper on horizontal accuracy of radio-navigation systems

# Introduction

The minimum maritime navigational requirement for general navigation positioning by means of radio-navigation was developed at IMO and published as part of Resolution A915(22) in “The Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS)” and adopted on 29th November 2001 at the 22nd Session of the IMO Assembly.

The Resolution set out to identify the need for a future civil and internationally controlled global navigation satellite system to contribute to the provision of navigational position-fixing for maritime purposes throughout the world for general navigation., including navigation in harbour entrances and approaches and other waters in which navigation is restricted.

It was felt necessary to identify at an early stage the maritime user requirement for a future GNSS to ensure that such requirements are taken into account in the development of such a system. ( GNSS2 later known as GALILEO was being conceived at the time).

# OPERATIONAL REQUIREMENTS

The Resolution states that “Future GNSS(s) should meet the maritime users’ operational requirements for general navigation, including navigation in harbour entrances and approaches and other waters where navigation is restricted. The minimum maritime user requirements for general navigation are given in Appendix 2 to the Resolution”.

# HORIZONTAL ACCURACY

In Appendix 2 – The Table of minimum user requirements for general navigation – gives a horizontal accuracy requirement of 10 metres for the Ocean, Coastal, Port Approach, Restricted Waters and Inland Waterways together with a ‘Port’ requirement of 1 metre.

It has become clear that this requirement is causing concern due to its highly exacting accuracies and it is felt that the provision of radio-navigation systems to meet this requirement is unlikely to be cost-effective or, indeed, realistic.

# Next steps

The General Lighthouse Authorities of the United Kingdom and Ireland (GLA), having considered the minimum user requirements set down in Appendix 2 of IMO Resolution A.915(22), propose that these maritime horizontal accuracy requirements be reviewed because of the stringent levels set.   The GLAs propose a change in the requirement as follows:

Port Approach and Restricted waters	 	Horizontal Accuracy of 8 [to 20] metres

Coastal navigation				Horizontal Accuracy of 20 [to 50] metres

Ocean navigation				Horizontal Accuracy of 50 [to 100] metres

[the upper limits shown in square brackets may not be necessary]

These proposed user requirements have been considered by the ANM Committee and agreement reached that the IMO be requested to revise or replace Resolution A.915(22) in order that a more realistic and achievable level of horizontal accuracy be required.

# Action requested

The e-Navigation Committee is requested to consider this proposal from the ANM Committee and, if supported, submit an input paper to IMO, for approval by Council, requesting action to resolve this matter.