# OTHER FACILITIES

## Pilotage

### Pilotage as an Aid to Navigation

Pilotage is a specialised, and usually, licensed service that may be applied to navigation in “restricted” waters.  The skill draws on a local knowledge of the relative positions of geographic points or aids to navigation, submerged features, tides, currents and climatic conditions.

Pilotage may be required in coastal waters, estuarial waters, rivers, ports, harbours, lakes, canals, or enclosed dock systems or any combination of these areas.

When a pilot boards a vessel, it is customary for the pilot to be given "conduct of the vessel", but not “command”.  The role of the pilot is to:

- act as an adviser to the master, but this often includes:

  - giving the necessary instructions to the ships personnel operating equipment essential to the safe navigation and manoeuvring of the vessel;

  - assisting with local communication to a VTS centre, port control and other vessels;

  - communicating instructions to tugs and linesmen if berthing or sailing;

- provide current and specialist knowledge of;

  - local conditions and traffic;

  - operational status of aids to navigation;

  - sailing directions;

  - restrictions applicable to the piloted vessel;

In addition the pilot needs to be able to adapt quickly to:

- operational culture aboard the vessel;

- the handling characteristics of the vessel and;

- state of navigation equipment aboard (and able to compensate for deficiencies).

### Types of Pilotage

Pilotage is commonly applied within declared ports but may also be applied to some coastal areas, lakes and inland waterways.  These areas would normally fall within the definition of “restricted waters” (see Section xxxxx).

Where pilotage services are licensed, it is usual for the applicable pilotage area to be stated on the licence.  The service provider may then be described as a port pilot or a coastal pilot etc.

Various levels of enforcement can be applied to a pilotage area:

- Compulsory (Mandatory) pilotage:- Applicable vessels must take a pilot when entering a declared area.

  - A country may seek IMO approval for an area to be declared a Particularly Sensitive Sea Area (PSSA).  If approved, the declaration allows special marine environmental protection measures to be applied to shipping. This can include compulsory pilotage arrangements. (IMO Resolution A710 (17) is an example)

- Recommended pilotage:- An authority can promulgate notices recommending that masters of applicable vessels, who are unfamiliar with a particular area, should engage a licensed pilot.

### Other Pilotage Considerations

- Pilot Services can be provided by public or private operators.  However generally the pilot licensing authority would be a government agency.

- The IMO has set the minimum standards for pilots and includes recommendations on the qualification and training of pilots.  However individual countries may impose more stringent standards.

- When developing proposals for marking restricted waterways, the requirements for pilotage services should be considered concurrently with the selection of the aids to navigation.

## Audible Signals

Although Audible Signals still exist, it has been IALA policy since 1985 that these devices should only be used as a hazard warning.

It is a matter for the appropriate authority to determine whether a hazard requires an audible signal.  Certain man-made obstructions such as offshore structures, bridges and breakwaters are examples of applications for audible signals.

Refer to IALA documents: - Recommendations for the calculation of the range of a sound signal (E-109), latest edition; - Recommendations for the marking of fixed bridges over navigable waters (O-113), latest edition; - Recommendations for the marking of offshore structures (O-114), latest edition.

## nautical publications

### Navigational Warnings

SOLAS Chapter V Regulation 13 requires for contracting governments to provide navigational information to mariners.  The Regulation 13 states:

Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

This information falls into three basic categories:

- information about planned changes, such as:

  - dredging, surveying, pipe and cable laying;

  - changes to an existing aid or the establishment of new aids to navigation;

  - changes to traffic arrangements;

  - commercial maritime activities;

  - short term events (naval exercises, yacht races, etc.).

- information about navigational un-planned events, such as:

  - the failure to aids to navigation;

  - marine incidents (groundings, collisions, wrecks etc.) ;

  - search and rescue activities.

- new information arising from survey work or previously undiscovered hazards.

### World-Wide Navigational Warning Service

The promulgation of information on navigational safety is coordinated by means of the World-Wide Navigational Warning Service that was established jointly by the IMO and the IHO in 1977.

The World-Wide Navigational Warning Service is administered through 16 NAVAREAS, as is shown in Fig 6.5.  Each NAVAREA has an Area Coordinator who is responsible for collecting information, analysing it, and transmitting NAVAREA Warnings.

Figure 6.5

### Lists of Aids to Navigation

#### List of Lights and List of Radio Signals

Lists of aids to navigation are produced by (or for) most national authorities as part of the navigational information made available to mariners in support of SOLAS Chapter V Regulation 13.  They provide details of:

- name;

- location;

- the characteristics of the aids;

- operating schedule.

These lists will not always include buoys and unlit aids to navigation.

#### Standard Descriptions

Joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ 15).

This document provides definitions of standard terms to describe particular events that should be used when composing Navigational Warnings.  Some of the terms that are relevant to the condition of aids to navigation have been defined as follows:

Sample of the COMSAR/Circ 15 standard terms.

| Descriptors for Lighthouses, Beacons Buoys and Lightvessels | Comments |
| --- | --- |
| UNLIT | Incorrect Terms include: Out, Extinguished, Not Burning, Not Working |
| LIGHT UNRELIABLE | Incorrect Terms include: Weak, Dim, Low Power, Fixed, Flashing incorrectly, Out of Character |
| DAMAGED | Use only for major damage where there is a significant loss of functionality |
| DESTROYED | Incorrect Terms include: Temporarily destroyed |
| ESTABLISHED (+ location) | New light |
| OFF STATION | Buoy (lightvessel) not in the charted position |
| MISSING | Buoy (lightvessel) completely absent from position |
| RE-ESTABLISHED | Only appropriate for lights that have previously been Charted or listed as Destroyed |

The above list of terms and definitions do not adequately cover all of the situations that an Authority might want to use when issuing a Navigation Warning.  An expanded set of definitions of terms for use in Navigation Warnings is provided for consideration in the following table.

A suggested expanded list of terms and definitions for use in Navigation Warnings

| Term | Definition |
| --- | --- |
| Station | The authorised and exact location of an aid to navigation. |
| Established in position | Any type of aid placed in operation for the first time at a given station. |
| Re-established in position | Any type of aid placed in operation at a station at which a similar type of aid with identical characteristics had been previously established, but subsequently destroyed, withdrawn or discontinued. |
| Unlit | When a light is out because of defective equipment, or any unintentional or deliberate occurrence and it is intended to restore it to normal as soon as practicable. |
| Unreliable | When an aid of any type is not exhibiting its correct characteristics and it is intended to restore it to normal as soon as practicable. |
| Reduced power | When an aid of any type is not operating at its correct power, but is exhibiting the correct characteristics and it is intended to restore it to normal replace it as soon as practicable. |
| Off station | When a floating aid is adrift, missing or out of position and it is intended to replace it as soon as practicable. |
| Altered | When the characteristics or structure of any aid have been altered, without changing the type of aid or its station. |
| Altered in position | When a change is made to the station of an aid (ie its location) without changing the type of aid, character or type of structure. |
| Destroyed | Any type of aid that has been damaged to the extent that it is no longer of use as an aid to navigation, but remnants of the structure may remain. |
| Restored to normal | Any type of aid that has been previously described as unlit, unreliable, reduced power or temporarily discontinued and has now been serviced so as to exhibit its correct characteristics and power. |
| Replaced in position | When a floating aid previously described as off station or temporarily discontinued is returned to station or replaced by another with the same characteristics. |
| Temporarily replaced by | When any aid is discontinued, temporarily withdrawn or off station and another aid of different type or characteristics is immediately established at the same station. |
| Temporarily withdrawn | When a floating aid has been entirely removed from its station and no similar aid is left in its place, but it is intended to re-establish the aid in the near future. |
| Temporarily discontinued | When a sound signal or radionavigation service is silent because of maintenance requirements, or any unintentional or deliberate occurrence, and it is intended to restore it to normal as soon as practicable. |
| Permanently withdrawn | When a floating aid has been entirely removed from its station with no similar aid is left in its place and it is not intended to re-establish that aid in the near future. |
| Permanently discontinued | When any aid, other than a floating aid, is removed from a station or the service is terminated or silenced because it is no longer required. |

Positions and Bearings

Positions

The Joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ 15) states that positions should always be given in Degrees, Minutes and decimal minutes in the form:

- DD-MM.mmm N or S;

- DDD-MM.mmm E or W;

- leading zeros should always be included;

- the same level of accuracy should be quoted for both Latitude and Longitude.

Recording of Aids to Navigation Positions

IALA has produced a Recommendation for the recording of aids to navigation positions (O118), June 2000.

The Recommendation states that:

- where an Authority has operational DGPS stations, a program should be implemented to determine the WGS84 positions of each aid to navigation (fixed and floating) within the coverage area, and for this information to be passed to the national hydrographic authority for future use.  It is anticipated that the information would assist the hydrographic authority in checking the accuracy of charts, planning future survey requirements and for updating List of Lights.

- in the case of lighted fixed aids to navigation the WGS84 position should be measured close to the focal centre of the light so that the WGS84 elevation is also determined.  Alternatively, several positions around the optic or lantern house could be measured and a central position computed.

- in the case of unlighted fixed aids to navigation the WGS84 position should be the base of the structure.

- in the case of floating aids to navigation the WGS84 position should be the position of the anchor.

- each position should be recorded to three decimal places of a minute and include the time, date and details of the measuring equipment.

  - where an Authority has to refer to charts of different datums, positions are communicated with the appropriate datum reference.

  - for example 51 04.372’N, 100 26.794’E (WGS 84).

Bearings

Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner.  Observing a practice of communicating bearings with the suffix  ‘TBS” or True Bearing from Seaward will minimise the risk of confusion.

## Maritime safety information

### Maritime Safety Information (MSI)

Within a NAVAREA, there can be a hierarchy of Warnings promulgated by the National co-ordinator.  Collectively referred to as Maritime Safety Information (MSI).  The Warning hierarchy covers:

- NAVAREA Warnings:- that are concerned with information that ocean-going vessels require for safe navigation;

  - are transmitted in English and, where appropriate, in other languages;

  - are promulgated by;

  - radiotelephony;

  - Digital Selective Calling (DSC);

  - Enhanced Group Calling (EGC);

- NAVTEX  (used for the automatic broadcast of localised Maritime Safety Information (MSI) using radio telex);

- covers the specific NAVAREA and portions of adjacent areas;

- details of the broadcast schedules are shown in the List of Radio Signals published by Hydrographic Offices and in the publications of the ITU;

- are generally promulgated for a sufficient period of time to ensure its safe reception after which it is cancelled or published in a Notice to Mariners;

- Coastal Warnings:- that are concerned with information relating to a regional area covering 100-200 nautical miles from the coast.  These are:

  - transmitted from a national network of coastal radio stations;

  - broadcast at scheduled times;

  - use English and the national language;

- Local Warnings:- that cover the area within the limits of a harbour or port authority:

  - to supplement Coastal Warnings, and;

  - may be limited to the national language.

Information concerning navigational warnings can be obtained from the Joint IMO/IHO/WMO Manual on Maritime Safety Information (MSI), February, 1998.

### Off-Station Warnings for Major Floating Aids

IALA Recommendation for off-station signals for major floating aids (O -104), November 1989 states that: - When any Lightvessel, Lightfloat or Lanby (LNB) manned or unmanned is out of position such that it could be misleading to navigation, all its aids to navigation (lights, sound signals, racon, radio beacon) should be discontinued. - To avoid the risk of collision with passing vessels, warning lights should be continuously displayed as follows: - Two all-round red lights in a vertical line similar to those prescribed by Rule 27 of the COLREGS for a vessel "Not under command". - If the appropriate Administration requires a sound signal to be operated, it should be coded MORSE "D" as prescribed by Rule 35 of the COLREGS for a vessel "Not under Command" - If a Racon is deployed, it should be coded MORSE "D".

## Electronic Chart Display and Information System (ECDIS)

### Description

Although ECDIS is not an “aid to navigation” as defined by IALA, it deserves to be mentioned because this piece of ship borne equipment is likely to bring major changes to the ways that vessels are navigated.  ECDIS uses digital vector data in a way that will allow it to replace the traditional paper chart with a more versatile electronic product that can draw on a variety of positioning and data inputs, such as GPS, DGPS, radar, echo sounder, compass, an electronic chart, navigational publications and the chart amendments.

### Performance Standards

The performance standards for ECDIS have been defined by the International Maritime Organization (IMO), in conjunction with the International Hydrographic Organization (IHO).

IMO Resolution A.817(19) enables marine administrations to accept ECDIS as a legal equivalent to the paper charts that are required to be carried for compliance with SOLAS regulation V/20.

There are two key performance elements to ECDIS:

- An approved processing system (or ‘box’), and;

- Electronic Navigational Charts (ENCs) that have been prescribed by a national hydrographic office and meet the standards set down in the 3rd Edition of the IHO Special publication No.57 (S57)

While an ECDIS ‘box’ may be capable of reading other forms of electronic charts, it ceases to be a compliant system without the official ENCs.

All other non-ECDIS electronic charts are classified as Electronic Chart Systems (ECS).  These include:

- Raster Navigation Charts (RNC) that are effectively electronic copies of paper charts, and;

- electronic charts that are not issued by a national hydrographic authority or differ from the S57 standard .

### Commercial Availability

The availability of ECDIS for commercial use has, and continues to be, limited because of the time it has taken to collect and compile the digital vector data sets.  However, a number of ships are being used to evaluate Electronic Navigational Charts (ENC) over defined routes.

Many national hydrographic authorities have embarked on major projects to collect and compile ENC data but may also issue RNC data.  Various companies are also producing ECS data.

## Tide Gauges and Current Meters

### Purpose

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping.  The latter is generally used to overcome the sometimes considerable differences between actual tide heights and predicted values due to meteorological and mean sea level fluctuations.

### Intergovernmental Oceanographic Commission

The Intergovernmental Oceanographic Commission (IOC) is responsible for co-ordinating the Global Sea Level Observing System (GLOSS) program to establish global and regional networks of sea level stations for providing essential information for international oceanographic research programmes, including those dedicated to the study of aspects of climate change, and operational oceanography.  The main component of GLOSS is a network of 287 sea level stations worldwide (referred to as the 'Global Core Network').

IALA supports and encourages participation in the GLOSS program.

Authorities that are procurement or upgrading of Sea Level Measurement devices, are encouraged to consider using equipment that can support the requirements of the GLOSS program.  Typically this calls for gauges capable of measuring to centimetre (1 cm.) accuracy in all weather (especially wave) conditions and for the free exchange of hourly sea level data to an International Sea Level Centre.

Information on the GLOSS Programme can be found at: http://www.pol.ac.uk/psmsl/programmes/gloss.info.html.

Technical recommendations on sea level observations can be found at: http://www.pol.ac.uk/psmsl/manuals/.

Verify the info Refer to IMO Resolution A.485(XII) and NAV 46.

Also known as Narrow Band Direct Printing, or NBDP.