The provision of buoys, leading lights and sector lights in waterways

and the resultant risk reduction values

1.	Introduction

1.1	The optimum number and arrangement of buoys, leading lights and sector lights provided in a waterway depend substantially on the average channel width, the channel length, whether the section is straight or curved, the functions that these Aids to Navigation perform and the ease by which they can be used to assist mariners.

1.2	These Aids to Navigation can be considered to be provided for two related but different purposes;

1.2.1	Buoys are provided basically to mark the channels in a waterway in which it is safe for ships to navigate and take into account geographical and other natural hazards.  There are four different arrangements of buoys commonly in use, they are: single centre buoys; single staggered buoys: unsynchronised paired buoys; and, synchronised buoys.

1.2.2	Leading lights and sector lights are provided basically to indicate the safest route to sail in a channel and the adjustments in course necessary to attain that route.  However, it is unlikely that both leading lights and sector lights will be used together in the same section of a channel.

1.3	A reasonable combination of buoys, leading lights and sector lights normally used in channels of different average widths is given in the following Table:

| Channel Width (Metres) | Arrangement of buoys |  |  |  | Leading lights or Sector lights |
| --- | --- | --- | --- | --- | --- |
|  | Single |  | Paired |  |  |
|  | Centre | Staggered | Unsynchronised | Synchronised |  |
| 100 – 400 |  |  |  |  |  |
| 400 - 800 |  |  |  |  |  |
| 800 - 1600 |  |  |  |  |  |

Table 1.  Combination of buoys, leading lights and sector lights used in channels of different width

2.	Optimum buoy separation distances and standard number of paired or single buoys.

2.1	The optimum separation distance between buoys in a waterway differs depending on the length of the channel, its average width and whether the section is straight or curved.  The optimum separation distance for buoys in any section of a waterway can be calculated as follows:

- The optimum separation between paired buoys along a straight section is the length of the section divided by 3 times the average channel width;

- The optimum separation between paired buoys along a curved section is the length of the section divided by 3.2 times the average channel width;

- The optimum separation between single buoys along a straight section is the length of the section divided by 2.1 times the average channel width;

- The optimum separation between single buoys along a curved section is the length of the section divided by 2.25 times the average channel width;

2.2	The standard number of paired or single number of buoys for a section of a waterway is a function of the length of the section and the appropriate optimum buoy separation distance, as follows;

- The standard number of paired buoys for a section of a channel is twice the length of the section divided by the optimum separation of each pair of buoys.

- The standard number of single buoys for a section of a channel is the length of the section divided by the optimum separation of each buoy.

3.	Risk reduction values of buoys, leading lights and sector lights.

3.1	Risk reduction values should be highest in those sections of a waterway which have the greatest risk of groundings or collisions, that is, as the channels become narrower, the greater should become the risk reduction values.

3.2	The overall range of risk reduction values arising from the use of buoys, leading lights and a sector light used in these Guidelines is from 0 to 30.  The range for buoys as given in Figure 1 below is from 0 to 24 and the remaining 6 being applied when leading lights or a sector light is provided in addition to buoys.  However, it is recognised that this range may need to be changed so that it is compatible with the requirements of the Administration concerned.

3.3	Using the calculations given in 2 above, the risk reduction values for unsynchronised paired buoys can be derived from Figure 1.

3.3.1	The risk reduction values for synchronised paired buoys can be determined from Figure 1 by multiplying the risk reduction value appropriate to the percentage of the standard buoy number by 1.2,

3.3.2	The risk reduction for staggered single buoys can be determined from Figure by multiplying the risk reduction value appropriate to the percentage of the standard buoy number by 0.9.

3.3.3	The risk reduction for centre single buoys can be determined from Figure 1 by multiplying the risk reduction value appropriate to the percentage of the standard buoy number by 0.8.

3.4	Applying Figure 1 and appropriate calculations the maximum range of risk reduction values for each combination of buoys and or leading lights/sector lights is given in the Table 2.

| Channel Width (Metres) | Arrangement of buoys |  |  |  | Leading lights or Sector lights | Range of risk reduction values |
| --- | --- | --- | --- | --- | --- | --- |
|  | Single |  | Paired |  |  |  |
|  | Centre | Staggered | Un-synchronised | Synchronised |  |  |
| 100 – 400 |  |  |  |  |  | 6 - 30 |
| 100 - 400 |  |  |  |  |  | 6 - 26 |
| 100 - 400 |  |  |  |  |  | 0 - 24 |
| 100 - 400 |  |  |  |  |  | 0 - 20 |
| 400 - 800 |  |  |  |  |  | 0 - 24 |
| 400 – 800 |  |  |  |  |  | 0 - 20 |
| 400 - 800 |  |  |  |  |  | 0 - 18 |
| 400 - 800 |  |  |  |  |  | 0 - 16 |
| 800 - 1600 |  |  |  |  |  | 0 - 20 |
| 800 - 1600 |  |  |  |  |  | 0 - 18 |
| 800 - 1600 |  |  |  |  |  | 0 - 16 |

Table 2.  Range of risk reduction values for various combinations of buoys, leading lights and sector lights

4.	Summary of calculations

To determine the combined risk reduction factor of buoys, leading lights and sector lights in a section of a channel the following steps should be taken:

4.1.	Determine the standard number of buoys for the section using the following formula:

For paired buoys,

Sn =        2(channel length of section)

x(average channel width of section)

where	Sn = standard number; and,

x = 3 for paired buoys in a straight section of the channel; or,

3.2 for paired buoys in a curved section of the channel.

For single buoys,

Sn =        channel length of section

x(average channel width of section)

where	Sn = standard number; and,

x = 2.1 for single buoys in a straight section of the channel; or

2.25 for single buoys in a curved section of the channel

4.2.	Count the number of buoys in the section and calculate the percentage of buoys compared with the standard number of buoys for the section relevant to the buoy arrangement.

4.3.	Determine the risk reduction value from the graph “Risk Reduction values for unsynchronised paired buoys”.

4.4.	To take into account the comparative ease of use of the various buoy arrangements by mariners, multiply the risk reduction value determined from the graph by the appropriate factor from the following:

Synchronised paired buoys	1.2

Unsynchronised paired buoys	1

Staggered single buoys		0.9

Centre single buoys		0.8

The value obtained is the resultant risk reduction factor for the section of the channel if leading lights or a sector light is not provided.

4.5.	Where leading lights or a sector light is provided in the channel, add 6 to the calculated risk reduction factor for the buoys determined by steps 1 to 4.

The value obtained is the resultant risk reduction factor for the section of the channel if leading lights or a sector light is provided.

Notes

The figures of 3.2 times the channel width for the optimum separation of paired buoys on a curved section and 2.25 times the channel width for the optimum separation of single buoys on a curved section are based on decreasing the separation distance between pairs of buoys in curved sections by approximately 7%.  However, as more experience is gained, it may be advisable to review this value and introduce a variable percentage of decrease depending on the characteristics of the curve, such as its radius and how much of the curve is visible from a ship entering the section of the channel.

The graph in Figure 1 conforms to the square law Kn2, where “K” is a constant used to establish the overall risk reduction range scale and “n” is a value on the y axis from 0 to 10.  If it is required to change the overall scale from 0 to 20 to, say, 0 to 30, it is necessary only to change the value of K from 2 to 3.

Figure 1 is based on unsynchronised buoys because this is considered to be the most common arrangement of buoys.

The weighting of the various buoy arrangements is based on the completeness of the information provided to mariners and the ease with which it can be used to assist in the safe navigation of ships.