Annex 4

of

Input paper for IALA ANM 8, point 6 of the provisional agenda

Develop guidance on channel design and a minimum comprehensive mix of AtoN

| Fairway width | Excellent A to N |  |  | Good A to N |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PIANC additional width | Navigation accuracy (IALA paper) | a) Mix, b) buoys: 1: percent of stand. number 2: separation distance | PIANC additional width | Navigation accuracy (IALA paper) | a) Mix, b) buoys: 1: percent of stand. number 2: separation distance |
| 100 | 0 | 10 | a) only buoys 1: 100 % 2: 300 m | 10 | 15 | a) only buoys 1: 73 % 2: 410 m |
| 200 | 0 | 20 | a) leading lights and buoys 1: 90 % 2: 660 m | 20 | 30 | a) leading lights and buoys 1: 55 % 2: 1100 m |
| 400 | 0 | 40 | a) all included b) buoys 1: 90% 2: 1300 m | 40 | 60 | a) all included b) buoys 1: 60% 2: 1950 m |