IALA Recommendation on the marking of fish farms

THE COUNCIL

RECOGNISING the increase in number of fish farms being established and the potential danger it poses to shipping,

RECOGNISING ALSO that it is a matter for a National Authority to decide on whether a fish farm needs to be marked, depending on the risk involved,

RECOGNISING FURTHER that marking is to improve the safety of navigation and protect the fish farm itself,

HAVING CONSIDERED the proposals by the IALA Operations Committee,

RECOMMENDS

1.	that when a fish farm is considered to represent a danger to shipping it should be marked using the IALA Maritime Buoyage System; special marks, lateral or cardinal marks, or a combination thereof.

1.1	If special marks are used, the perimeter of the fish farm should be marked but depending on the size, extent and location of a particular farm, it may be sufficient to mark only part of the perimeter or the centre, as specified better to the following paragraph 2.

1.2	If there is a requirement for vessel movements through a fish farm, that a channel is marked with lateral marks.

1.3	If the prevailing situation warrants it, cardinal marking alone may be used to steer shipping away from the fish farm.

1.4	To improve the visibility of the lights, the synchronisation of the various characters of the lights may be considered.

2.	a simple method to establish the number and the position of the special marks necessary to delimit a fish farm is that one to distinguish the followings two possible cases:

·	rectangular fish farm;

	circular fish farm.

2.1	in the case of a rectangular fish farm, in operation of the length of the sides and the extension of the area, the following cases can be verified (Table 1):

2.1.1	Type 1:

Size:	side A and side B    500 metres.

Prescriptions:	only a light buoy in the middle of the fish farm.

2.1.2	Type 2a:

Size:	side A (sea side and coast side)    2.500 metres

side B (lateral side)    500 metres.

Prescriptions:	two light buoys to the vertices of the side sea and two day buoys to the vertices of the coast side.

2.1.3	Type 2b:

Size:	side A (sea side and coast side)    500 metres

side B (lateral side)    2.500 metres.

Prescriptions:	two light buoys to the opposite vertices of the fish farm and two days buoys to the vertices remainders.

2.1.3	Type 3:

Size:	side A and side B  500 and    2.500 metres
such as A  B ≤ 2.250.000 metres2

Prescriptions:	two light buoys to the opposite vertices of the fish farm and two days buoys to the vertices remainders.

2.1.3	Type 4:

Size:	side A and side B  900 and    2.500 metres
such as A  B  2.250.000 metres2 and  6.250.000 metres2

Prescriptions:	four light buoys at the vertices of the fish farm.

2.2	in the case of a circular fish farm, in operation to the length of the radius, the following cases can be verified (Table 2):

2.1.1	Type 1:

Size:	radious    500 metres.

Prescriptions:	only a light buoy in the middle of the fish farm.

2.1.2	Type 2:

Size:	radious   500 and    1.000 metres.

Prescriptions:	two light buoys to the opposite extreme on the circumference of the area and two day buoys on the circumference of the area positioned with an angle of 90 degrees respect the aforesaid light buoys.

2.1.3	Type 3:

Size:	radious   1.000 and    2.000 metres.

Prescriptions:	three light buoys on the circumference of the fish farm positioned with angle of 120 degrees the one from the other.

2.1.4	Type 4:

Size:	radious   2.000 metres.

Prescriptions:	three light buoys on the circumference of the fish farm positioned with angle of 120 degrees the one from the other and three day buoys on the circumference of the area positioned with an angle of 60 degrees respect the aforesaid light buoys.