- Navigation Accuracy by the use of buoys and leading lights in restricted waterways

- 1.	Introduction

1.1	Navigation Accuracy in a restricted waterway is a combination of the accuracy of determining position relative to a geodetic datum and navigating safely along the waterway.

Current methods of determining position accuracy can be considered as comprising of:

- Radionavigation systems, both satellite and terrestrial, where the position is related to the WGS 84 geodetic datum; and,

- Conventional Aids to Navigation systems, including lights, buoys and radar targets, where the position is related to the geodetic datum of the chart used on board the vessel concerned.  However many charts now include transformation information to enable information that is referenced to WGS 84 to be plotted.

Current methods of providing information to enable a vessel to navigate safely along a waterway can be considered as comprising of:

- Electronic Chart Display and Information Systems (ECDIS) incorporating an electronic chart to WGS 84 datum on which the position of the ship, as determined by a radio navigation system can be superimposed in the waterway and the required changes in course and speed to navigate safely can be seen readily on the display.  However, conventional Aids to Navigation are normally used when navigating visually and are also necessary to provide back-up in case of errors from the radionavigation system or malfunction of the shipborne equipment; and,

- Conventional Aids to Navigation, including lights, buoys and radar targets, by which the required changes in course and speed to navigate safely can be seen either by relating the ranges and bearings of the Aids to Navigation to a paper chart or, where an adequate number of aids are provided, visually in good visibility and by radar in bad visibility.

1.2.	Where Conventional Aids to Navigation are used to assist in navigating a vessel safely along a waterway, one important consideration is the number of such aids that are provided.  This is because the aids primarily indicate the limits of the navigable waterway and therefore can be expected to increase in number as the width of a waterway decreases and where changes in course are needed.

1.3.	For the purpose of this paper, the minimum navigational accuracies for two-way waterways are:

- For waterways 100 metres width or less - 10% of the width of the waterway.

- For waterways exceeding 100 metres width – starting at a percentage of 10% for a waterway of 100 metres width, the percentage is increased by 1% for every additional 20 metres in channel width, up to a maximum of 45% of the width of the waterway.

Table 1 shows the incremental increase of percentage for each 100 metres of channel width from 100 metres to 800 metres and the resultant navigation accuracies.

| Width of waterway in metres | Navigation Accuracy % of width metres |  |
| --- | --- | --- |
| 100 | 10% | 10 |
| 200 | 15% | 30 |
| 300 | 20% | 60 |
| 400 | 25% | 100 |
| 500 | 30% | 150 |
| 600 | 35% | 210 |
| 700 | 40% | 280 |
| 800 | 45% | 360 |

Table 1

- It should be noted that no account is taken in this paper of the density or distribution of marine traffic using a waterway.  Consequently, consideration should be given on whether, depending on the marine traffic, a more stringent Navigation Accuracy should be applied in a waterway in order to reduce risks to an acceptable level.  Where a more stringent Navigation Accuracy is to be applied Tables 2 or 3, as appropriate, can be used to indicate the percentage of the standard number of buoys that will probably provide the required accuracy.

- 2.	Navigating Accuracies from the use of buoys and leading lights

2.1	The optimum separation distance between buoys in a waterway differs depending on the length of the channel, its average width and whether the section is straight or curved.  The optimum separation distance for buoys in any section of a waterway can be calculated as follows:

- The optimum separation between paired buoys along a straight section is 3 times the average channel width;

- The optimum separation between paired buoys along a curved section is 2.8 times the average channel width;

- The optimum separation between single buoys along a straight section is 2.1 times the average channel width;

- The optimum separation between single buoys along a curved section is 2 times the average channel width;

2.2	The standard number of paired or single number of buoys for a section of a waterway is a function of the length of the section and the appropriate optimum buoy separation distance, as follows;

- The standard number of paired buoys for a section of a channel is twice the length of the section divided by the optimum separation of each pair of buoys.

- The standard number of single buoys for a section of a channel is the length of the section divided by the optimum separation of each buoy.

It is assumed that the standard number of buoys will provide navigation accuracy in the order of 10% of the average width of the waterway.  When less than the standard of number of buoys is provided, the navigation accuracy obtainable from the buoyage will be proportional to the increasing navigational accuracy factor (NAF) as shown in Fig. 1.

Figure 1

2.3	NAF can be calculated by determining the actual number of buoys in a section of the waterway as a percentage of the standard number of buoys, using the formula:

NAF =     1

FNB

Where  FNB is the decimal fraction of the percentage of the number of standard buoys, e.g, 80% = 0.8 and the resultant Navigation Accuracy factor for 80% of the standard number of buoys is 1.25.

2.4	Table 2 contains an assessment of the navigational accuracies for waterways of width between 100 and 700 metres when navigation is by buoys. The Table also indicates the minimum percentage of the standard number of buoys that would provide navigation accuracy suitable for the width of the waterway concerned.

| Navigation Accuracies by use of Buoys |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Width of waterway in metres | Navigation Accuracy % of in width Metres |  |  | Percentage of Standard Number of buoys |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 100% | 90% | 80% | 70% | 60% | 50% | 40% | 30% | 20% |  | 10% |  |
| 100 | 10% | 10 |  | 10 | 11 | 12 | 14 | 17 | 20 | 25 | 33 |  | 50 |  | 100 |
| 200 | 15% | 30 |  | 20 | 22 | 25 | 27 | 33 | 40 | 50 | 67 |  | 100 |  | 200 |
| 300 | 20% | 60 |  | 30 | 33 | 37 | 43 | 50 | 60 | 75 | 100 |  | 150 |  | 300 |
| 400 | 25% | 100 |  | 40 | 44 | 50 | 57 | 68 | 80 | 100 | 134 |  | 200 |  | 400 |
| 500 | 30% | 150 |  | 50 | 55 | 62 | 71 | 83 | 100 | 125 | 167 |  | 250 |  | 500 |
| 600 | 35% | 210 |  | 60 | 67 | 75 | 86 | 100 | 120 | 150 | 200 |  | 300 |  | 600 |
| 700 | 40% | 280 |  | 70 | 78 | 87 | 100 | 117 | 140 | 175 | 234 |  | 350 |  | 700 |

- Table 2

2.5	Table 3 contains an assessment of the navigational accuracies for waterways of width between 100 and 400 metres when navigation is by leading lights and buoys. The Table also indicates the minimum percentage of the standard number of buoys that would provide navigation accuracy suitable for the width of the waterway concerned.

| Navigation Accuracies by use of Leading lights and buoyage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Width of waterway in metres | Navigation Accuracy % of in width Metres |  |  | Leading Lights and Buoys Percentage of Standard Number of buoys |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 100% | 90% | 80% | 70% | 60% | 50% | 40% | 30% | 20% |  | 10% |  |
| 100 | 10% | 10 |  | 9 | 10 | 11 | 12 | 14 | 17 | 20 | 25 |  | 33 |  | 50 |
| 200 | 15% | 30 |  | 18 | 20 | 22 | 25 | 27 | 33 | 40 | 50 |  | 67 |  | 100 |
| 300 | 20% | 60 |  | 27 | 30 | 33 | 37 | 43 | 50 | 60 | 75 |  | 100 |  | 150 |
| 400 | 25% | 100 |  | 36 | 40 | 44 | 62 | 57 | 69 | 80 | 100 |  | 134 |  | 200 |

- Table 3

A third possibility that should be considered is when positioning is by GPS/DGPS and navigating is by conventional Aids to Navigation.  The values of the position accuracy of GPS and DGPS were provided by the USCG

Tables 4 and 5 have therefore been prepared, by combining the positional accuracies from Table 1 with the Navigational Accuracies from Tables 2 and 3, to assist discussions on this possibility.

| Positioning by GPS/DGPS and Navigation by buoyage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Width of waterway in metres | Recommended Navigation Accuracy in metres Min Max |  | Position fixing system | Position Accuracy in metres | Navigational accuracies in metres provided by the following facilities: |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Buoys – Percentage of Standard Number |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 100 | 90 | 80 | 70 | 60 | 50 | 40 | 30 | 20 | 10 |
| 100 | 10 | 10 | DGPS | 10 | 10 | 11 | 12 | 14 | 17 | 20 | 25 | 33 | 50 | 100 |
| 200 | 20 | 30 | DGPS | 10 | 20 | 22 | 25 | 27 | 33 | 40 | 50 | 67 | 100 | 200 |
| 300 | 30 | 60 | GPS | 22 | 30 | 33 | 37 | 43 | 50 | 60 | 75 | 100 | 150 | 300 |
| 400 | 40 | 100 | GPS | 22 | 40 | 44 | 62 | 57 | 69 | 80 | 100 | 134 | 200 | 400 |
| 500 | 50 | 150 | GPS | 22 | 50 | 55 | 55 | 71 | 83 | 100 | 125 | 167 | 250 | 500 |
| 600 | 60 | 210 | GPS | 22 | 60 | 67 | 75 | 86 | 100 | 120 | 150 | 200 | 300 | 600 |
| 700 | 70 | 280 | GPS | 22 | 70 | 78 | 87 | 100 | 117 | 140 | 175 | 234 | 350 | 700 |

- Table 4

| Positioning by GPS/DGPS and Navigation by Leading lights and buoyage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Width of waterway in metres | Recommended Navigation Accuracy in metres Min Max |  | Position fixing system | Position Accuracy in metres | Navigational accuracies in metres provided by the following facilities: |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Leading Lights and Buoys Buoys – Percentage of Standard Number |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 100 | 90 | 80 | 70 | 60 | 50 | 40 | 30 | 20 | 10 |
| 100 | 10 | 10 | DGPS | 10 | 9 | 10 | 11 | 12 | 14 | 17 | 20 | 25 | 33 | 50 |
| 200 | 20 | 30 | DGPS | 10 | 18 | 20 | 22 | 25 | 27 | 33 | 40 | 50 | 67 | 100 |
| 300 | 30 | 60 | GPS | 22 | 27 | 30 | 33 | 37 | 43 | 50 | 60 | 75 | 100 | 150 |
| 400 | 40 | 100 | GPS | 22 | 36 | 40 | 44 | 62 | 57 | 69 | 80 | 100 | 134 | 200 |

- Table 5