IALA Risk Management Tool

Lateral Distribution of Traffic

Assuming a waterway is divided into sections that are either straight or curved, the risk of vessels colliding in a section or going out of bounds is dependent upon a number of factors including:

Geographical factors such as channel length and average lane width;

Hydrographic factors such as direction and speed of current;

Meteorological factors such as visibility and direction and speed of wind;

Traffic factors including:

- The beams of ships;

- The speed of ships;

- The safe distances between passing ships;

- The distribution of shipping in the section, over a given time, in respect of both the average time separation of ships transiting in each direction and their lateral distribution related to the centre of the traffic lane they are using;

- The Standard Deviation (STD) calculated from the lateral distribution of each type of ship in each of the lanes of the section.

Many of these factors, including Geographical, Hydrographical, Meteorological and items 1, 2 and 3 of Traffic factors together with the average time separation of different types of ships transiting the section can normally be obtained from historical records of by simple calculations.  However, the lateral distribution of each type of ship is more difficult to determine unless there are reliable historical records of the lateral distribution and constant monitoring of the section that enables these records to be kept up to date.

Looking at the problem of the lateral distribution of ships a little more closely, it is apparent that a good knowledge of this is essential to enable a reasonably accurate assessment of risk to be calculated because by comparing the STD to the lane width it is possible to assess the number of ships that will probably go outside of the limits of the lane on either side.

It is therefore worth considering the extent to which aids to navigation influence the lateral distribution of ships in a waterway.   Aids to Navigation (AtoNs) are used to designate the outside edges of a lane or channel as a means of assisting ships to keep within it.  The effectiveness of this approach depends to a large extent on the type, function and location of the AtoNs that are provided.  An initial discussion on how the effectiveness of buoys varies depending on the width of the lane or channel and the distance between buoys is given in Annex 1.

The Gaussian distribution curve, from which STD is calculated, was derived from a study of natural events in all spheres of life.  It can therefore be expected that under normal circumstances some ships will always exceed the limits of any channel or lane.  However, it is also natural for mariners to avoid risk wherever possible, therefore it is probable that greater care is taken when navigating in a lane where exceeding the limits would result in grounding than when navigating in a lane where exceeding the limits would not present such a risk.

Since the introduction of traffic routeing schemes and deep water routes many channels and lanes have been marked primarily for these purposes with the possible result that mariners, particularly on smaller vessels, knowing that there is no risk in going outside the marked lane, may not be so diligent in keeping to the lane.  It is also relevant that although the introduction of traffic routeing schemes substantially reduced the total number of collisions, it actually increased, to a small amount, the number of overtaking collisions.

Detailed study of the actual distribution of different types of vessel in and adjacent to marked channels or lanes should give a clear indication of the lateral distribution and permit the STD to be calculated for each different type of vessel.

It might be possible to obtain the lateral distribution of different types of vessels from some historical results and also from results obtained by the use of AIS in the Oresund.  In addition to assessing the STD for each vessel type, study of such results may enable the following to be identified:

- Whether there is any difference in the lateral distribution of vessels of the same type when:

- AtoNs are used for marking a channel in which there is high risk of grounding if vessels go outside of the channel compared with marking a channel for traffic routeing or management purposes;

- The vessels domains are changed as a result of the effects of wind and/or current

In addition to assessing the risks of collisions and out of bounds in a waterway by using the lateral distribution, and taking into account whether the channel is marked because of a high risk of grounding or for traffic routeing of management purposes and the effects of wind and current, the information derived from such a study could also assist in assessing the most cost effectiveness arrangement of AtoNs for any given traffic density in channels of a given length and average width.

*     *     *

Annex 1

IALA Risk Management Tool

Risk reduction factors for buoys, leading lights and sector lights

The risk reduction values on a section of buoys, leading lights and sector lights in a channel depend substantially on the average channel width, the channel length, whether the section is straight or curved, the functions that these Aids to Navigation perform and the ease by which they can be used to assist mariners.

The risk reduction values need to be highest in those sections of a waterway which have the greatest risk of groundings or collisions, that is, as the channels become narrower, the greater should become the risk reduction values.

These Aids to Navigation can be considered to be provided for two different purposes;

- Buoys are provided basically to mark the channels in a waterway in which it is safe for ships to navigate and take into account geographical and other natural hazards.  There are four different arrangements of buoys commonly in use, they are: single centre buoy; single staggered buoys; unsynchronised paired buoys; and, synchronised paired buoys; while,

- Leading lights and sector lights are provided basically to indicate the safest route to sail in the channel and the adjustments in course necessary to attain that route.  However, it is unlikely that both leading lights and sector lights will be used together in the same section of a channel.

For the purpose of this assessment it is assumed that the combination of buoys, leading lights and sector lights normally used in channels of different average widths conform to Table 1:

| Channel Width (Metres) | Arrangement of buoys |  |  |  | Leading lights or Sector lights |
| --- | --- | --- | --- | --- | --- |
|  | Single |  | Paired |  |  |
|  | Centre | Staggered | Unsynchronised | Synchronised |  |
| 100 – 400 |  |  | YES | YES | YES |
| 400 - 800 | YES | YES | YES | YES |  |
| 800 - 1600 | YES | YES | YES |  |  |

Table 1.  Combination of buoys, leading lights and sector lights used in channels of different width

To calculate the risk reduction factors for buoys the following assumptions have been used:

- The optimum separation between paired buoys along a straight section is 1.7 times the average channel width;

- The optimum separation between paired buoys along a curved section is 1.9 times the average channel width;

- The optimum separation between single buoys along a straight section is 1.2 times the average channel width;

- The optimum separation between single buoys along a curved section is 1.3 times the average channel width;

- The standard number of paired buoys for a section of a channel is twice the length of the section divided by the optimum separation of each pair of buoys.

- The standard number of single buoys for a section of a channel is the length of the section divided by the optimum separation of each buoy.

The overall range of risk reduction values arising from the use of Buoys, leading lights and a sector light used in this report is from 0 to 30.  The range for buoys being from 0 to 24 and the remaining 6 being applied when leading lights or a sector light is provided in addition to buoys.  However, it is recognised that this range may need to be changed so that it is compatible with the IWRAP model.

Using the assumptions given above, the risk reduction values for unsynchronised paired buoys can be derived from Figure 1.

The risk reduction values for synchronised paired buoys can be determined from Figure 1 by multiplying the risk reduction value appropriate to the percentage of the standard buoy number by 1.2,

The risk reduction for staggered single buoys can be determined from Figure by multiplying the risk reduction value appropriate to the percentage of the standard buoy number by 0.9.

The risk reduction for centre single buoys can be determined from Figure 1 by multiplying the risk reduction value appropriate to the percentage of the standard buoy number by 0.8.

Applying Figure 1 and appropriate calculations the maximum range of risk reduction values for each combination of buoys and or leading lights/sector lights is given in the Table 2.

| Channel Width (Metres) | Arrangement of buoys |  |  |  | Leading lights or Sector lights | Range of risk reduction values |
| --- | --- | --- | --- | --- | --- | --- |
|  | Single |  | Paired |  |  |  |
|  | Centre | Staggered | Un-synchronised | Synchronised |  |  |
| 100 – 400 |  |  |  | YES | YES | 6 - 30 |
| 100 - 400 |  |  | YES |  | YES | 6 - 26 |
| 100 - 400 |  |  |  | YES |  | 0 - 24 |
| 100 - 400 |  |  | YES |  |  | 0 - 20 |
| 400 - 800 |  |  |  | YES |  | 0 - 24 |
| 400 – 800 |  |  | YES |  |  | 0 - 20 |
| 400 - 800 |  | YES |  |  |  | 0 - 18 |
| 400 - 800 | YES |  |  |  |  | 0 - 16 |
| 800 - 1600 |  |  | YES |  |  | 0 - 20 |
| 800 - 1600 |  | YES |  |  |  | 0 - 18 |
| 800 - 1600 | YES |  |  |  |  | 0 - 16 |

Table 2.  Range of risk reduction values for various combinations of buoys, leading lights and sector lights

Summary of calculations

To determine the combined risk reduction factor of buoys, leading lights and sector lights in a section of a channel the following steps should be taken:

1.	Determine the standard number of buoys for the section using the following formula:

For paired buoys,

Sn =        2(length of section)

x(average width of section)

where	Sn = standard number; and,

x  = 1.7 for paired buoys in a straight section of the channel; or,

1.9 for paired buoys in a curved section of the channel.

For single buoys,

Sn =        length of section

x(average width of section)

where	Sn = standard number; and,

x  = 1.2 for single buoys in a straight section of the channel; or

1.3 for single buoys in a curved section of the channel

2.	Count the number of buoys in the section and calculate the percentage of buoys compared with the standard number of buoys for the section relevant to the buoy arrangement.

3.	Determine the risk reduction value from the graph “Risk Reduction values for Buoyage”.

4.	To take into account the comparative ease of use of the various buoy arrangements by mariners, multiply the risk reduction value determined from the graph by the appropriate factor from the following:

Synchronised paired buoys	1.2

Unsynchronised paired buoys	1

Staggered single buoys		0.9

Centre single buoys		0.8

The value obtained is the resultant risk reduction factor for the section of the channel if leading lights or a sector light is not provided.

5.	Where leading lights or a sector light is provided in the channel, add 6 to the calculated risk reduction factor for the buoys.

The value obtained is the resultant risk reduction factor for the section of the channel if leading lights or a sector light is provided.

Notes

The figure of 1.7 times the channel width for the optimum separation of paired buoys along a straight section was determined by assessing the buoyage on the approach to Portsmouth Harbour.  The length of the section was 2300 yards, the average channel width was 200 yards and there were 7 pairs of buoys in the section.

The figure of 1.9 times the channel width for the optimum separation of paired buoys on a curved section was assessed by taking into account the need for mariners to see at least the next pair of buoys throughout the turn.

Figure 1 was based on unsynchronised buoys because this was considered to be the most common arrangement of buoys.  The graph conforms to the square law 2n2, where n is a value on the y axis from 0 to 10.  If it is required to change the overall scale from 0 to 20 to, say, 0 to 30, it is only necessary to change the integer 2 to 3.

The weighting of the various buoy arrangements is based on the completeness of the information provided to mariners and the ease with which it can be used to assist in the safe navigation of ships.