IALA Recommendation E-107

On

The Designs Of Normal Moorings

Edition 1.1

December 2005

Edition 1

May 1998

AISM Association  Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01   Fax + 33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      Web site: www.iala-aism.org

EEP13/8/1

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| December 2005 | Entire document | Reformatted to reflect IALA documentation Hierarchy |

Recommendation On The Design Of Normal Moorings

(Recommendation E-107, May, 1998)

THE COUNCIL:

RECOGNISING the need to develop guidelines on the design of buoy moorings under normal conditions for conventional aid to navigation buoys1, i.e. buoys with buoyant bodies whether carrying an aid to navigation light or not;

RECOGNISING ALSO that it is not possible to word the guidelines other than in general terms, since this is a field where local circumstances and empiricism are highly significant;

TAKING INTO CONSIDERATION the proposals of the IALA Engineering Committee;

ADOPTS the “Guidelines on the design of normal moorings”, described in the Annex to this Recommendation;

RECOMMENDS that National Members take into account the “Guidelines on the design of normal moorings” when considering new or replacement mooring arrangements for aids to navigation buoys located in waters of approximately 80 metres depth or less and whether carrying an aid to navigation light or not.

REVOKES IALA Recommendation on the design of normal moorings, dated June, 1975

Table of Contents

1	INTRODUCTION	5

2	GENERAL CONSIDERATION	5

3	CHAINS	5

4	MOORING LINE DESIGN	5

4.1	Selection of a mooring system	5

4.2	First method	6

4.3	Second simpler method	7

4.4	Subsequent Rules	9

4.5	Special consideration	10

5	DETAILS OF THE MOORING LINE	10

6	SINKERS	11

7	MAINTENANCE OF MOORINGS	12

8	SPECIAL DISPOSITIONS FOR BUOYAGE IN ICE CONDITIONS	13

ANNEX

GUIDELINES ON THE DESIGN OF NORMAL MOORINGS

(IALA Recommendation E-107, May, 1998)

# INTRODUCTION

These Guidelines are given in general terms because this is a field where local circumstances and empiricism are highly significant. However, they have been proved to work satisfactorily during long practice.

# GENERAL CONSIDERATION

The purpose of a mooring system is to provide a flexible link between the buoy and a fixed point on the bottom of the sea.

The flexible link permits the damping of the forces imposed by the waves on the buoy.

# CHAINS

Buoy moorings usually consist of open link type chain, the bar diameter ranging from 13 to 47.5 mm, the weight being 3.5 to 43.7 kg/m respectively.

The chain material may be mild steel alloy and where excessive wear is encountered, a medium carbon steel can be used, but this is more expensive.

A stud link chain is not normally used for buoy moorings, due to its initial high capital cost.

# MOORING LINE DESIGN

## Selection of a mooring system

The selection of a mooring system for a buoy should, whenever possible, be governed by reference to identical buoys which have been found operating satisfactorily under similar sea conditions.

In the absence of such a reference, it will be necessary to resort to calculation to obtain the chain bar diameter, the weight per unit length, the overall length of the chain mooring and the position of the attachment of the mooring to the buoy body. Several methods are available for calculating the above, which assume various degrees of simplification.

## First method

The first method requires the use of a computer and enables a mooring system to be evolved to meet the following conditions:

Assuming a given buoy to be moored at a given site, the length and diameter of the chain and the position of the chain connecting link on the buoy body are determined in order to meet the following requirements:

- The chain is tangential to the sea bottom under all conditions of current and wind at the site,

- The buoy axis is vertical under the most common conditions of current and wind,

- The ratio of the breaking stress of the chain to the calculated stress is not less than 5 under the most unfavourable conditions of current and wind.

- The reserve buoyancy of the fully equipped buoy is sufficient under the most unfavourable conditions of current and wind.

This method of calculation takes into account:

- the drag of the wind on the buoy superstructure,

- the drag of the current on the buoy,

- the drag of the current on the chain.

Equations given in footnotes   and  can be used to calculate each of these three actions.

Putting this method into practice has shown that the drag on the chain due to the current could be neglected in a first approximation for mooring depths less than 40 m

However there is obviously a limit to the depth of water under which the requirement (d) (sufficient reserve buoyancy) cannot be met with a certain type of buoy, a given chain diameter and given current and wind conditions. In such a case it will be necessary to select an alternative buoy or to resort to a composite mooring to give the required reserve buoyancy.

The programme using the above described method of calculation is available at the IALA Secretariat.

## Second simpler method

A second simpler method of calculation avoiding the assistance of a computer consists in considering the shape of the part of the chain that is raised above the bottom of the sea as a catenary. This method enables the following problem to be solved:

Assuming a buoy to be moored at a given site, the length and the diameter of the chain are determined in order to meet the following requirements:

- The chain is tangential to the sea bottom under all conditions of current and wind at the given site,

- The ratio of the breaking stress of the chain to the calculated stress is not less than 5 under the most unfavourable conditions of current and wind,

- The reserve buoyancy of the buoy is sufficient under the most unfavourable conditions of current and wind.

This method of calculation takes into account:

- the drag of the wind on the buoy superstructure.

- the drag of the current on the buoy.

The method does not ensure that the buoy will remain truly upright under the most common conditions of current and wind. The effect of the current drag on the chain is neglected.

The method takes the following parameters into account:

- Characteristics of the buoy

P: Weight					(N)

U: Volume					(m3)

Br: Minimum Reserve Buoyancy		(m3)

S: Weight area (mid-section)		(m2)

- Characteristics of the site

H: Maximum depth of water, it is the highest tide level plus half the maximum wave height at that particular station (m)

Vw: Wind velocity assuming it is horizontal and uniform on the whole height of the buoy Superstructure (m/s)

V: Current velocity assuming it is horizontal and uniform on the whole water depth at the site (m/s)

These parameters make it possible to determine the resultant F (N) of the horizontal forces caused on the buoy by the current and wind under the most unfavourable condition. These forces are to be derived using the formulae given above in footnotes 4 and 5.

The chain is characterized by the following three parameters:

p:  Weight in water by length unit	(N/ml)

Tr: Breaking stress			(N)

L: Length				(m)

The above requirements i), ii) and iii) are complied with when the parameters meet with the following expressions:

where g is the acceleration of gravity at the given site. Figure 1 shows the notations used in the present paragraph.

## Subsequent Rules

Putting the above described methods into practice shows in general that the following approximate rules.

Fig1 – Schematic diagram illustrating the simplified method meeting requirement i) stated under headings 3.1 and 3.2:

- the length of the chain is three times the water depth at highest sea level,

- in sites where the buoy is submitted to both waves and current, it is recommended to increase the length of the chain. This increase will be selected between 0 and three times the water depth when the current varies between 0 and 6 knots.

## Special consideration

The maximum swinging radius (watch circle radius) of the buoy is:

It may be necessary to reduce the length of tile mooring chain in order to reduce the swinging radius (watch circle radius). In this case it is not recommended to reduce the length of the chain to less than two times the depth of the water, where the latter is less than 50 m. For more than 50 m water depths, a ratio of 1.5 can be accepted.

# DETAILS OF THE MOORING LINE

To facilitate inspection and also for reasons of economy a mooring line may consist of several parts suitably assembled.

Figure 2 shows a typical mooring line arrangement. The mooring line comprises four sections of chains which are from the top to the bottom:

- the tail chain,

- the riding chain,

- the thrash chain,

- the ground moor chain which may be made of used chain.

Mooring lines can be arranged according to depth and local conditions.

It is recommended to connect the tail chain and the riding chain together using a swivel and two shackles.

The swivel will permit the free rotation of the chains around their axis and will prevent kinks in the mooring line.

Fig 2 - Typical buoy mooring

For depths over 40 m it is recommended to arrange the connection between the riding chain and the thrash chain in the same manner.

The tail chain is connected to the buoy and the chain to the sinker by means of shackles of the appropriate shape.

# SINKERS

Sinkers may be made of either cast iron or concrete or of stones for small buoys. Concrete sinkers are cheaper than cast iron ones. However cast iron sinkers are easier to handle than concrete ones of equivalent buoyant weight. Large stones may also be used for the mooring of small or medium size buoys.

The sinker must not capsize under a horizontal pull. When in concrete, it is recommended to arrange a cavity of small height in the lower face which helps the sinker to adhere the bottom through suction on a sandy or muddy soil.

It is recommended that the concrete mix contain 400 kg of cement per cubic meter of concrete in situ.

Figures 3 and 4 show recommended shapes of sinkers.

It is recommended that the dimensions of the branches of the anchor ring (Fig. 5) and those of the cavity in which it is housed provide adequate clearance for placing the lifting hook and the sling necessary to handle the sinker. The clearance is also necessary to position and key the connection shackle of the ground mooring chain. When a buoy is stationed where currents are very strong and the water depth small, it is sometimes necessary to lay two sinkers of unequal weight linked by a chain of 15 to 20 m, the lighter sinker being at the end of the mooring ling (Fig. 6)

It is not recommended to use marine anchors as sinkers because their projections might foul the chains.

An emergency sinker may be made out of a heap of used chains of adequate weight. This type of sinker behaves particularly well on rocky bottom.

The arrangement where the mooring chain is connected by a bridle to two sinkers of equal weight is to be used with caution because the risk of kinks is very high.

# MAINTENANCE OF MOORINGS

It is recommended to comply with the following directions:

The maintenance of moorings in situ requires periodical visits the frequency of which depends on the risk of wear. Generally moorings are inspected once a year or more according to local conditions.

During each visit the following operations should be performed:

- Clear the tail and ride chains from the shells and algae of all species which could help the formation of kinks.

- Check the wear of the axis of the shackles and check the behaviour of the tapered pins and lead pellets of anchor-cable shackles; shackles with a loose key must be changed.

- Check the free movement of each swivel around its head; a sticking swivel must be freed or replaced.

- Check every link of the thrash length of the chain; the diameter of used links is measured every meter on the nips and sides; the weld of the links must also be checked.

- If the depth allows, a worn riding chain may be reversed.

- Change a chain when any link shows excessive wear. Generally this occurs when the link is reduced to about 3/5 of the initial diameter.

The ground chain and sinker, which are not exposed to wear, will be examined less frequently (e.g. 2 or 3 years).

# SPECIAL DISPOSITIONS FOR BUOYAGE IN ICE CONDITIONS

Biconical buoys are more suitable for use in ice conditions. They withstand pressures exerted by ice quite well. It may therefore be advisable to substitute biconical buoys for buoys of other shapes before ice formation.

Normal conditions for conventional aid to navigation buoys” means “depths less that 80m approximately

See report 3.2.2 to the 7th International Conference on Lighthouses and other Aids to Navigation – Rome, 1965: “Mooring chains for buoys” by A. K. Laing, R. K. Buhr, S.L Gertsman (Canada).

The importance of this ratio is principally justified by the fact that the calculation does not take into account impacts between chain links.  Higher ratios are often taken into account to allow for wear.

- The force Fw (N) imposed by the wind at height h over a small portion dh of the buoy superstructure is supposed to be horizontal. It is to be derived from the formula:

dfw = ½ ρa V² w l Cw dh

where

ρa  : Unit mass of air (kg/m³)

Vw : Wind velocity at height h (m/s)

L  : Width of superstructure at height h (m)

Cw : aerodynamic drag coefficient of the relevant part.

Values for Cw are as follows:

Lattice construction (angles)		1.7 < Cw < 2

Lattice construction (tubes)		0.3 < Cw < 0.4

Cylinder					0.3 < Cw < 0.4

Plate (when it is normal)			Cw < = 1

The hydrodynamic drag Fd (N) imposed by the current on the buoy is given by the formula:

Fd = ½ Cd ρo SV²

Where;

Cd : Hydrodynamic drag coefficient

The value of Cd lies between 0.8 and 1.2 for buoys with a draught greater than or equal to 1/5th of their diameter. If the draught is smaller than 1/5th of the diameter, Cd lies between 0.3 and 0.5

ρo : Unit mass of sea water   (kg/m³)

S  : Mid body area of buoy  (m²)

V  : Current velocity       (m/s)

- The drag dFe of small portion of chain with length ds resulting from a current with velocity V running at angleθwith ds, is the resultant of two components:

dFt = ½ ρo V² CtdS tangential component

dFn = ½ ρo V² CndS normal component

dS = v ds, v is the width of a link (10/3 of the chain diameter)

Ct = (0.069 + 0.124 sinθ) cosθ

tangential drag coefficient

Cn= (0.049 + 1.273 sinθ– 0.637 sin ²θ) sinθ

normal drag coefficient

- Underestimation of chain lengths and tensions is less than 10 %.

The case of composite mooring lines is not treated in this recommendation.

The angleθbetween the tension at the top of the mooring line and the horizontal force F is given by the formula: