Draft for discussion – Michael Card, Monday, 11 October 2004

Modified by A.Wilkins 17 October 2004

Notes on Making a Comparison of Competing Buoy Technologies

Contents

1.	Background	1

2.	Examples of different buoy technologies	1

3.	Primary points for consideration	1

4.	Financial analysis and the NPV method	2

5.	Notes on costs for use in NPV analysis	2

6.	Examples	3

Example 1 – AMSA, Australia	4

Example 2 – CETMEF, France	5

Example 3 – JCG, Japan	5

Appendix 1 - Advantages and Disadvantages of the use of large plastic buoys (1.8 to 3.0m diameter)	5

### Background

Aids to navigation services are faced with a choice of buoy technologies. These technologies employ various materials and manufacturing methods. A change of buoy technology might allow a service to reduce its operating costs. However the decision to change from the existing buoy technology to a different technology must be made with care. This paper sets out some points which should assist the decision making, and recommends the use of Net Present Value [NPV] analysis for financial evaluation.

### Examples of different buoy technologies

Various materials are used for the manufacture of floating AtoN buoys. These include

- Steel

- Aluminium

- Plastics

These materials may be used alone. They can also be used in combination to manufacture buoys which combine the benefits of different materials. For example:

- Steel buoy hull with aluminium tower

- Plastic buoy float with steel centre tube and aluminium tower

- All plastic hull and tower, with internal metal structural members

- Aluminium buoy with plastic foam filling within the hull

- GRP buoy hull with steel or aluminium tower

These are simply a few of the possible choices. The materials choices are influenced by the size of buoy required, and by the environmental conditions under which it must operate. Changing to a different technology may offer the AtoN operating authority a reduction in overall costs, via reduction in servicing costs or reduction in first costs.

This subject should also be extended to compare conventional buoys with fixed beacons (usually single pile) or spar buoys which are moored at the seabed with a universal joint or short chain. [These are know by various names, including “spar buoys”, “resilient beacons”, and “elastic beacons”.] This type of buoy can offer potential cost savings by reducing the frequency of mooring inspection and replacement (if local tidal regime and water depth allow the use of these alternatives). Refer Example 3 below.

### Primary points for consideration

If an Authority is to consider using a different buoy technology then the following questions should be addressed:

- AtoN signal – it is important to compare equal with equal

  - Is the new buoy big enough as a day-mark?

  - Is it big enough to carry a lantern with the required range, and other required equipment, and the associated power system?

  - Is the focal high sufficient for local sea conditions?

  - Can a suitable radar reflector be mounted?

  - In summary, is the new buoy equal to the old?

    - If not there are two different questions to be answered.

    - Can the service to the user be reduced?

    - What is the best means of providing this new signal?

- Serviceability

  - Is it possible to connect a suitable mooring to the buoy?

  - Is the motion of the buoy on station to rapid or of to great angular displacement for the AtoN function to be clear to the mariner [e.g. vertical divergence]

  - Is the buoy’s motion acceptable for maintenance personnel to work on the buoy?

- Local conditions

  - If there are ice conditions will the buoy be suitable?

  - If self coloured materials (usually plastic) are used how long will the coloured surface last in the local UV environment.

- Service intervals

  - At what interval must the day-mark components be replaced? In order to retain correct signal colours.

  - What are the inspection and maintenance intervals of [plastic] parts, mooring system, energy system and lantern?

  - How often must the new  and existing buoys be visited to maintain the lantern, power system, mooring etc.? You must be able to identify the critical items that will wear out and define the maintenance periods for the new buoy.

- Purchase costs

  - What are the costs of the purchase of the existing buoys?

  - What are the costs of the purchase of the new buoys?

- Maintenance costs

  - What time and resources are required to maintain the existing buoy – and how much does it cost?

  - What time and resources are required to maintain the new buoy – and how much does it cost?

  - Will there be any saving in vessel costs from using a lighter buoy?

  - Will there be a saving in vessel time at each servicing visit?

  - Will the number of servicing visits be reduced?

  - Will the same vessels/facilities be used for the new buoys? {I.e. no cost saving for these cost components.]

  - Can a different mooring type be used?

  - Will the new buoy mean a reduction in mooring components costs?

  - NOTE full cost advantage may only be possible if the entire buoy fleet can be changed from the old to the new possibly allowing painting facilities to be closed down and a smaller servicing vessel to be used.

### Financial analysis and the NPV method

Financial analysis for the comparison of different buoy strategies should use the NPV method.

Net Present Value [NPV] analysis is an approach used in capital budgeting where the present value of cash inflow is subtracted from the present value of cash outflows. It is accepted internationally as a suitable and popular method for making financial analysis to assist decision making, and often used for commercial decision making, where there are costs and income streams, and where the decision must be made whether to go ahead with a project.

For most buoy decision analysis there will be the comparison of costs, but income could arise from sales of redundant items, or from marketing ‘spare’ ship time.

Explanation of  NPV theory is not given here. The reader is referred to the Internet or to textbooks, which can be found on the topic in most languages.

Example 1 below is a good example of the use of NPV analysis for making a decision on the best buoy technology to employ.

### Notes on costs for use in NPV analysis

In making an NPV analysis for the comparison of buoy system options, the following points should be borne in mind.

- Previously-spent sums [sunk costs]

  - These must not be included – this is a common accounting convention

- Existing service vessels, facilities, staff and crews

  - How to treat

  - Freeing facilities for other duties

  - No saving in vessels costs if

  - Reductions in operating costs

- Do not treat cost savings as income

  - Compare costs

  - If necessary, make two NPV tables, one for the existing situation, and one for the new proposal

- Disposal costs and environmental costs

  - For disposing of certain scrap, there may be disposal charges. Include these in the costs.

  - Environmental regulations may require complete re cycling of new buoys. This may influence choice of new product.

- Disposal income from scrap

  - In many cases, the sale of scrap, e.g. steel will be the only income in the NPV study.

- Repair systems, training of staff, special tools facilities.

  - Take account of any costs for special facilities relating to disposal of materials [e.g. solvents, scrap plastic], and the costs of any staff training or work clothing.

- Use of contractors

  - e.g. steel can be repaired “anywhere”. Structural steel specialists are widely available

  - Aluminium repair may be difficult away from industrial centres.

  - High specification paint systems for steel and aluminium require sophisticated surface preparation and application facilities .

  - Repairs to some plastics may require the contractor to have special tools and skills.

  - How will this affect the total costs?

- Occupational safety and health

  - Are there any special costs?

  - Do staff need special safety and health training? What will this cost?

- Interest rate

  - Setting an interest rate is not an easy decision.

  - It should generally be greater than zero.

  - Discuss with your accounting department if advice is needed

- Initial costs

  - Existing buoys, with remaining life should be treated as having zero cost.

  - But consider in what year they must be replaced, repaired, etc.?

- Income from selling ship time

  - This must be treated carefully.

  - How secure is this income stream?

  - Will it benefit the authority, or would such income accrue elsewhere?

  - What are the accounting rules in force in the service and do these affect the NPV analysis?

  - In general such income should be treated with caution, and conservative figures, or zero, used in the income cells in the NPV spreadsheet.

### Examples

Examples of application of the above methodology follow. They are

Example 1 - AMSA, Australia, analysis and decision re steel buoys MORE DETAILS of how spares ratio, maintenance periods etc. were established would be very useful.

- Example 2 - CETMEF, France, change to composite buoys using plastic, steel, and aluminium, with a different tender fleet

- Example 3 - JCG, Japan, change from conventional  buoys to spar buoys

- DETAILS OF THE USCG USE	OF’DISPOSABLE’ FOAM	BUOYS TO REPLACE CONVENTIONAL SMALL STEEL BUOYS COULD BE USEFUL.

#### Example 1 – AMSA, Australia

AMSA has recently performed a cost benefit analysis of options for replacement buoys in Australia taking into consideration a number of presently available plastic alternatives to their current steel buoys. The analysis resulted in a decision to continue with steel buoys for the next five years.

Whilst the decision to stick with steel buoys is very interesting in itself, the methodology used to arrive at this decision is even more interesting for other AtoN authorities.

The approach used for evaluating the capital cost savings was as follows:

- Select which scenarios to analyze

- Define any basic assumptions made for the analysis

- Select a timeframe for the analysis

- Calculate the initial replacement costs

- Calculate the maintenance costs each year

- Calculate the Net Present Value (NPV) of all costs within the selected timeframe

- Compare the NPV of each scenario

The following scenarios where selected for the AMSA analysis:

- Continue to use the current design of buoys of steel construction

- Change to plastic construction buoys which have reduced maintenance requirements.

The following basic assumptions where made for the analysis:

- Costs common to all scenarios can be omitted

- Maintenance cycle of steel hull buoys requires purchase of 1.5 times the number in service

- Maintenance cycle of plastic hull buoys requires purchase of 1.25 times the number in service

A time frame of 20 years was selected.

The following factors where taken into consideration for calculating the initial costs:

- Unit Cost

- Maintenance Spares (proportion in store)

- Installation components/fittings

- Cost of ship time for installation

While calculating the maintenance costs each year, the following factors where considered:

- Minor maintenance costs at interval of X years

- Major maintenance costs at interval of Y years

- Buoy replacement costs at interval of Z years (buoy lifetime)

In each case the costs where split into the cost of ship time and other costs (materials and services).

Comparing the NPV of each scenario revealed what potential capital cost savings could be obtained for each scenario relative to the present scenario.

A spreadsheet example of this basic NPV methodology is found at the end of this example.

The above mentioned factors can more or less be characterized as direct cost drivers. In addition to these there may be a number of indirect cost drivers such as workshop facilities that may be affected the alternative scenarios.

Thanks to Allan Crossing and Gary Prosser from AMSA for providing this information.

[Edited by Ómar Frits Eriksson]

AMSA example continued

| Item | Current Steel Buoy | Buoy A | Buoy B | Buoy C | Buoy D |
| --- | --- | --- | --- | --- | --- |
| Initial Costs |  |  |  |  |  |
| Unit Cost | 9600 | 9700 | 14000 | 25000 | 55000 |
| Maintenance Spares (proportion in store) | 0,5 | 0,25 | 0,25 | 0 | 0 |
| Installation components/fittings | 6300 | 6300 | 6300 | 1800 | 1800 |
| Ship time for installation (days) | 0,5 | 0,5 | 0,5 | 2 | 2 |
| Installation costs | 2550 | 2550 | 2550 | 10200 | 10200 |
| Daily Ship Cost | 5100 |  |  |  |  |
| Maintenance Costs |  |  |  |  |  |
| Minor maintenance costs at interval of:- (years) | 2 | 2 | 2 | 2 | 2 |
| Ship Time (days) | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 |
| Ship Cost | 2550 | 2550 | 2550 | 2550 | 2550 |
| Other Costs (materials & services) | 2700 | 2700 | 2700 | 1500 | 1500 |
| Major maintenance costs at interval of:- (years) | 4 | 4 | 4 | 6 | 6 |
| Ship Time (days) | 0,5 | 0,5 | 0,5 | 1 | 1 |
| Ship Cost | 2550 | 2550 | 2550 | 5100 | 5100 |
| Other Costs (materials & services) | 3000 | 2400 | 2400 | 3000 | 3000 |
| Buoy Changeout costs at interval of:- (years) |  |  |  | 10 | 10 |
| Ship Time (days) |  |  |  | 2 | 2 |
| Ship Cost |  |  |  | 10200 | 10200 |
| Other Costs (materials & services) |  |  |  | 6000 | 9000 |
| Year | Current | Buoy A | Buoy B | Buoy C | Buoy D |
| 0 | -23250 | -20975 | -26350 | -37000 | -67000 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | -5250 | -5250 | -5250 | -4050 | -4050 |
| 3 | 0 | 0 | 0 | 0 | 0 |
| 4 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 5 | 0 | 0 | 0 | 0 | 0 |
| 6 | -5250 | -5250 | -5250 | -8100 | -8100 |
| 7 | 0 | 0 | 0 | 0 | 0 |
| 8 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 9 | 0 | 0 | 0 | 0 | 0 |
| 10 | -5250 | -5250 | -5250 | -16200 | -19200 |
| 11 | 0 | 0 | 0 | 0 | 0 |
| 12 | -5550 | -4950 | -4950 | -8100 | -8100 |
| 13 | 0 | 0 | 0 | 0 | 0 |
| 14 | -5250 | -5250 | -5250 | -4050 | -4050 |
| 15 | 0 | 0 | 0 | 0 | 0 |
| 16 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 17 | 0 | 0 | 0 | 0 | 0 |
| 18 | -5250 | -5250 | -5250 | -8100 | -8100 |
| 19 | 0 | 0 | 0 | 0 | 0 |
| 20 | -5550 | -4950 | -4950 | -16200 | -19200 |
| Total | -77250 | -71975 | -77350 | -113950 | -149950 |
| NPV at |  |  |  |  |  |
| 5% | -56.033 | -52.023 | -57.398 | -80.619 | -113.591 |
| 10% | -45.084 | -41.709 | -47.084 | -64.214 | -95.817 |

(The figures shown are not the real figures obtained by AMSA)

#### Example 2 – CETMEF, France

reference IALA Sydney conference – can CETMEF make a simple summary of the financial analysis?

#### Example 3 – JCG, Japan

Can we get some notes?

### Appendix 1 - Advantages and Disadvantages of the use of large plastic buoys (1.8 to 3.0m diameter)

Advantages:

- The plastic does not corrode.

- Less maintenance on station; only removal of marine growth, no painting on station.

- Less maintenance on shore in the workshops for the plastic component (no grid blasting, no paint system of 3 layers or more) therefore fewer personnel and smaller workshops, lower personnel and equipment costs.

- Lower weight (1/2 to 1/3 of the weight of steel buoys in the diameter range 1.8 – 3.0 m) therefore the service can use smaller buoy tender.

- CETMEF and Rijkswaterstaat have more than 10 years experience of operating rotationally moulded polyethylene (PE) plastic buoys in the 1.8 – 3.0 m diameter size range.

- At least 6 commercial companies offer large rotationally moulded plastic buoys.

- Linear PE is recyclable (if not foam filled with polyurethane).

- It is possible to hot-flame-spray PE if the surface colour needs to be changed, but this is not necessary in many cases.

- Large plastic buoys are typically of modular construction. It is therefore possible to change individual body segments if they are damaged.

- Easier to transport and store the sections of the body when compared with conventional single component bodies.

- It is possible to install a radar reflector within a plastic buoy’s superstructure.

- The superstructure can be rotationally moulded.

Disadvantages:

- A large plastic buoy will usually contain metal components such as the tail tube, mooring-eyes, lifting-eyes, connecting bolts, lantern-mounting, etc. These metal items will still need maintenance.

- Less weight will result in more rapid motion on station when compared with the heavier steel buoy.

- Repairs are not simple or may often not be possible: it is more difficult to weld PE than steel. It is very difficult to monitor the weld quality.

- There is little experience in ice conditions or in very cold waters although Sweden has some good experience with PE spar-buoys in ice conditions.

- It is more difficult to change the colour of a plastic buoy, as conventional painting is not reliable. This may increase the number of spares needed, compared with steel buoys, i.e. red, green, black and yellow spares may have to be carried rather than re-painting steel components.

- Shorter lifetime than steel buoys –PE lifetimes are estimated to be better than 10 years compared with proven steel lifetimes often considerably in excess of 30 years.

- Less resistance to collision damage than steel.

- Plastic buoy components will be specific to each manufacturer compared to steel components that can be manufactured by any competent steel fabricator.