Action on C Baker from WG4

Trinity House Lighthouse Service – Solar Lightvessel Main Light Exhibition Lux Level Test

Location: 50 45.6 N; 001 17.46 W

This data was collected to confirm the photocell operation on a recently solarised light vessel.  There was some background lighting from nearby floodlights that automatically came on at around 200 to 150 lux, this was noticeable on the lux meter but did not appear to affect the test.  The formation of a red sky noticeably slowed the fall in lux level.  The fall from 100 to 50 lux took 4min 10s.

| lux | UTC |
| --- | --- |
| 900 | 19:42:40 |
| 800 | 19:45:10 |
| 700 | 19:47:20 |
| 600 | 19:48:40 |
| 500 | 19:52:20 |
| 400 | 19:56:10 |
| 300 | 19:59:55 |
| 200 | 20:05:40 |
| 150 | 20:09:20 |
| 130 | 20:10:40 |
| 100 | 20:12:00 |
| 87 | 20:13:00 |
| 50 | 20:14:10 |