Product Template 5a – Racon

# Purpose

This product template identifies the parameters that should be defined and the standards against which each parameter should be tested to enable a realistic comparison to be made between competing products.

# Scope

The template is applicable for racons used as aids to navigation.

## Product Certification Template 5a – Racon

| Ref. No | Parameter category | Measurement | Test Method | Comments |
| --- | --- | --- | --- | --- |
| 1 | Radio signal performance |  |  |  |
| 1.1 |  | Operating frequency band or bands |  | Manufacturer to state Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.2 |  | Receiver out-of-band signal rejection |  |  |
| 1.3 |  | Side-lobe suppression technique |  | Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.4 |  | False triggering protection |  |  |
| 1.5 |  | Antenna polarisation per band |  | Manufacturer to state Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.6 |  | Antenna azimuth response per band |  |  |
| 1.7 |  | Antenna vertical response per band |  |  |
| 1.8 |  | Antenna gain per band |  |  |
| 1.9 |  | Output power and band(s) |  | Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.10 |  | Response code |  | Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.12 |  | Response delay |  | Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.13 |  | Response proportional scaling |  |  |
| 1.14 |  | Frequency accuracy of transmitted response |  | Also refer to IALA Recommendation R-101 On Marine Radar Beacons (racons) Edition 2 Dec. 2004 |
| 1.15 |  | Programming |  | Manufacturer to state |
| 2. | Electrical |  |  |  |
| 2.1 |  | Power supply normal and extreme voltage | IEC 600945 section 7 | State voltage tolerance |
| 2.2 |  | Reverse polarity circuit protection | IEC 600945 section 7.2 |  |
| 2.3 |  | Power consumption - quiescent |  |  |
| 2.4 |  | Power consumption - at 33% duty cycle with 10 radars present |  |  |
| 2.5 |  | Power consumption - at33% duty cycle with 100% interrogation |  |  |
| 2.6 |  | Power supply terminations/connectors |  |  |
| 2.7 |  | Monitoring communications interface |  | Define connector and/or terminal type |
| 2.8 |  | Monitoring data output and format | No test method | Manufacturer to state |
| 2.9 |  | Monitoring connector type | No test method | Manufacturer to state |
| 3. | Physical |  |  |  |
| 3.1 |  | Maximum Height | No test method | Manufacturer to state |
| 3.2 |  | Maximum Diameter | No test method | Manufacturer to state |
| 3.3 |  | Maximum Weight | No test method | Manufacturer to state |
| 3.4 |  | Method of fixing | No test method | Manufacturer to state |
| 3.5 |  | Location of antenna(s) - Distance from reference level | No test method | Manufacturer to state |
| 3.6 |  | External package materials | No test method | Manufacturer to state |
| 3.7 |  | Description of fixing |  |  |
| 4. | Environmental |  |  |  |
| 4.1 |  | Temperature – operational and storage |  |  |
| 4.2 |  | Humidity – operational and storage | IEC 600945 section 8.3; IEC 60068-2-3 |  |
| 4.3 |  | IP Rating |  |  |
| 4.4 |  | Immersion | IEC 600945 section 8.9 |  |
| 4.5 |  | Salt air and sea water spray | IEC 600945 section 8.12 |  |
| 4.6 |  | Special requirements - Inert gas charging - Other | No test method | Manufacturer to state |
| 4.7 |  | Ultraviolet exposure |  |  |
| 4.8 |  | Shock and vibration | IEC 600945 section 8.7 Or Vibration MIL-STD-810F method 514.5 category 10 And shock MIL-STD-202G method 213B |  |
| 4.9 |  | Electromagnetic interference | IEC 600945 sections 9 & 10 Or USA MIL-STD-461E requirement CS103 and requirement RE102 |  |
| 4.10 |  | Hazardous Area rating |  | Manufacturer to state and supply certificate |
| 5. | Service |  |  |  |
| 5.1 |  | Maintenance free period |  |  |
| 5.2 |  | Rework at stated intervals |  |  |
| 5.3 |  | Assured service life of all components |  |  |
| 6. | Disposal |  |  |  |
| 6.1 |  | Obligatory and recommended disposal methode |  |  |