| From: EEP-Committee | EEP7/output/WP15 |
| --- | --- |
| To: AIS | Date: 05. October 2005 |

Liaison Note

Items for consideration in revision of IALA Recommendation A-126 (AIS as an Aid to Navigation)

- Introduction

The IEC Working Group 14 (WG 14) identified a number of items that would be appropriate for revision in IALA Recommendation A-126.  This liaison note was sent to the AIS Committee (AIS17) and the AIS Committee identified specific items for further discussion by the EEP Committee.

- Action Requested

The EEP Committee is requested to consider the two questions below, and respond to IEC WG 14.

    - Isolation requirements

It was noted that the document IALA Recommendation A 126 requires that an AtoN AIS unit must meet isolation requirements according to IEC 60950 to avoid leakage currents between AtoN batteries and connections.   In addition, the isolation is not only on the power supply aspect, but also for the RF elements.  Discussion identified that this may not be required, as isolated supplies tend to reduce efficiency.  The option to leave this to the competent authority was discussed, and that this point could be moved to A-126 rather than include in the AtoN IEC Standard.  In particular, the difference in requirements between fixed and floating AtoNs.  From EEP-Committees point of view there is no difference between small fixed and floating aids.

Is there a requirement for isolation testing in the IEC AIS AtoN standard?

The EEP-Committee doesn’t see any need for galvanic isolation between the AtoN equipment and the AIS equipment assuming a single common power supply. Therefore there is no need for isolation testing.

    - Testing conditions

- For testing of AtoN units, is it required to test to conditions beyond IEC 60945 for more extreme conditions (i.e. for vibration and shock)? Yes, particularly in terms of acceleration and vibration tests. Figures of 10G have been used by national authorities. In northern countries (Finnish Maritime Administration) accelerations up to 22G have been measured in case of ice movement at fixed structures.