Report of the IALA RNAV Committee

Inter-sessional WG on DGNSS

Introduction

At the final meeting of the Radionavigation Committee for the period 2002-2006, it was proposed that an inter-sessional meeting of the Working Group on DGNSS should be arranged to prepare a draft Recommendation on the Future of DGNSS. This proposal was approved by the Council in December 2005 and the meeting took place from 14-16 March 2006 at Trinity House in London. 17 members attended (Annex 1). In addition to the draft Recommendation, a template was prepared for submission of DGNSS services to IMO, for recognition as components of the WWRNS and a plan was formulated for achieving mandatory carriage of DGNSS receivers on SOLAS vessels. These three documents are attached as Annexes 2, 3 and 4.

# Presentations

Presentations were made during the first day of the meeting by the RNAV Chairman, the US Coast Guard, the German Waterways Administration and the Norwegian Coast Directorate in conjunction with Kongsberg-Seatex. The Chairman’s presentation explained the background to the meeting, set out the requirements for a future DGNSS and the options for implementation. Several questions were posed for the group to consider, in particular which signals should be augmented, whether the corrections should be broken down into their component error sources and the timeline to be followed. The USCG presentation reported progress with the US re-capitalization project, described the software RSIM and ethernet connection approach and mentioned some ongoing areas of R & D activity. The German Waterways presentation described the current status of the coastal DGPS stations, including the new MF antennas installed and then reported on the inland waterways initiatives.

The Norwegian Coast Directorate reported on the current status of their DGPS service and their plans for a new monitoring system. Kongsberg-Seatex described their approach to augmentation, which separates the error components to improve efficiency of datalink use.

# Draft Recommendation

A working document was circulated before the meeting, based on the output from the RNAV Committee. This was revised by the group, reflecting points that emerged from discussion, in particular an explanation of the continued need for augmentation, the fact that the requirement for modernisation was pressing in some countries, but not in those where equipment had only recently been installed and that the user requirement for enhanced services would need to be defined. It was also agreed that the submission of DGNSS to IMO as a component of the WWRNS and the mandatory carriage of DGNSS receivers were essential elements of the strategy for the Future of DGNSS.

The draft Recommendation (Annex 2) will go forward to the first meeting of the new e-NAV Committee in September 2006, but may also be presented at the DGNSS session during the IALA Conference in Shanghai.

# Template for WWRNS Submission

The document provided by the USCG on their submission was used in conjunction with Resolution A.953(23) to prepare a template for future submissions (Annex 3).

The following principles were agreed:

1. Submissions should be made by individual member states.

2. IALA’s role would be one of co-ordination.

3. Submissions should all follow the same format.

4. The first batch of submissions should all go into the same IMO meeting*

* This could be MSC 83, May 2007, if IALA Council  approves the proposed process in Dec 2006. MSC would be expected to pass the submission to NAV53, July 2007, for consideration. The GPS (and GLONASS?) submissions were referred to IALA, by correspondence, for assessment.

# Plan for Mandatory Carriage

In many parts of the world almost all mariners are using GNSS as a primary means of navigation. Currently available GNSS does not fulfil the requirements of IMO Resolution A.953(23) with respect to accuracy and integrity. The need for integrity and accuracy  has been  increased by developments such as the introduction of AIS and ECDIS. The use of IALA maritime DGNSS does fulfil these requirements. Additionally, international maritime standards exist for both receiver and DGNSS service. Some countries are considering a national mandatory carriage requirement for DGNSS receivers.

Although SBAS (WAAS, EGNOS, MSAS) enabled receivers are increasingly available and being marketed for maritime applications, these systems have not been standardised or approved for marine use and there are no initiatives to do so at present. In addition, limitations of performance in high terrain and at high latitudes need to be considered.

For these reasons the WG considered that carriage of maritime DGNSS receivers should be made mandatory for SOLAS class vessels. The recognition of individual service providers’ IALA DGNSS under the WWRNS would be the first step in this process. It would then be necessary to ensure that the mandatory carriage of DGNSS receivers is considered in the next revision of SOLAS Chapter V, but this was not likely before 2008.

An interim measure would be to prepare an IMO SN Circular pointing out the safety arguments for using DGNSS.  It would also be necessary to make the case for the adoption of the carriage requirement by presentations at meetings and conferences.

This process has been presented in the form of an action plan (Annex 4).

A provisional Timeline for future DGNSS was discussed and is included as Annex 5.

# Conclusions

The documents drafted at this meeting prepare the way forward for IALA DGNSS and are commended to the e-NAV Committee for approval.

ANNEX 1

# Attendance

Ryozo Shigematsu (Japan CG)

Yuji Nishi (Japan CG)

John Merrill (USCG)

Dave Wolfe (USCG)

Rolf Bäckström (Finnish Maritime Administration)

Kaisu Heikonen (Finnish Maritime Administration)

Rob Langen (Netherlands Ministry of Transport)

Peter Douglas (NLB, Scotland)

Colin Day (CIL, Ireland)

Luo Siming (China MSA)

Miao Meng (China MSA)

Michael Hoppe (Federal Waterways Administration, Germany)

Tom Braute (Norwegian Coast Directorate)

Arne Rinnan (Kongsberg Seatex)

Ivan Carlsson (Royal Danish Administration of Navigation & Hydrography)

Marek Dziewicki (Maritime Office, Poland)

Nick Ward (GLA, UK & Ireland)

ANNEX 2

IALA Recommendation R-xxx

On

the Future of DGNSS

##### DRAFT

##### March 2006

IALA Recommendation on the Future of DGNSS

### THE COUNCIL

NOTING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment,

NOTING ALSO IMO Resolutions A.915(22) on Maritime Policy for the Future Global Navigation Satellite System (GNSS) and A.953(23) on the World Wide Radio-navigation System,

NOTING FURTHER ITU-R Recommendation M.823-2 on the Technical Characteristics of differential transmissions for Global Navigation Satellite Systems (GNSS) from maritime DGNSS sites in the frequency band 285-325 kHz (283.5-315 in Region 1),

RECOGNISING the need to ensure that Differential GNSS (DGNSS) services in the frequency band 283.5-325 kHz continue to meet the minimum requirements set out in relevant ITU-R recommendations and IMO resolutions,

RECOGNISING ALSO that the equipment providing the DGNSS service in several members states requires replacement and that the present service will not provide for all the applications described in IMO Resolution A.915(22).

RECOGNISING FURTHER that some maritime applications are not fully met by the existing GNSS services.

HAVING CONSIDERED the proposals made by the e-Navigation Committee:

ADOPTS the strategy proposed for the Future of the IALA DGNSS, set out in the annex to this Recommendation; and

RECOMMENDS National Members and other appropriate Authorities providing or intending to provide DGNSS services in the band 283.5-325 kHz, to implement the strategy set out in the annex to this Recommendation.

ANNEX

### Strategy for the Future of IALA DGNSS

1. General

The IALA DGNSS beacon system was developed in the 1990s and systems were installed in many countries over the period 1995-2000. The system has been adopted as the international, maritime standard for providing differential corrections to Global Navigation Satellite Systems (GNSS). The Radio-navigation (presently e-Navigation) Committee assessed the current and potential use of the system, and concluded that there may be a requirement to recapitalise older systems. There is also potential to develop the system for the benefit of existing users and to enhance GNSS capabilities to take account of technical innovations, in accordance with IMO Resolution A.915 (22).

2. Alternative Technologies

There are potential alternatives to the IALA beacon system for the distribution of safety related differential services, in particular Satellite Based Augmentation Systems (SBAS) and the Automatic Identification System (AIS). SBAS, such as WAAS and EGNOS are now becoming operational and are being promoted for maritime use. The use of AIS as a means of broadcasting differential corrections has also been proposed and demonstrated. There are other ways of providing enhanced services, including Real Time Kinematic (RTK), Eurofix and pseudolites. All these options have been considered and a subjective cost–benefit analysis carried out, a summary of which is presented below.

| System | Accuracy | Coverage | Integrity/Continuity |
| --- | --- | --- | --- |
| IALA DGNSS | 1-3 m | local/regional | Yes/High |
| SBAS | 1-3 m | regional/global | Yes/High |
| AIS | 1-3 m | local | ?/Moderate |
| Pseudolites | sub-meter | local | Yes/Moderate |
| Eurofix | 1-3 m* | regional | Yes/High |
| RTK | sub-meter | local | ?/Low |

* assuming Loran system is available

?  depends on implementation

| ## System | Provider cost | User cost | Marine Standard |
| --- | --- | --- | --- |
| IALA DGNSS | moderate | low | Yes |
| SBAS | very high | low | No |
| AIS | low | low | Yes |
| Pseudolites | high | moderate | No |
| Eurofix | low* | moderate | No |
| RTK | moderate | high | No |

3. Factors Affecting Future of DGNSS

On the basis of the cost-benefit summary above the IALA beacon DGNSS is still the preferred option for maritime applications, particularly where existing infrastructure can be re-used. However, the time for a decision on its future is approaching. Driving factors include the age of equipment - the original computing and communications equipment has already become obsolete and been replaced in many cases. Some manufacturers of reference station and integrity monitor equipment have withdrawn from this market, although maintenance contracts will still be supported for a few years.

Normal re-capitalisation periods for such equipment would be 10 years. Budgetary requirements in most administrations dictate up to four years lead-time for planning re-equipment.

Developments in GNSS (GPS L2C, L5, and Galileo) will require the introduction of new message types and new equipment.  A concept of operations needs to be developed for the use of multiple frequencies for DGNSS services, to determine whether frequencies should be used separately or the data should be combined.

With the emergence of new technologies, the way in which the original DGNSS system was implemented needs to be re-evaluated. For example, instead of specialised, dedicated hardware units for reference stations and integrity monitors, it may be more cost-effective to consider off-the-shelf receivers with the reference station and integrity monitor functions implemented in software.

Receiver Autonomous Integrity Monitoring, RAIM, is a method that provides GNSS integrity information on the user receiver without external augmentation. RAIM is not continuously available with the present GNSS satellite constellation and is not likely to be fully available until the full implementation of Galileo. The current IMO GNSS receiver performance standards call for fault detection (RAIM). Future standards for receivers using combinations of GNSS will almost certainly contain the same requirements.

4. Applications

IMO Requirements for future GNSS cannot all be met by the existing system, e.g. 1 m accuracy. New approaches could meet those requirements and provide greatly enhanced services, but must preserve backward compatibility. Some new applications that could be served are hydrographic survey, port operations, berthing and Aids-to-Navigation management. The introduction of AIS and new telematic applications on inland waterways increase the requirements for higher integrity and accuracy than is provided for in the present IMO A.915(22).

5. Options for Future Development

If no action is taken the service from older systems will become unreliable, difficult to maintain and will no longer meet international requirements, therefore this is not an acceptable approach. Putting off a decision will make it more difficult and expensive to re-engineer or replace the system when it becomes essential to do so.

Systems could be modernised or replaced using the same architecture, but there are few potential suppliers and this is unlikely to meet future needs. This could lead to a proliferation of additional systems to meet specialised applications.

Development to meet future requirements and changes in the core GNSS would be the strategy most likely to result in a single integrated system meeting a wide range of applications, whilst maintaining backward compatibility and compliance with existing standards.

Discontinuing the service and replacing it with something else (SBAS, AIS) could render existing user equipment obsolete and would require the development of new standards. It is unlikely that this would be acceptable to administrations, providers or users.

6. Recapitalisation Requirements

The baseline requirements and principles for the re-capitalisation of the IALA DGNSS service are as follows:

- Maintenance of legacy signals (backward compatibility)

- Compliance with IMO future requirements

- Internationally applicable solution

- Ability to transmit additional messages

- Life-time of at least 10 years

7. Re-engineering Options

Action must be taken to prevent the service becoming unreliable and unable to meet international requirements.

Even if a decision is made to close down the IALA DGNSS system at some point in time, an adequate period of notice is required to allow users to update to alternative technologies. Since this period could be several years, older systems have to be recapitalised whether or not a decision is made to close it down eventually.

Recapitalisation offers the opportunity to provide enhanced performance to serve high integrity and high precision applications (IMO A.915 (22)). Furthermore, the system could be designed to support emerging and upgraded GNSS.

Replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM) would appear to be a low-risk option – the technology is known and procedures are in place. However, the choice of suppliers is limited and once this route is chosen, it will be difficult to develop the system to meet future needs. This could lead to an undesirable proliferation of non-standard systems to meet specialised applications.

There are at least three other options that should be considered: a software RSIM, a Virtual Reference Station and integration with SBAS. These options for re-engineering the IALA DGNSS system are considered here and further investigations proposed to enable the best choice to be made.

7.1 Software RSIM

An alternative already investigated by some administrations is to implement the Reference Station, Integrity Monitor, Communications and Control in

software, using commercial, off-the-shelf GNSS receivers for position and time input. This has the advantage of flexibility, the hardware would be easy to replace and reconfigure. However, the long-term maintenance of the software would be a concern. This might be partially alleviated by using software modules developed in a widely-used application language. This would also facilitate continuing development of the system to meet changing needs. Comparisons show that this is a cheaper option than like-for-like replacement of existing equipment.

7.2 Virtual Reference Station

Another option is to develop a Virtual Reference Station (VRS) network. In this case the corrections and integrity messages are computed at a central site using data fed from remote receivers. The receivers could be off-the-shelf, although with high specification. The software at the central site could be based on well-known principles and standard algorithms. This would offer performance advantages, since accuracy could be uniform over the coverage area, but the availability of the service would then be dependent on the communication links, which would also introduce extra running costs. Use of the internet and provision of backup systems would need to be considered. The use of data from an existing network such as those implemented by national survey organisations would be a possibility.

7.3 SBAS Integration

Integration with an existing SBAS, such as WAAS or EGNOS could offer a low-cost solution. This could be achieved in a number of different ways, some of which will be discussed in more detail under Network Techniques. The approaches range from SBAS receivers on each site to direct data-links to the SBAS control centre.

The service would then be dependent on the SBAS provider so that a Service Level Agreement would be needed and possibly backup systems would have to be considered. Performance would be limited to that provided by the SBAS-accuracy of a few meters at present. There are also limitations in coverage, especially at high latitudes.

8. Enhanced Performance

The recapitalisation solution should be able to enhance the level of service. Enhanced performance would include the ability to transmit additional messages, to cater for multiple GNSS and give better than 1 m accuracy, meeting harbour navigation, docking and other specialised requirements.

The capability of providing messages for Galileo, GLONASS M and GPS L2C and L5 would be required, even though these signals will not be fully available for some years.

A Real Time Kinematic solution using carrier-phase corrections would have the most potential for enhanced accuracy, but there are constraints imposed on data rates by the bandwidth of the present datalink. New modulation techniques could be used to broadcast additional messages, too long to fit into the existing low rate data stream. A hybrid data-link, capable of accommodating both legacy and new data channels could be employed, for example, with a 50-200 baud low rate channel for legacy services and a new high rate channel capable of providing RTK observables, detailed Troposphere & Ionosphere models, precise orbit data and emergency messaging.

The legacy DGNSS signal typically broadcasts RTCM SC-104 type 9-3 messages regularly to provide timely corrections and to provide integrity to meet the IMO requirement of 10 second time to alarm.  Typically DGNSS sites broadcast Type 9-3 messages for 12 satellites every 8.4 seconds at 100 bps. The repetition rate of these corrections was partially based on the dynamics of selective availability (SA).  With SA removal in 2000, the latency of the signal may be increased to meet the same accuracy standards.  Legacy standards provides for the increase of typical transmission rate to 200 bps. In addition, compression algorithms and alternative modulation schemes may be employed to harvest additional capacity in the data channel. With additional data channel capacity, additional capabilities may be available to the DGNSS system.

Reducing the frequency of type 9 message transmission should not affect the system ability to meet the legacy integrity time to alarm requirements.   Means may be used to mitigate this concern by allowing the type 9 messages to have priority to interrupt other broadcasts as necessary to meet requirements.   Additionally, messages may be checked prior to transmission to ensure quality.  This pre-integrity concept may reduce the potential interrupts to near zero. If needed, additional messages or integrity flags in other message headers can be devised to fulfill legacy integrity requirements.

Over the next several years, additional GNSS signals will be available to the public.   These signals include Galileo, GPS L5, GPS L1C, GPS L2C and GLONASS M.  Means may be available in the future DGNSS to augment these systems as well as GPS L1 to provide a higher level of accuracy and integrity to the user.

There may be merit in broadcasting corrections for the individual GNSS error components instead of only providing a single pseudorange correction encompassing all error components.   Error components such as the tropospheric and ionospheric errors are common to all GNSS systems and may be broadcast independently of components specific to the individual GNSS systems.   Independent error components such as precise orbits and precise time may be broadcast for each GNSS satellite.

In addition to the traditional MF data link, alternative data channels may be considered to transmit the data discussed above as well as additional high rate information such as GNSS observables.  Alternative data transmission methods include internet gateways and broadcasting over other RF bands that can carry other DGNSS data.

With the availability of additional time slots in the DGNSS data channel, means may be deployed to utilize type 16 or similar messages to broadcast standard, pre-determined, non-navigational information to the user community such as, but not limited to, weather conditions, search/rescue information, ATON warnings, etc.

Range Rate Corrections (RRC) should be removed. In some conditions the error may actually become larger by using RRC and their removal could reduce the amount of data by 20%, reducing data latency and improving position accuracy.

New message types, such as message 27 for improving station selection, should be implemented.

9. DGNSS Network Techniques

Various techniques are under consideration for the networking of DGNSS stations. In the Regional Area Augmentation System (RAAS) mode the radio-beacon would be left unchanged, but a new type of receiver would be required by users. This would be compatible with the existing standard, but able to take advantage of RAAS-mode operation by virtue of its multi-channel reception capability and RAAS software.

A wide area network implementation could involve integration with SBAS, as discussed in the re-engineering options. The beacon stations would no longer necessarily be equipped with reference stations. Instead, they would be supplied with an SBAS (e.g. WAAS) message compiled from the data measured at multiple reference stations (not sited at the beacon stations). The beacons would transmit this message, using an extended version of the present broadcast standard. The hardware of the user’s receiver would be unchanged, but its software would generate an RTCM output, to be passed on, as now, to the associated GNSS receiver.

In transmitted RAAS each DGNSS station would acquire the messages transmitted by its neighbours, by means of a receiver. It would transmit this message, using an extended version of the present broadcast standard. This should be well within the data capacity of the present system. The hardware of the user’s differential receiver would be unchanged, but its software would generate an enhanced RTCM output to the associated GNSS receiver, using the combined set of messages.

The beacons could eventually be fully integrated with SBAS, re-broadcasting the data of the US WAAS or the European EGNOS systems. In this case the IALA DGNSS stations would become terrestrial alternatives to the geo-stationary satellites. The data capacity of the channel would need to be enhanced, probably by means of new modulation techniques now under development. The hardware of the user’s receiver would have to be changed to accommodate this new transmission, but the receiver’s software would perform the normal WAAS or EGNOS operations to generate an RTCM output.

10. Further Work

Further investigations are required before proposals for the enhanced future system can be put forward for international consideration, so that specifications can be drawn up. In particular, the cost and feasibility of the re-engineering options must be investigated. The advantages and disadvantages of the DGNSS networks need to be explored, the new modulation techniques require evaluation and new message types have to be agreed and standardised.

11. International Standards

Any solution proposed must meet international requirements, since the present service is the maritime standard. The technical characteristics of the system and any changes to spectrum allocations must be agreed by ITU, message formats are generally developed and approved first in RTCM and then the ITU Recommendation is revised accordingly. The operational requirement and receiver performance standards must be approved by IMO.

System performance standards and operating procedures are dealt with by IALA and IEC produces receiver specifications. A lead-time of several years must be allowed for the regulatory process if any substantial change is planned to the existing system.

DGNSS is not yet recognised as a mandatory carriage requirement under the IMO SOLAS Convention. Also, most countries have not submitted their DGNSS as a component of the World Wide Radio-navigation System. These issues should be addressed in parallel with any re-engineering of the system.

As long as international standards are met, the exact method of implementation remains a decision for the individual administration, although there may be advantages in sharing technical developments.

12. Recommended Strategy

It is recommended that

- IALA members should submit their DGNSS as a component of the World Wide Radio-navigation System

- IALA members providing DGNSS services should plan the re-engineering of older services to enhance performance and meet future maritime requirements.

- The alternative development routes for IALA DGNSS (software RSIM, VRS and SBAS integration) should be investigated in detail and costed.

- Whichever option is chosen it should meet the minimum requirements set out in Section 6 and conform to the standards mentioned in Section 11.

- The probable need to provide additional messages relating to Galileo, GPS L2C/L5 and Glonass M should be taken into consideration.

- The feasibility of providing additional signals giving sub-meter accuracy should be investigated.

- The advantages of networking stations should be explored.

- The possibility of additional integrity messages or message header flags should be considered.

It is further recommended that concurrent studies be conducted in the following subjects:

- Service providers should define the intended performance levels (accuracy, availability, integrity, reliability, continuity) before specifying solutions.

- Determine the maximum available time slots in the DGNSS channel that may be available to other types of data

- Determine the merit of pre-integrity, identify the rules for integrity interrupt and develop any additional integrity messages or RTCM integrity message headers that may be required.

- Develop the basic message types and transmission schemes for independent error components to provide for new and existing GNSS signals.

- Investigate alternative data channels and alternative uses of the DGNSS broadcast channel.

13. References

  - International Maritime Organization. “World-Wide Radionavigation System”, IMO Resolution A.953(23)

  - International Maritime Organization. “Maritime Policy for a Future Global Navigation Satellite System (GNSS) IMO Resolution A.915(22)

  - International Telecommunication Union – Radiocommunication Sector (ITU-R), “Technical Characteristics of Differential Transmissions For Global Navigation Satellite Systems (GNSS) from Maritime Radio DGNSS sites in the frequency Band 283.5-315 kHz in Region 1 and 283.5-325 kHz in Regions 2 and 3”, Recommendation M.823-2, Geneva 1997

  - IALA Recommendation on the performance and monitoring of DGNSS services in the frequency band 283.5-325 kHz

  - Presidential Decision Directive NSTC- tbc.

  - Ward, N, IALA;  “The Future of IALA DGNSS”  Proceedings of the Institute of Navigation, NTM-2006, Jan 2006.

  - Gutman, Seth; Wagoner, Sher; Mihal Codrescu, Mihal; and Fuller-Rowell, Tim;  National Oceanographic and Atmospheric Administration; The Use of Atmospheric Models to Improve Differential GPS Positioning Accuracy.

  - Last, David; Grant, Alan; Williams Alwyn; Ward, Nick; Enhanced accuracy by regional operation of Europe’s new radiobeacon differential system.  Proceedings of the Institute of Navigation, ION-2002, Sept 2002.

  - Radio Technical Commission for Maritime Services, RTCM Special Committee No. 104, RTCM Recommended Standards for Differential Navstar GPS Service, V2.3, RTCM Paper 136-2001/SC104-STD, August 20, 2001.

  - Radio Technical Commission for Maritime Services, RTCM Special Committee No. 104, RTCM Recommended Standards for Differential NAVSTAR GPS Reference Stations and Integrity Monitors (RSIM), V1.1, RTCM Paper 12-2001/SC104-248, May, 2001.

  - Kee, Changdon; and Kim Jeonghan; Efficient Transmission Technique of Compact RTK Correction Messages for Low-rate RTK Data-link. Proceedings of the Institute of Navigation, ION-NTM 2003, January 2003

  - Hartnett, Richard; Gross, Keith; Bovee, Ben; Nassar, Sam; Johnson, Gregory; Sanders, George; Swaszek, Peter; Wolfe, David; DGPS Accuracy Observations and Potential Data Channel Improvements. Proceedings of the Institute of Navigation, ION-AM 2003, June 2003.

  - Park, B., Kim, J., Jun, S., Kee, C., Kalafus. R.; The Need for Range Rate Corrections in DGPS Correction Messages, , Proceedings of the Institute of Navigation, ION-NTM 2004, Jan 2004.

  - Bartone, C., Barton, I., Zhang, Y., Cleveland, A., Parsons, M.; Antenna Performance Analysis for a High Performance NDGPS Architecture, Proceedings of the Institute of Navigation, ION-NTM 2004, Jan 2004.

  - Ferguson, K., Albright, M., Remondi, B., Cleveland, A., Parsons, M.; NDGPS Reference Station and Integrity Monitor Architecture Modernization, Proceedings of the Institute of Navigation, ION-AM, June 2004.

  - Dr. Remondi, Benjamin, Ferguson, Kendall, Albright, Michael; HA-NDGPS RTK Positioning as a Function of Baud Rate 50 – 1000 bps, Proceedings of the Institute of Navigation, ION-AM, June 2004.

ANNEX 3

Template for Submission of DGNSS Services for Recognition as Components of the IMO WWRNS

Background

IMO Resolution A.953(23) paragraph 3 invites Governments and organizations providing radionavigation systems to consent to recognition of these systems by IMO for inclusion as a component of the Worldwide Radionavigation System (WWRNS). The subject resolution outlines five criteria that the IMO should consider when deciding whether or not to recognize a radionavigation system. They are listed below with an explanation of how the [organisation] system meets the criteria.

# Discussion

1. The Government or organization providing, funding and operating the system has stated formally that the system is operational and available for use by merchant shipping.

Maritime DGPS was declared Full Operational Capability (FOC) on March 15, 1999. FOC declares that the system operates to the recommended standards set forth in the Federal Radionavigation Plan (FRP). The 1999 edition of the FRP similarly declares Maritime DGPS FOC and ready for use.

2. Its continued provision is assured.

The 1999 edition of the FRP states “...GPS will be the primary Federally provided radionavigation system for the foreseeable future.” The Maritime DGPS system is a vital augmentation of the GPS system that has gained a large number of users. We can state with absolute certainty that the Maritime DGPS system will be a cornerstone of U.S. radionavigation for years to come. Indeed, the Maritime DGPS service is being used as the basis to expand this system across the U.S.

3. It is capable of providing position information within the coverage area declared by the Government or organization operating and providing the system with an accuracy not less than that given in the Appendix.

The DGPS system is operated under the parameters set out in the USCG Broadcast Standard for the DGPS Navigation Service and the FRP. Both of these documents have criteria and standards which are in accordance with the guidelines given in the Appendix of IMO Resolution A.953(23).

4. Adequate arrangements have been made for the publication of the characteristics and parameters of the system and of its status, including amendments as necessary.

DGPS system characteristics and parameters are published in USCG Broadcast Standard for the DGPS Navigation Service and the FRP. System status is maintained and updated on the Navigation Center webpage and can be obtained on the webpage,  via phone or by email.

5. Adequate arrangements have been made to protect the safety of navigation should it be necessary to introduce changes in the characteristics or parameters of the system which could adversely affect the performance of shipborne receiving equipment.

Changes to the DGPS system characteristics or parameters will be published in the FRP and on the Navigation Center webpage. If temporary changes are made on short notice or in an emergency, users will be notified through the USCG Broadcast Notice to Mariners, USCG Local Notice to Mariners and the Navigation Information Service at NAVCEN.

# Future and Implications

This offer to IMO reiterates the United States commitment to availability of the DGPS system for the foreseeable future on a continuous basis and free of direct user fees as provided in the Current Marine Navigation Requirements of the Federal Radionavigation Plan. Furthermore, the U.S. has stated that we will take all necessary measures to maintain the integrity of the service and expects to be able to provide at least 6 years notice prior to termination of DGPS operations. The USCG DGPS system meets the five criteria in IMO Resolution A.953(23) and should be accepted as part of the WWRNS. Furthermore, it is believed that making this system available in the U.S. and its acceptance by IMO as part of the WWRNS will encourage other states to continue to develop and deploy compatible systems to gain acceptance as part of the WWRNS.

(Greyed out text is the US response, given as an example for guidance only)

ANNEX 4

# Action Plan for Achieving Mandatory Carriage of DGNSS Receivers

1. Prepare a presentation on the need for DGNSS carriage for use at appropriate meetings and conferences.

2. Draft an IMO Safety of Navigation Circular for input to IMO by IALA, pointing out the safety benefits of using DGNSS.

3. Encourage IALA Members to bring their DGNSS services to full operational status and declare them operational.

4. Each Administration to submit their DGNSS service to IMO for recognition under the WWRNS.

5. Prepare a submission to IMO proposing mandatory carriage of DGNSS receivers.

6. Ascertain which national Administrations would support a submission to IMO for mandatory carriage of DGNSS receivers in the next revision of SOLAS Chapter V.

7. Make the submission to IMO.

ANNEX 5

#### Provisional Timeline for Future DGNSS

Galileo	Galileo	GPS modernization

OS FOC	SoL FOC        IOC           FOC

l              l              l             l

| 2006 | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

specification      tendering     installation >	    	specification      	tendering     	installation   >

Re-capitalisation program 1st phase

Legacy DGNSS

Re-capitalisation program 2nd phase

Future DGNSS