New Availability and Continuity definitions and standards – a proposal

Luís Sardinha Monteiro

Portuguese Naval Academy

- BIOGRAPHY

Luís Sardinha Monteiro is an Officer of the Portuguese Navy, currently serving at the Portuguese Naval Academy as Professor of Navigation. He holds a MSc in Navigation Technology from the University of Nottingham and a PhD from the same University with a thesis about DGPS. He is Vice-President of the Portuguese Institute of Navigation.

1. Current definitions and standards

Generally, radionavigation systems are described by means of a set of parameters, which are coverage, availability, continuity, accuracy and integrity. The first three parameters are deeply related to each other and are the main topic of this paper. Recently, some concern has risen, namely among IALA and the EMRF, regarding continuity standards. However, there are also some aspects of the definitions and standards of coverage and availability, which require a revision. Therefore, this is a good opportunity, not only to review the continuity standards, but also to take a look at coverage and availability, with the aim of building a more coherent set of definitions and standards.

Coverage, availability and continuity are very important parameters of a radionavigation service – although the two latter ones are clearly much more important than coverage, because what is the use of a system covering a certain area if it is not available for an adequate percentage of time and if its service is not continuous?

IMO and IALA define coverage as “that surface area or space volume in which the signals are adequate to permit the user to determine position to a specified level of performance” [References 1 and 2]. This definition, which is very similar to the one included by the US authorities on the successive “Federal Radionavigation Plans”, seems insufficient, because it does not mention the need to keep a certain level of integrity. In other words, this definition seems tailored for stand-alone GPS and is of little value when applied to augmentation services, because it does not mention the need to keep a certain level of integrity.

Availability is defined in IMO Resolution A.915(22) [Reference 1] as “the percentage of time that an aid, or system of aids, is performing a required function under stated conditions. Non-availability can be caused by scheduled and/or unscheduled interruptions”. In the IALA / IMO standards, availability is expressed over a 30-day period in the ocean phase, but is calculated over 2 years in coastal navigation and harbour approaches and entrances.

This definition of availability is not sufficiently clear, because it does not specify what means “performing a required function under stated conditions”. Nevertheless, the availability percentages required on IMO Resolution A.953(23) [Reference 3] apply to signal availability, which is defined by IMO as “the availability of a radio signal in a specified coverage area” [Reference 1]. From this definition, it is possible to conclude that signal availability is affected not only by the transmitter availability (i.e. down-time periods of the transmitter) but also by atmospheric noise, interference and fading, all of which determine the availability of the radio signal within the coverage area. Nevertheless, and despite IMO clearly mentioning that the required availability percentages are for signal availability [Reference 3], the IALA “Recommendation on the Performance and Monitoring of DGNSS Services” does not take the propagation losses into account when providing guidance for the calculation of signal availability, although they are considered in the discussion of coverage prediction. This becomes more important since IMO Resolution A.953(23) states that the signal availability is “calculated with guidance contained in IALA Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5-325 kHz” [Reference 3].

In practice, the availability of a navigation solution for the mariner is, seldom, even lower than the signal availability, because it depends also on the performance of the user’s DGPS receiver. The availability at the user is designated by IMO system availability (although some call it user availability) and includes signal availability and the performance of the user equipment.

“Continuity” is a new concept, which recently replaced the term “reliability”. The definition of continuity, as included in IMO Resolution A.915(22) [Reference 1], is “the probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area”. In this case, the IMO definition is clearer as it states that continuity is measured at the receiver and that the continuity assessment implies the capability to determine accurate positions with adequate integrity, which seems the most reasonable approach. Nevertheless, IALA, in Reference 4, sees continuity only from the viewpoint of the service provider, contradicting the definition itself.

In what concerns availability and continuity requirements, the current standards are the ones contained in Table 1. This table has been approved at the 23rd IMO Assembly [Reference 3] and has been included by IALA in its “Recommendation on the Performance and Monitoring of DGNSS Services in the Band 283.5-325 kHz” [Reference 5].

| Area | Absolute Horiz. Accuracy | Signal Availability | Continuity | Augmentation |
| --- | --- | --- | --- | --- |
| Ocean | 100m | >99.8% over 30 days | N/A | None |
| Coastal/harbour with low volume of traffic and/or low degree of risk | 10m | >99.5% over 2 years | 99.85% over 3 hours | Single station |
| Coastal/harbour with high volume of traffic and/or high degree of risk | 10m | >99.8% over 2 years | 99.97% over 3 hours | 2 or more stations |

Table 1 – Accuracy, availability and continuity standards [Reference 5]

As the definition of continuity states, this parameter must be measured on a “(short) time interval” and IMO and IALA decided to adopt 3-hour periods [References 1, 2 and 3], because the intent of having a parameter like continuity is “to provide guidance for continuity of service during a specific manoeuvre, such as port entry or docking (…) [which take] typically less than 3 hours” [Reference 6]. The aviation sector adopted a much shorter interval of 150 seconds for approach and landing [Reference 7], which seems more appropriate than the period adopted for marine navigation. With the extended 3-hour period, the required standards of 99.97% for waters with a high risk and of 99.85% on waters with lower risk become very difficult to meet, especially when it comes to the need of providing a 10-second TTA.

Analysing these three definitions altogether, it is possible to say that they do not form a homogeneous body, as some contradictions arise between them.

The definition of coverage seems tailored to apply to unaugmented satellite navigation systems and is of little value for augmentation services, because being integrity a key service provided by differential systems, it is not considered on the definition of coverage. Therefore, for DGPS systems, availability is a much more useful parameter. However, when it comes to availability, IMO and IALA have produced a set of standards and concepts which is not entirely coherent and homogeneous.

The availability standards apply to signal availability (i.e. not considering the receiver performance), while the continuity standards do consider the performance of the receiver. Moreover, the guidance provided by IALA to calculate DGPS signal availability and continuity is not entirely adjusted to the definitions, because it neglects propagation losses and bases the calculations only on scheduled and unscheduled interruptions of the Broadcast Station. This is a very restrictive interpretation that excludes the effects of noise, interference and skywave propagation.

The diagram below illustrates how the radionavigation parameters, with their current definitions and standards, interact with each other.

Figure 1 – Interaction between current radionavigation parameters

- 2. Practical results on availability and continuity

During the process of validating the Portuguese DGPS network, on 2003 and 2004, several trials were conducted to assess the respective performance, with a special emphasis on accuracy, availability and continuity. The availability and continuity trials led to important conclusions, which contribute to clarify the adequacy of the current definitions and standards and may also help on their revision.

The measurement of field availability and continuity was based on studying the behaviour of DGPS receivers. This evaluation relied on analysing the age of corrections as output by the receivers, which is a pragmatic way of incorporating the performance of the receiver in the overall measured availability, because it allows assessing if the DGPS receiver, for any reason (external or internal), lost any corrections message. A temporary loss of signal is seen as an increase in the age of corrections and, eventually, a reversion to stand-alone GPS (if the loss of signal takes more than the maximum age of corrections – 4 minutes in the receiver used in this trial: Trimble DSM 212).

With type 9-3 messages broadcast at 200 bps, the age of corrections ranges from about 2 seconds to about 3 seconds, considering no losses of propagation. The interference of groundwave signals from other beacons and of skywave signals, from own station or from distant beacons, can cause the loss of some messages, therefore increasing the age of corrections. These effects, namely ionospheric interference, are stronger at larger distances to the Broadcast Station and, therefore, system availability and continuity are lower at the coverage boundary.

To measure availability experimentally with confidence when the level of availability is high requires the collection and analysis of sufficiently large amounts of data to give statistical significance to the results. However, this was beyond the range and the scope of these validation trials, because it would imply very large resources. Therefore, this assessment was based only on three days of data gathered on each site, which means that the results must be analysed with caution. This does not diminish their value for assessing the adequacy of the definitions of radionavigation parameters and of the corresponding standards for the maritime sector. For the analysis of results, a restrictive interpretation of availability was adopted, according to which the DGPS service is only available when providing the stated accuracy of better than 5 m (95%) and the advertised TTA of less than 10 seconds.

In what concerns continuity it is the definition itself, which implies the ability to determine position with specified accuracy and to monitor the integrity.

The full results of the validation trials can be found on Reference 8, but only the most relevant results will be presented and analysed in this paper, namely the performance at Varzim when tuned to Sagres Broadcast Station and the performance at Lisbon when tuned to Carvoeiro Broadcast Station.

  - Availability and continuity at Varzim receiving Sagres Broadcast Station

The collection of data at Varzim occurred from 6 to 9 June 2003. Varzim is a suburban area, 264.7 NM away from Sagres Reference Station. It is sited beyond the estimated night-time coverage of this beacon, but it is still within the day-time coverage, according to the coverage predictions made with the software developed by the team led by David Last, at Bangor.

The next plot illustrates the ages of corrections of the DGPS receiver sited at Varzim, showing several losses of signal, which on certain occasions amounted to nearly 1 minute.

In terms of accuracy, it was found that the frequent losses of signal at Varzim did not affect significantly the user performance, because with no SA, differential corrections remain valid for several minutes. During the 3-day period the 95% error at Mira was 1,84 m.

In what concerns the integrity, the impact of the signal losses is much more relevant. The total TTA is obtained by adding three parcels: Reference Station processing time + duration of the message which includes the integrity warning + receiver processing and decision time. The age of corrections, as output by DGPS receivers, incorporates the first two parcels, to which it must be added the time the receiver takes to respond to integrity flags. On high-end receivers this period can be reduced – 0.4 seconds, according to Rudolph Kalafus [Reference 9], but on lower quality equipment it can be several seconds. Therefore, when the corrections age reaches 9.6 seconds, it will be very difficult even for the best DGPS receiver to avoid exceeding the TTA threshold of 10 seconds.

The plot from Varzim is a very typical one, because all the occasions in which the corrections age exceeded 9.6 seconds occurred at night, when the fading caused by signals received via ionospheric paths (either from the own beacon or from distant beacons) is higher and when the atmospheric noise is also higher.

Plot 1 – Age of corrections at Varzim (Sagres beacon)

During the day, the service being provided met the advertised accuracy and integrity during the totality of the time, and at night only for 0.53% of the time was the 10-second TTA exceeded.

With regard to service continuity the scenario is much darker. As mentioned before, each time the age of corrections rises above 9.6 seconds, the capability to monitor integrity is compromised and the continuity of service is interrupted, meaning that the continuity was not ensured for the previous 3 hours. The next plot illustrates with red transparent boxes all periods during which continuity of service was not provided. Those boxes extend from each service interruption until 3 hours before.

Plot 2 – Continuity of service at Varzim (Sagres beacon)

It is clear that at night there was hardly a continuous period of 3 hours with no messages loss, which compromised the continuity at night-time and in the period before sunset.

  - Availability and continuity at Lisbon receiving Carvoeiro Broadcast Station

This collection of data occurred from 13 to 16 July 2004. During this period data was recorded on 3 different monitoring sites, but the most interesting results were obtained in Lisbon. In this site the loss of only a few messages resulted in a very low continuity, despite the availability being perfectly acceptable. Lisbon is only 40.3 NM away from the Broadcast Station of Carvoeiro and is clearly within the coverage of this beacon, by day and by night, according to the predictions made with the Bangor Coverage-prediction Model.

The plots of age of corrections confirm the good availability at this site, as only twice did the corrections age rise above 9.6 seconds, which is the threshold beyond which the TTA requirement cannot be met. Interestingly both occasions coincided with dawn: 04h51m on the 14th of July and 04h39m on the 16th of July. It is well known that during twilights the propagation of radio signals is severely affected by the re-composition of the ionospheric layers and, in this case, this effect manifested itself very clearly. Probably, the re-configuration of the ionosphere created favourable conditions for the transitory reception of a long-distance signal which interfered destructively with the direct path signal from Carvoeiro.

Nevertheless, during the 3 days of data gathering, the TTA requirement of 10 seconds was violated only for approximately 2 minutes, corresponding to a system availability of 99.96%, which is very satisfactory. However, with regard to continuity the situation was much worse, because each of the two times messages were lost for more than 9.6 seconds, the continuity of service was not provided for a period of 3 hours. Therefore, according to the current way of calculating continuity, its percentage was only slightly above 90%. These results are the clearest evidence of the need to review the continuity standards, because in a site where only a few messages were lost for more than three days, the corresponding system availability was above 99.95% and the continuity was only about 90%...

Plot 3 – Age of corrections at Lisbon (Carvoeiro beacon)

  - CONCLUDING REMARKS ABOUT THE FIELD RESULTS

This assessment confirmed with real data that the concerns regarding the standards for continuity are entirely justified. In fact, on some sites with very high system availability, the measured continuity of service was low. The reason behind this is the very long continuity interval adopted in the maritime sector, coupled with the relatively high percentages required in IMO Resolution A.953(23) [Reference 3]: 99.97% or 99.85%, depending on the volume of traffic and degree of risk.

| Monitoring site | Broadcast Station | Distance (NM) | Accuracy (95%) | Measured availability |  |  | Measured continuity |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Day | Night | Total |  |
| Varzim | Sagres | 264.7 | 1.84 m | 100% | 99.47% | 99.80% | ~50% |
| Lisboa | Carvoeiro | 40.3 | 1.04 m | 100% | 99.90% | 99.96% | ~90% |

Table 2 – Short term availability and continuity DGPS beacon on different sites

In what concerns availability, the standards seem reasonable, despite the standards applying to signal availability and the tests having assessed system availability.

Another known effect which was observed on this trial had to do with propagation disturbances during twilights. At dawn and dusk, the propagation of radio signals is severely affected by the recomposition of the ionospheric layers, which can create favourable conditions for the transitory reception of long-distance signals, capable of interfering destructively with the direct path signal.

- Proposed definitions and standards

The main issue with regard to the new definitions of availability and continuity (and also coverage) will be turning them into “service-oriented”. The current definitions, besides being incoherent, are formulated in terms of the service providers or in general terms. It is time to move to the application field, because users expect a proper formulation that describes the performance of the service being provided.

Availability can then be defined as the percentage of time that, assuming a fault-free receiver, a user is able to determine position with specified accuracy and is able to monitor the integrity of the determined position over a relatively long-time period (30 days in ocean navigation and 2 years in coastal navigation and harbour approaches and entrances). With this definition, availability would be affected not only by the transmitter availability (i.e. down-time periods of the transmitter) but also by atmospheric noise, interference and fading and also by the receiver performance.

As for continuity, the current definition seems appropriate as it states that continuity is measured at the receiver and that the continuity assessment implies the capability to determine accurate positions with adequate integrity. The definition could then remain as: the probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area.

With those proposed definitions for availability and continuity, the concept of coverage can be re-written, so as to mean the surface area or space volume in which the service is available for users (i.e. the user availability is equal or above the standards), with the required continuity, accuracy and integrity.

Getting back to continuity, another important issue to be addressed is the current required percentages and the time interval, which are clearly too demanding.

Assuming that 2 short interruptions of service per day are acceptable and tolerable (one at dawn and another at dusk) to have a continuity of 99.97% then the continuity interval should be 12.96 seconds and to have a continuity of 99.85% then the interval should be 64.8 seconds. However, these intervals are too short, which means that not only the 3-hour continuity interval must be decreased but also the continuity percentage has to be more permissive.

Let’s begin by discussing the continuity interval. This issue is also under discussion at IALA, whose Radionavigation Committee was the first to raise the question of the too long duration of the continuity interval [Reference 10]. The current interval of 3-hours was adopted because some specific manoeuvres, such as port entry, anchoring or docking, usually take less than 3 hours. However, as the aviation sector did, by adopting a 150-second interval, the continuity interval must express the duration of critical manoeuvres or parts of the manoeuvre and during a long entry manoeuvre, taking 3 hours, there are less critical moments – when the loss of the radionavigation service does not compromise the safety of navigation, because there is time to change to an alternative means of obtaining the position. Additionally, during a long entry manoeuvre, taking 3 hours, there is the opportunity to lay anchor – in case of loss of the radionavigation service – and therefore not compromise the safety of navigation.

The Aids to Navigation Committee of IALA considers three phases of manoeuvres in restricted waters navigation: turning, recovering and track keeping, with turning and recovering being the most critical ones [Reference 11]. According to that committee, it is not possible to define the time each phase lasts, because it depends on a lot of internal and external factors such as vessels size, draught, rate of turn, speed, water depth, weather etc, but experience says that maximum times of manoeuvring in restricted waters are:

  - Turning: 10 minutes

  - Recovering: 3 minutes

  - Track Keeping: 3 hours

Although the period associated with recovering is the shortest, it is clear to all mariners that the phase when a ship is more vulnerable is turning. This is a first reason for the adoption of a continuity interval of 10 minutes. One can also add that 10 minutes is a reasonable period for a large ship, in case of loss of positioning information, to lay anchor. Therefore, a continuity time interval of 10 minutes is proposed, because it corresponds to the period associated with the most critical phase of restricted waters navigation (turning). Additionally, it is not likely for a ship to be involved in a manoeuvre, entirely and critically dependant on the radionavigation service for a longer period.

Accepting this interval and accepting that 2 short (less than 30 seconds) interruptions of service may occur each day (one at dawn and another at dusk) without seriously affecting ships transiting on harbours with high volume of traffic and/or degree of risk, then it is possible to do the calculations backwards, which gives as a result a continuity percentage of 98.5%. It is important to say that with these standards, only short interruptions are permitted – the kind of disruptions likely to occur at dawn and dusk. If longer interruptions take place then the 98.5% continuity requirement would be compromised, not forgetting that longer interruptions would also affect the availability performance. As the difference between more and less risky waters is already reflected in the availability standards, it is proposed the adoption of this same continuity requirement of 98.5% over 10 minutes on waters with low volume of traffic and/or degree of risk.

| Area | Absolute Horiz. Accuracy | Signal Availability | Continuity |
| --- | --- | --- | --- |
| Ocean | 100m | >99.8% over 30 days | N/A |
| Coastal/harbour with low volume of traffic and/or low degree of risk | 10m | >99.5% over 2 years | 98.5% over 10 minutes |
| Coastal/harbour with high volume of traffic and/or high degree of risk | 10m | >99.8% over 2 years |  |

Table 3 – Proposed availability and continuity standards

With the proposed definitions and standards, the most important radionavigation parameters will interact in a more coherent way, as shown on Figure 2.

Figure 2 – Interaction between proposed radionavigation parameters

Coverage would be the area where the service could provide the required availability, continuity, accuracy and integrity. Continuity would no longer be a kind of more demanding availability with very stringent requirements. It would be a parameter with some similarities to availability, but with a special focus on short time (10 minutes) performance, while the focus of availability is on long period performance (2 years). In other words, availability and continuity would be two sides of the same coin. Then, after providing the appropriate availability and continuity, the radionavigation services would have to provide the stated accuracy and integrity, with this last parameter being, naturally, the most stringent. Finally, such “service-oriented” definitions would constitute a move to the application field, shifting the current focus from the service providing to the user.

REFERENCES

- IMO, Resolution A.915(22) - “Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS)”, 29 November 2001

- IALA, “Recommendation on the Performance and Monitoring of DGNSS Services in the Band 283.5-325 kHz – IALA Recommendation 121” (RNAV21/output/01), 2004.

- IMO, Resolution A.953(23) - “World Wide Radionavigation System”, 5 December 2003.

- IALA Radionavigation Committee, “Draft Policy Paper on the Future of Marine LF/MF DGNSS” (RNAV 15/5/3r1), 15th Session, 8 March 2001.

- IALA, “Recommendation on the Performance and Monitoring of DGNSS Services in the Band 283.5-325 kHz – IALA Recommendation 121” (RNAV21/output/01), 2004.

- IALA Radionavigation Committee, “Draft Note to the IALA Council on Availability and Reliability for Modification of IMO Resolution A.815(19)” (RNAV 14/5/7), 14th Session, 14 September 2000.

- Fairbanks, M., Ward, N., Grant, A., Roberts, W. Dumville, M., Ashkenazi, V., “Maritime GNSS performance”, Proceedings of the ENC-GNSS 2004 Conference, 16-19 May, Rotterdam (The Netherlands)

- Sardinha Monteiro, L., “designing, configuring and validating the Portuguese DGPS network”, PhD Thesis submitted to the Institute of Engineering Surveying and Space Geodesy (The University of Nottingham), December 2004.

- Kalafus, R. M., “GPS Integrity Channel RTCA Working Group Recommendations”, “Journal of The Institute of Navigation”, Vol. 36, N.º 1, Spring 1989

- IALA Radionavigation Committee, “Liaison Note – Discussion Paper on Continuity Requirements” (RNAV 23/output/03rev1), 23rd Session, 23 September 2005.

- IALA Aids to Navigation Management Committee, “Liaison Note – Discussion Paper on Continuity Requirements from RNAV 23”, 7th Session, October 2005.