5.3.1 Transmission of Base Station Reports when a base station (in the

given area) is acting as a semaphore

The assignment of the transmission of Base Station Report (msg 4) with increased

reporting rate due to a base station switching into semaphore mode is justified, be-cause

the reporting interval is decreased to a value below 4 seconds. This constitutes a

When defining the usage of Base Station Report (msg 4) with increased reporting

rate due to a base station acting as a semaphore, the following aspects should be taken

- Table 1B, footnote 1, of Recommendation ITU-R M.1371-1 states, that the in-creased

reporting rate by a base station should be once per 3 1/3 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repeti-tive

pattern.

- This translates into eighteen (18) slots reserved for that purpose per frame for a reporting interval of 125 slots or into

a FATDMA increment of 250 (msg 20)  in one FATDMA reservation on each channel.

- Since the requirement for a base station to act as a semaphore is infrequent and

transient in nature, one or several short-lived FATDMA reservations may suffice

for the duration of the base station acting as a semaphore.

When transmitting Base Station Reports (msg 4) with an increased reporting rate due

a) there should be one (1) slot reserved with an increment of 250 on each channel, and

b) the required FATDMA reserved slots should be allocated by one [Short-Lived]

FATDMA reservation with time-out value pre-set to maximum, i. e. 8 minutes,

and consecutive [Short-Lived] FATDMA reservations with the same parame-ters

as long as the base station is acting as a semaphore (see above).

c) Instead, the required FATDMA reserved slots may be taken from a continu-ously

and in advance reserved pool of usage category 2 slots within the same

FATDMA scheme, FATDMA schedule or FATDMA plan. Although the latter op-tion

would smooth the operation of the VDL by avoiding transient FATDMA res-ervations,

more VDL capacity will be used when it will not actually be needed.