Drafting Group Output

Interference Issues for AIS Implementation

The WG reviewed test results from studies by MariTEL and the USCG, which investigated the potential for some AIS shipborne units to cause interference to other users of the maritime channels.  This potential interference can vary widely by manufacture models and may occur under various conditions.  The source of this interference is the AIS ramp-up and ramp-down, which is not tested in IEC 61993-2 test specification.  The WG voiced concern that if Class A manufactures are deviating from ITU-R M.1371-1 guidelines for AIS message ramp-up and ramp-down, Class B Carrier Sense-TDMA may be impacted.

The WG recommends the following actions:

- Add a precautionary note to the IALA Guidelines of the AIS VOL 1 Part II Part B/section 4.3 as follows:

4.3– Potential mobile station interference with Shipborne Installations

Test results of AIS mobile station devices highlight the possibility that some AIS shipborne units cause interference to other maritime users and other maritime users to cause interference to AIS devices.  This interference can vary widely by manufacture model and may occur under various conditions.

AIS interference manifests itself differently for different maritime services.  Maritime voice services may experience a “pop” during communications, which varies in loudness based on the level of interference.  Maritime data services comparably, may experience higher bit-error rates and lower throughput at comparable levels of interference.  Similarly, other maritime services may also cause interference with AIS units that impact AIS performance.

The following steps may be useful to alleviate potential interference.

  - Insure the latest IMO installation shipborne guidelines are followed – currently SN/Circ.227- 6 January 2003.

  - Provide the maximum RF separation available between AIS and other maritime systems.

  - Additional engineering techniques such as filtering or unique antenna designs may also further reduce interference.

2.	While the WG generally agrees that IEC-61993-2 test standard should be modified to fully test the entire AIS transmission and, while considering the need to modify other documents, the WG feels a better understanding of class B developments and operational requirements is needed to finalize these recommendations.

3.	This paper, along with associated input papers, should be input into AIS 15 for final recommendations.  Input documents include:

- AIS 14 / 7 / 2

- AIS 14 / 9 / 2

- AIS 14 / 7 / Info 2

- The WG suggest a liaison note to IEC-62287 Class B Working Group clarifying the impact of Class A ramp-up and ramp-down characteristics to class B Carrier Sense – TDMA performance.  [This information is necessary to determine whether IEC 61993-2 should be modified].

[IALA requires this information to maintain the Technical Clarifications of ITU-R M. 1371-1, as indicated in recommends 4 of 1371-1]

[Process – letter from IALA to IEC – content of item 2 and 4 as approved by AIS14.  Letter to be reviewed by AIS Chairmen prior to forwarding to IEC.  Include request for response prior to AIS15.

IEC – identify for WG8A and WG14]