IALA ANM 17

Robert Mc Cabe – Rapporteur e-Navigation Shipboard User Experience

Questions posed to Nautical Institute Sea Going Correspondence Group (SGCG).

e-Navigation 
e-Navigation is an IMO initiative to harmonise the collection and presentation of maritime information, to improve navigation safety.  Goals for the project include better integration of navigation information, reduction of redundant ship/shore reporting, alarm management etc.

1	Have you heard of this initiative before?

2	What other major on board navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?

AIS

3	What is your current assessment of AIS reliability?

4	Can you display AIS on your radar and ECDIS?

5	Are there any operational issues of using AIS on radar that need to be addressed.  e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.

__________________________________________________________________________

e-Navigation

1. Have you heard of this initiative before? 
 
YES
 
2. What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?
 
 A) COMPENSATE FOR THE POOR BRIDGE LAYOUTS WITH NAV EQUIPMENT PLACED AD HOC ALL OVER THE BRIDGE 
B) ENABLE A SAFER ONE MAN WATCH AT SEA
C) ENSURE BETTER SITUATIONAL AND SPATIAL AWARENESS IN CROWDED WATERS
D) 

AIS 
3. What is your current assessment of AIS reliability? 

FAIRLY GOOD. BUT CANNOT BE TOTALLY TRUSTED - NEEDS TO BE CROSS CHECKED.
 
4. Can you display AIS on your radar and ECDIS? 
 
YES

5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.
 
ESPECIALLY AT CLOSE RANGE, THERE IS A DIFFERENCE IN THE POSITION OF THE RADAR AND AIS 'TARGET' AIS INFORMATION TO BE SHOWN ON SCREEN AS REQUIRED (THERE SHOULD BE A CHOICE) VECTORS NOT RELIABLE WHEN COURSES BEING CHANGED

e-Navigation

1. Yes, I heard of the initiative before.

2. The major factors I can see improved are, apart from the obvious and already mentioned ones, POSITIONING - RECEPTION OF WARNINGS, NOTICES - SHIP/PILOT AND SHIP/PORT SERVICES.

3. I have a good assessment. Operator misuse seems to be a major factor, when things are not as good as they should be (very often, wrong information displayed). In some cases, electronic disturbances create errors, but I have only a few experiences, too few to worry and obvious, anyway.

4. Yes, I can.

5. I am happy with the actual possibilities. Never had any problems with too many targets, basically because, in restricted waters, the scale of the charts and radar limit that number, anyway.

eNavigation 
eNavigation is an IMO initiative to harmonise the collection and presentation of maritime information, to improve navigation safety. Goals for the project include better integration of navigation information, reduction of redundant ship/shore reporting, alarm management etc.

1. Have you hear of this initiative before?  Yes
2. What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?

Personally I feel that there is more than enough to contend with already. It is clear that the e-navigation project is getting bogged down in its own minutae and the self interests of numerous involved parties - not least equipment manufacturers. The current AIS / Radar / ECDIS integration is proving to be a real mess with often the AIS data overwhelming everything else. Before seeking yet more changes and "advancements" it would be better to get the current project sorted out satisfactorily first.

AIS 
3. What is your current assessment of AIS reliability? Its reliability is a misconception. It is unreliable and often provides erroneous information. 
4. Can you display AIS on your radar and ECDIS? Many ships I work on (as a pilot) can.

5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets. AIS symbols not being coincident with radar targets, multiple radar targets with the same AIS tags, AIS tag swap, the size of the AIS labels on ECDIS overlays. All are software issues related to AIS / Radar / ECDIS integration

eNavigation 
eNavigation is an IMO initiative to harmonise the collection and presentation of maritime information, to improve navigation safety. Goals for the project include better integration of navigation information, reduction of redundant ship/shore reporting, alarm management etc.

1. Have you hear of this initiative before? Yes

2. What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?

- The reduction of times that a shore station calls a ship to identify itself and provide information that is readily available elsewhere - name - call sign - tonnage - number of souls on board etc. This invariably occurs at the worst time with the ship navigating through a port entrance or busy traffic area.

AIS 
3. What is your current assessment of AIS reliability? The reliability of the equipment now appears to be pretty good. The reliability of the users always ensuring the correct information is being transmitted is less so. This appears to occur more frequently, although definitely not exclusively, the case of ships on routes between only a few ports, e.g. ferries, small coasters etc.

4. Can you display AIS on your radar and ECDIS? Yes, on both

5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets. By and large I'm happy with the way the system operates. We regularly transit the Dover Straits and other busy waterways, but don't find a need to reduce the number of targets.

eNavigation 
eNavigation is an IMO initiative to harmonise the collection and presentation of maritime information, to improve navigation safety. Goals for the project include better integration of navigation information, reduction of redundant ship/shore reporting, alarm management etc.

1. Have you hear of this initiative before? Only through being part of the SGCG.

2. What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)? As a marine pilot I am not aware of any ‘major’ onboard navigational issues. I believe that an electronic chart with a RADAR input and overlay of AIS information really is the way ahead for vessel safety in the future. There should be 2 systems on board, each as a backup to the other. The navigator / pilot /  master / OOW can then readily see a real time picture in front of him which can be cross checked with ease. The RADAR echoes of the land should match up with the land masses of the chart and the RADAR echoes of surface contacts should match up with the information from AIS.

Vessels navigating through Traffic Separation Schemes or Fairways can be distinguished from those not following the TSS of fairway by using a combination of the AIS / RADAR and the Electronic chart. All this can be easily achieved with a reduced need for the navigator to have to fix the vessels position by other means at frequent intervals and compare his findings with other systems on board. I work on many ships, most of which have poorly designed layouts. The chart table is usually at the back of the bridge, the RADARs on one side of the bridge and the AIS display by the bridge front windows causing the navigator to move around needlessly in his pursuit of the safety of navigation. If he had all the information in one place in front of him he could concentrate on the job at hand.

AIS 
3. What is your current assessment of AIS reliability? No serious problems have been encountered. There are occasions when a vessel’s MMSI number is displayed on the electronic chart as opposed to its name but these cases are rare. I did have one event where a vessel’s name kept dropping off the screen for no apparent reason but this was only one noteworthy event.
4. Can you display AIS on your radar and ECDIS? Not applicable as the Pilot Station does not have RADAR. We do display the AIS information on an Electronic Chart though which is very effective for traffic monitoring in the port.
5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets. Yes, when approaching the port the tugs come out to meet the ship. If the AIS target information is not de-activated before arrival many alarms are activated during the critical stage of pilotage i.e. approaching the berth, which causes a distraction to the pilot (especially when he is unfamiliar with the layout of the keyboard and can’t find the mute button in the dark!)

eNavigation 
eNavigation is an IMO initiative to harmonise the collection and presentation of maritime information, to improve navigation safety. Goals for the project include better integration of navigation information, reduction of redundant ship/shore reporting, alarm management etc.

1. Have you hear of this initiative before? 
Yes.  Caution should be exercised with regard to making redundant ship/shore reporting points as this form of reporting establishes a positive communication means that identification and communication procedures have been established.

2. What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?
An internationally agreed standard display should be agreed.  Displaying information that is essential navigation information such as Ships heading, course overground, speed overground, water log speed.

 
AIS 
3. What is your current assessment of AIS reliability? 
AIS as target identification is relaible, the positioning of the vessel etc isn't, I have seen quite a large number of errors.  The feeding of other information on targets such as course, speed etc is relaible but there has been situations where the reception of AIS and transmitting of the information has been questionable.

4. Can you display AIS on your radar and ECDIS? 
ECDIS only as I currently work with PPU. 

5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.
The ability for the user to filter information and alarms is exceptionally important for close quarters and confined navigation to keep the ability to view charting information and prevent overload of information onto the screen.

e-Navigation

1.      Have you hear of this initiative before? - YES

2.        What other major onboard navigational issues might be improved

by better integration and presentation of navigational information (please identify up to 4 issues)?

-          Reduce excessive reporting (ISPS, pre-arrival, Notice of

arrival, Port reports, VTIS etc)

-          Reduce over done testing (arrival, departure, equipment, DSC,

radio, AIS, SART)

-          Reduce sending false alarms

-          Reduce mistakes of untrained, negligent,  crews

-          Overcrowding of working channels;  VTS trying to help  in

certain regions, but also mistake if VTS wants to take over control of navigation.

-          Re-inventing new rules daily. Good rule of the thumb:

Improved seamanship with integration of all available information.

-          Help OOW in Collision avoidance decision making process.

-          Advise mandatory training in all these aspects and BTM

refresher course. Putting more equipment on the bridge without trained (and occasionally illiterate) operator is not preferable option.

AIS

3. What is your current assessment of AIS reliability? - very good 4. Can you display AIS on your radar and ECDIS? - yes / very useful 5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e.

switch off) at close range or in confined waters to reduce the number of targets. - Arrival for example: Fujairah or Singapore disable integration AIS - Radar due to excessive amount of ships (sometimes more than 500!) Use AIS alone, if needed to see name of ship on anchor, crossing one, rogue one etc.

Switch off AIS during transiting piracy area, because it can be used for wrong purposes.

Some AIS (JRC particularly) must be made more user friendly. Better lighting in night and day screen nearly useless.

Radar - Its time for revision of frequencies and presentation. We have to start thinking of presentation of real picture of the target and not 'blip-and-vector' on the PPI. Its 21st Century.

e-Navigation 
Just a quick note and I'm not sure if it has any relevance to what you are after.
 
I've recently started Piloting for the Port of London and have found that on the more modern ships they have been fitted with an  ENC ECDIS which means they are not required to carry paper charts onboard.
 
The amount of navigational information displayed and also the many AIS contacts that are around the Thames estuary is immense and I have found it can be difficult discussing a proper passage plan with the master with so much information on the ECDIS screen.
 
It is so much easier going through a passage plan using the traditional method of paper charts especially with such area as the Thames.

_________________________________________________________________________

e-Navigation 
 
1. Have you hear of this initiative before? 

yes i have heard this before from the maritime bulletin, magazine, & other news.

2. What other major onboard navigational issues might be improved by better integration 
and presentation of navigational information (please identify up to 4 issues)?

- e-navigation (sailing without paper chart) only by using ECDIS.
i read about this issue from a bulletin on board.it is said some shipping company had ships sailing without paper charts for over a year and a half.i just wonder why they can do this especially to pass inspections?

- fight against piracy
i hope that there is some program or software installed on board the ship to track all the pirates activity or the hijacked vessel automatically.
 
AIS 
3. What is your current assessment of AIS reliability? 

- AIS is reliable enough to identify the target but it cannot be used for collision avoidance in single mode. it has to be used together with radar, ECDIS, or any other navigation equipment on board. AIS is very usefull to asses the target identity.
AIS does not replace other navigational system. AIS does not be give a complete or correct picture of shipping traffic in its vicinity.

4. Can you display AIS on your radar and ECDIS? 

- yes we can display AIS on both radar and ECDIS.even though some AIS symbol target is not on their correct heading.
And some still forget to update their status (e.g. moored, underway using engine, destination etc.)
  
5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; 
AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.

- we can switch off the AIS display on radar so the AIS symbol will not appear. for ECDIS we can set the maximum AIS symbol until 200.
 _____

eNavigation

1. The concept has been mentioned before and I am aware of it, according to The Pilot magazine editied by John Clandillon-Baker, it can be attributed to:

The term e-Navigation first appeared formally just one year ago in a presentation given by Dr. Sally Basker of the General Lighthouse Authority (GLA) and was rapidly adopted by our own DfT who introduced it to a wider audience via a keynote speech to the Royal Institute of Navigation given by Dr. Stephen Ladyman, the shipping minister.

(cut & paste from The Pilot magazine.)

Beyond this reference, it seems to have taken a somewhat back-seat in the marine world, as far as a I am aware.

2. Taking a holistic view, standardisation is the keyword, here. I pilot so very many ships, each with equipment manufactured in a different way to another, simply finding one’s way around the basic controls can be daunting. From radars, electronic chart displays, VHF-sets and AIS-display equipment. All vary subtly in some way or another, and often the same manufacturer can have an outwardly similar looking piece of equipment, but it can work in a different manner to the same equipment on the ship I piloted the day before! It really is quite bizarre and confusing when you step aboard; matters are different if you have been seconded to a vessel as crew for a number of months, as such differences are learnt and accepted and the mind adjusts for the duration one is seconded for the voyage.

AIS

3. Again from a pilot’s point-of-view, AIS operates and provides important data in a ready manner. Idiosycracies apart, it is largely reliable in my district. Over the past year, only one ship I have piloted (out of a total of perhaps 160 or thereabouts) I have noticed a serious datum-shift of the AIS overlay against radar targets, by some margin of nearly one-mile. I believe this error was specific to that particular ship, as no other failures were recorded in the locality.

On perhaps a handful out of those 160-odd vessels there was complete system malfunction,and no data was available.

A common and noteworthy theme I do experience however is a difference which exists between AIS CPA/TCPA prediction, and Radar CPA/TCPA prediction, on tracked targets. This is so noticable in many cases that I never rely simply on the predictions given by one piece of equipment alone. I do not know the reason for theses error differences.

4. Question not applicable to my role.

5. see my reference to CPA/TCPA in point 3. above. Filtering - Again from a pilotage perspective, when handling tugboats alongside a vessel, continual collision warnings are generated due to proximity of the AIS-equipped tugs. This can be overcome by switching-off the AIS data, but then that also removes the benefit of data from distant ships, which is counter-productive. In order to overcome this problem (and still retain AIS-data from other vessels) I would suggest a close-proximity zone of say 0-300 metres from the centre of own ship, and within which AIS-data from tugs would NOT generate close proximity alarms.

eNavigation 
eNavigation is an IMO initiative to harmonise the collection and presentation of maritime information, to improve navigation safety. Goals for the project include better integration of navigation information, reduction of redundant ship/shore reporting, alarm management etc.

- Have you heard of this initiative before?

Yes I have heard of  e-navigation, initially from an article written in Seaways magazine, in more recent times it has become quite the topic online.

- What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?

- Passage Planning:  In particular automatic updates of Radio Navigation Warnings on ECDIS and early exchange of a pilotage plan prior to arrival at a port.

- Chart corrections:  Automatic correction under e-navigation would remove the time delays associated with chart corrections.

- S Mode:  E-navigation would enable the use of the S mode concept which would much reduce confusion when a new person joins a ship with equipment that they may not be familiar with.

- The automatic display of the position of a DSC distress alert:  A DSC distress alert automatically plotted on the ECDIS of those vessels within the area of the DSC would eradicate time delays associated with plotting on a paper chart and help avoid any confusion.

AIS 
3. What is your current assessment of AIS reliability?

The main source of unreliability is with incorrect display of static information, voyage related data and more worryingly dynamic data including the AIS target being a small distance from the actual ARPA target or incorrect heading being displayed.   I have found AIS AtoN to be reliable, although buoys will be picked up at less distance than normal VHF range.  I find the AIS target really highlights the AtoN.

All OOW’s should check their AIS once a watch to ensure it is displaying the correct data, the internal GNSS should be compared with the ships GNSS, heading input should be checked and the voyage and dynamic data should also be checked.  Companies should be encouraged to include these checks in their SMS.

4. Can you display AIS on your radar and ECDIS?

Yes we have AIS overlay on both our Radar and ECDIS.

5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.

We  had an issue with target association which was down to lack of user knowledge.  When operating normally I leave the AIS display facility on the Radar on as I feel an AIS target even a sleeping one is much more conspicuous than just the ARPA target on its own, it is also a handy  tool for seeing targets around a headland.  However I found that when I plotted the AIS target as well as the ARPA target, the information source displayed on the RADAR kept reverting to AIS data.    There is a button that can be used to toggle between the two sets of data, even after selecting RADAR data  I found that the information source tended to default after a few seconds back to AIS data.  In a close quarters situation all I am interested in is the ARPA data. I ended up switching off the AIS data, so that I was sure that the data I was viewing was ARPA only.  One officer left the AIS data switched off on the S band and activated on the X band.  It turns out we had the target association settings based on AIS data rather than RADAR data, this is something which all OOWs need to be aware of as it can easily be confused particularly when under pressure and operating in areas of heavy traffic.

OOWs need to be aware that AIS information when used for collision avoidance is based on the other ships onboard GNSS equipment and will display ground stabilised information showing course and speed over the ground.  The OOW cannot be sure that the other ships GNSS equipment is sited correctly and transmitting the correct position, or that the other ships Gyro feed to their AIS unit is correct.  OOWs also need to be aware that AIS relies on GNSS for timing and that the signal can be jammed.  Collision avoidance decisions should not be based solely on AIS data!

e-Navigation

1. Have you hear of this initiative before?

YES

2. What other major onboard navigational issues might be improved by better integration and presentation of navigational information (please identify up to 4 issues)?

- Automatic presentation of NAVREA & NAVTEX info on ECDIS

- Automatic presentation of tidal data on ECDIS

- Presentation of ship’s maneuvering characteristic: radius of turn, rate of turn, wheel over point & predicted path on ECDIS. Normally, ECDIS “track mode” function takes in account all data; same ECDIS have predicted path and instant calculation on radius & ROT. But, it is not always easy to extract / see / use such data as reference for maneuvering / steering in traditional “hand steering” mode.

AIS

3. What is your current assessment of AIS reliability?

Generally good. Most common errors between data received from other vessels are:

1) Heading (gyro) information is not correct – huge discrepancy from heading and COG is very common (this is technical error as well as lack of checking “own ship data”).

2) Status of the vessel: some cargo vessels instead to display status “under way using engine” are “underway sailing” – this has relay nothing with technical capacities of AIS but with poor standard of officers on board

4. Can you display AIS on your radar and ECDIS?

YES, both on radars and ECDIS. Also, there is stand alone AIS unit where is possible to work out all AIS functions independently from radars & ECDIS.  – Mentioning this because there are some vessels where is possible to work / control AIS only on radars and this is not always so convenient.

5. Are there any operational issues of using AIS on radar that need to be addressed. e.g. AIS/radar target association; AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.

AIS/radar target association;

On radars, I have on board radar and AIS plot are shown separately with different symbols. COG & SOG data are in generally more precise, steady & reliable from AIS then from radar plot. But, dependence on GPS as base for AIS data asks for caution.

AIS filtering (i.e. switch off) at close range or in confined waters to reduce the number of targets.

On radars, I have on board it is possible to make selection of AIS targets by sector & range. This is very convenient in confined waters to reduce the number of targets. Otherwise, AIS comes to 100% capacity & become overcrowded and too slow.