AIS Availability

# Introduction

This paper has been submitted by the General Lighthouse Authorities (GLA) of the UK & Ireland and is a summary of the results of a study carried out for the GLA by Helios Technology. The study was a Failure Mode and Effect Analysis (FMEA) and Fault Tree Analysis (FTA) of the GLAs’ AIS Aid to Navigation (AtoN) service.

The aim of the study was to provide initial input from the GLAs to IALA to assist discussions on:

- the definition of a suitable service availability requirement for the AIS AtoN service; and,

- recommendations for the design of an AIS AtoN system to meet the defined service availability requirements.

- The Committee is invited to consider the results and use them as appropriate.

AIS AtoN service availability definition

The proposed AIS AtoN service availability definition is as follows:

The AIS AtoN service shall be available for ZZ% of the intended transmissions.

Where the proposed AIS AtoN service definition is:

The transmission of the following correct information (message 21) relating to all AtoNs identified as AIS AtoNs:

- The type of AtoN.

- The name of the AtoN.

- A valid 2D position of the AtoN to within the accuracy as indicated by the position accuracy indicator.

- A position accuracy indicator.

- Type of position fixing device.

- Off position indicator.

- Time stamp.

- Dimensions of the AtoN and reference positions.

- Virtual AtoN flag.

- RAIM flag.

The AIS AtoN transmissions containing the AtoN information shall have a signal level of greater than or equal to -70dBm when measured at the air-antenna interface of the user’s receiver within the following coverage areas:

5-10NM of the AIS AtoN for floating AtoN depending on height of AtoN.

10-25 NM of the AIS AtoN for fixed AtoNs depending of height of AtoN.

FMEA

The AIS AtoN FMEA has identified different failure modes and the effect they have on the AIS AtoN service, as provided by the GLAs and the MCA. In particular these can be split into two types of failure effects:

- Failures that directly affect the provision of the AIS AtoN service.

- These failures are largely associated with the AIS AtoN, AIS base station and the VHF communication functions.

- Failures that do not directly affect the provision of the AIS AtoN service but which affect the ability to detect a failure of the AIS AtoN service. These failures therefore affect the time to detect a failure of the AIS AtoN service and therefore the overall availability of the service.

- These failures are largely associated with the control and monitoring functions and the communications functions.

FTA

The FTA was carried out to calculate the probability of an AIS AtoN service unavailability event occurring, based on AIS AtoN system component reliability data. The FTA calculated that on average we would expect 1.4 service unavailability events per year for an AIS AtoN system comprising of 2 MCA base stations, 1 GLA base station (acting as an AtoN) and a single mobile AIS AtoN (using either fixed access or random access).  As a rough approximation this can be interpreted as the rate of service failure events normalised to number of failure events per AIS AtoN per year. (Note that this approximation will be an over estimation of the failure rate and includes failure modes from all parts of the AIS AtoN system.)

It was concluded that a monitoring system is required to quickly detect failures of the AIS AtoN service in order to ensure a high overall availability of the AIS AtoN service with a failure rate of ~1.4 failure events per year per AIS AtoN. Therefore the availabilities of the MCA and GLA control and monitoring functions are very important in ensuring the overall availability of the AIS AtoN service.

Calculation of theoretical AIS AtoN service availability

It should be noted that while the FTA has focused on the probability of an AIS AtoN service unavailability event it has not calculated the availability (which includes detection and repair times) of the AIS AtoN service. In order to calculate AIS AtoN service availability the following information is required:

- Probability of service unavailability event occurring for the entire AIS AtoN system (i.e. including all constituent base stations and AIS AtoNs).

- Probability of failure of the AIS AtoN control and monitoring function (required to calculate mean time to detection of an AIS AtoN failure).

- Calculation of time to detect and repair a failure of the AIS AtoN service or control and monitoring function.

- Calculation of the effect of interference from other transmissions as a result of the correct operation of the AIS channel access protocols.

- Calculation of GPS signal availability for providing position information in AIS AtoN message 21.

Conclusion

The proposed definition of AIS AtoN service availability is proposed as a basis for discussion, but the calculated failure rate would be unacceptably high and the assumptions used require validation.

Note that the service availability shall be calculated separately for each AIS AtoN.