REPORT

on

the 53rd session of the IMO Sub-Committee

on Safety of Navigation (NAV)

Contents

# 1.	Introduction

2.	e-Navigation

# 3.	Ships’ routeing and related matters

3.1.	AIS information and mandatory ship reporting system

## 3.2.	Traffic Separation Schemes (TSS) and other routeing measures

3.3.	Mandatory ship reporting system

4.	Electronic Chart Display and Information System (ECDIS)

5.	Shipborne Automatic Identification System matters

6.	Guidelines on the control of ships in a emergency

# 7.	Date of the next session.

## 8.	Recommended actions for IALA

## 8.1.	the Council

8.2.		the e-Navigation Committee

8.3.     the VTS Committee

8.4.		the Engineering, Environmental and Preservation Committee

8.5.		the Aids to Navigation Management Committee

### A N N E X

Sub-Committee on Safety of Navigation (NAV) – 54th session - Agenda proposed to the Maritime Safety Committee

*  *  *

1.	Introduction

Due to the refurbishment of the headquarters of the International Maritime Organization, the 53rd session of the IMO Sub-Committee on Safety of Navigation was held at the Royal Horticultural Halls and Conference Centre in London during the period 23 to 27 July, 2007. It was attended by representatives from 69 Member States and 29 Observer Organizations. The Secretary General, Nick Ward and Jean-Charles Leclair were representing the Association.

The most important item discussed during the week was the concept of e-navigation, its definition and the way to adopt an overall strategy to cover all of its aspects. IALA was very involved in the discussion and, from the beginning, has taken an active role in the development of the IMO work on this subject.  The Sub-Committee agreed on the definition of e-navigation as proposed by IALA and on the core objectives of the concept. It also decided that the e-navigation strategy should be user driven, rather than technology driven, and that it was first necessary to identify and define the user requirements before considering any technology standards

During this session, the Sub-Committee also adopted 26 different new or amended routeing measures and mandatory ship reporting systems. It agreed on a draft MSC Circular on Guidelines on the control of ships in an emergency, but it did not yet succeed to reach a consensus on mandatory carriage requirement of ECDIS for cargo ships and decided to continue the discussion on the matter during its next session.

2.	e-Navigation

In his speech, at the opening of the session, the Secretary General of IMO recalled the delegates that they should keep in mind that e-navigation “ought not be seen as an end in itself or as a panacea and also that, for the foreseeable future, the human element, with all its frailties, will remain the key component in any integrated and coordinated e-navigation concept”. He further added that “the basic technologies for an e-navigation system are available and the challenges lie, on the one hand, in ensuring the availability of all the other components (including electronic navigational charts) and, on the other, in combining them in a holistic and systematic manner, so that they can be used effectively to simplify the display of all pertinent navigational information, thus enabling the mariner to be aware, in real-time, of the environment in which his or her ship is navigating”.

He told the Sub-Committee that “such matters were discussed during a recent IALA Seminar on e-navigation, coinciding with the Association’s 50th anniversary, which he had the honour to address and which gave him the opportunity to express appreciation for IALA’s excellent work in putting in place a global network of aids to navigation; its central role in the development of new technologies; and its contribution to IMO’s regulatory work – in particular, that of the Sub-Committee and, more recently, the e-navigation item”.

At its 81st session (May 2006), the Maritime Safety Committee had instructed the NAV Sub-Committee, together with COMSAR, to develop an e-navigation strategy with a target completion date of 2008. As a first step, NAV 52 established a correspondence group, under the co-ordination of the United-Kingdom, to develop a clear definition and objectives for the concept of e-navigation. IALA and IHO were particularly invited to participate to the work of IMO on the matter and to provide relevant inputs.

In addition to its active participation in the work of the correspondence group, IALA submitted three different documents to this session of the NAV Sub-Committee, on these particular subjects:

- the work of IALA on e-navigation,

- position fixing on e-navigation, and

- display of AIS as an aid to navigation.

In addition, an oral report was made regarding the outcomes of the IALA seminar on e-navigation hold at Trinity House three weeks before the NAV meeting.

After preliminary discussions based on the report of the correspondence group and on different documents submitted by Japan, United Kingdom, ICS and IALA, the Sub-Committee agreed that the e-navigation strategy should be user driven, rather than technology driven, and was of the view that it was first necessary to identify and define the user requirements before considering any technology standards. Then, the Sub-Committee established a new Working Group on e-navigation under the chairmanship of M. Mike Sollosi.

First, the Working Group approved the definition of e-navigation as proposed by IALA with one only addition of the word “analysis” to read:

“E-Navigation is the harmonized collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment.”

It also agreed that the core objectives of an e-navigation concept should:

- facilitate safe and secure navigation of vessels having regard to hydrographic, meteorological and navigational information and risks;

- facilitate vessel traffic observation and management from shore/coastal facilities, where appropriate;

- facilitate communications, including data exchange, among ship to ship, ship to shore, shore to ship, shore to shore and other users;

- provide opportunities for improving the efficiency of transport and logistics;

- support the effective operation of contingency response, and search and rescue services;

- demonstrate defined levels of accuracy, integrity and continuity appropriate to a safety-critical system

- integrate and present information onboard and ashore through a human interface which minimizes navigational safety benefits and minimizes any risks of confusion or misinterpretation on the part of the user;

- integrate and present information onboard and ashore to manage the workload of the users, while also motivating and engaging the user and supporting decision-making;

- incorporate training and familiarization requirements for the users throughout the development and implementation process;

- facilitate global coverage, consistent standards and arrangements, and mutual compatibility and interoperability of equipment, systems, symbology and operational procedures, so as to avoid potential conflicts between users; and

- be scalable, to facilitate use by all potential maritime users.

Regarding the different system architectures developed by the correspondence group, the Working Group agreed that it was premature to agree on any one of the systems proposed before finalizing the users’ requirements. On this particular point, the Working Group was advised that the United Kingdom, IALA and IFSMA were working on developing a methodology to identify users and their needs and would be providing the appropriate input to the correspondence group. The Group also took note of the project undertaken by the Nautical Institute on “S-mode”, aimed at developing a standard presentation of information using a standard menu system for shipboard units.

The Working Group further agreed on the three key components of e-navigation and their contents as developed by the correspondence group but, at this stage, considered them more as broad expectations rather than firm objectives. The three outcomes agreed by the correspondence group focused on the onboard, shore and communications elements of e-navigation and were described as follow.

- Onboard: navigation systems that benefit from the integration of own ship sensors, supporting information, a standard user interface, and a comprehensive system for managing guard zones and alerts. Core elements of such system will include high integrity electronic positioning, electronic navigational charts (ENCs) and system functionality with analysis reducing human error, actively engaging the mariner in the process of navigation while preventing distraction and overburdening.

- Ashore: the management of vessel traffic and related services from ashore enhanced through better provision, co-ordination, and exchange of comprehensive data in formats that will be more easily understood and utilized by shore based operators in support of vessel safety and efficiency.

- Communications: an infrastructure providing authorized seamless information transfer onboard ship, between ships, between ship and shore and between shore authorities and other parties with many related benefits, including a reduction of single person error.

On the question of the redundancy of position fixing systems, the Working Group agreed with IALA on the need to provide an internationally agreed alternative system for complementing the existing satellite navigation, positioning and timing services to support e-navigation and recognized that potential back-up systems could be made available and that it was premature to identify any specific system before the users’ requirements for e-navigation had been finalized. The Group also agreed that e-navigation should not be viewed as a means to reduce or eliminate existing AtoN and that any decision to employ e-navigation as a means to replace traditional AtoN should only be considered once a full risk assessment had been carried out and the users’ requirements had been finalized.

The report of the Working Group and its proposals were all endorsed by the Sub-Committee which decided to re-establish the intersessional correspondence group with the following new terms of reference:

- identify all potential users of e-navigation;

- define the user needs for e-navigation;

- review the need to consult other maritime agencies and interest groups (navigational practitioners, support agencies, research organizations, equipment manufactures and port managers); and

- continue to develop other aspects of the strategic vision for e-navigation.

In order to structure the task of developing a strategic vision for e-navigation using a holistic and top-down approach it is essential to provide a methodology and logical phases to define the essential elements of e-navigation. In this context, the correspondence group should develop a strategic vision taking into account the logical phases relating to:

- user identification

- user requirements

- user services

- identify existing systems

- system requirements

- gap analysis

- role of cost benefit analysis, and

- system architecture.

This list should not be considered as a comprehensive list of logical phases and some of the work can be undertaken simultaneously.

The correspondence group should submit a document to COMSAR 12 (6 – 11 avril 2008) raising specific questions that should be addressed by COMSAR and prepare a final comprehensive report for submission to NAV 54 (29 June – 4 July 2008).

The way the terms of reference are now drafted gives a clear priority on the work to achieve on users list and users requirements, as IALA and its e-navigation Committee have already started to detail. In the same time, it also allows the correspondence group to deal with other developments of different phases as long as the results would be evaluated with the results of preceding phases.

3.	Ships’ routeing and related matters

3.1	AIS information and mandatory ship reporting system

During the discussions on ship routeing and ship reporting system, the Sub-Committee noted that, in specific circumstances, when using ship reporting systems provided for by SOLAS regulation V/11, there might be value in verbal contact with the mariner when a ship was entering or departing from a reporting area. AIS, although an important tool, was not always considered as an appropriate substitute for voice to voice communications between a ship’s bridge and a shore-based authority. Accordingly, the Sub-Committee requested all Member Governments to reconsider and revise as necessary mandatory ship reporting systems so as to avoid duplication of information and reduce the items in the reporting format to those which are not available through AIS and other sources. The Member States were invited to consider submitting proposals to the development of an SN circular.

3.2	Traffic Separation Schemes (TSS) and other routeing measures

The Sub-Committee approved and invited the Maritime Safety Committee to adopt:

- New Traffic Separation Scheme “Maas North-West” forming part of the approaches to Hook of Holland and at North Hinder

- New Traffic Separation Schemes in the Galapagos Area

- New Traffic Separation Scheme on the approaches to the Polish ports in the Gulf of Gdansk

- New Traffic Separation Scheme and attached two-way routes Off the southwest coast of Iceland

- Amendments to the “Mandatory route for tankers from North Hinder to the German Bight and vice versa” and to related traffic separation schemes “Off Texel”, “Off Vlieland, Vlieland North and Vlieland Junction”, “Terschelling-German Bight” and “German Bight western approaches”

- Amendments to the existing Traffic Separation Scheme “In the approaches to Hook of Holland and at North Hinder”

- Amendments to the existing Traffic Separation Scheme “In the Sound”

- Amendments to the existing Traffic Separation Scheme “In the Approaches to Chedabucto Bay”

- Amendments to Existing Traffic Separation Scheme “In the Strait of Dover and Adjacent Waters”

- Establishment of an Area to be Avoided “Off the Brazilian South-East coast, in the Campos Basin region”

- Amendment and expansion of the six existing Areas to be Avoided “in the Region of the North-West Hawaiian Islands”

- Amendment to the Deep-water route leading to Europort

- Amendment to the Area to be Avoided “at Maas centre” and “At North Hinder junction Point”

- Recommendations on navigation to the Polish ports through the Gulf of Gdansk traffic area

- Establishment of new two-way route  Off the southwest coast of Iceland

- Establishment of Areas to be Avoided Off the South and Southwest coast of Iceland

- Amendments to the Recommendation on navigation through the entrances to the Baltic Sea

- Establishment of new mandatory No Anchoring Areas on Sharks Bank and Long Shoal

- Establishment of a seasonal Area to be Avoided “In Roseway Basin, South of Nova Scotia”

- Amendments to the northerly and southerly limits of the Sandettie Deep-Water route and an amendment to the position of the Foxtrot 3 station

- Amendments to the Recommendations on Navigation through the English Channel and the Dover Strait

- Amendments to the Deep-Water Route “North-East of Gedser”

3.3.	Mandatory ship reporting system

The Sub-Committee approved and invited the Committee to adopt:

- New recommendatory/mandatory ship reporting system for the Papahanaumokuakea Marine National Monument

- New mandatory ship reporting system On the approaches to the Polish ports in the Gulf of Gdansk

- New mandatory ship reporting system Off the South and Southwest coast of Iceland

- Amendments to the existing mandatory ship reporting system “Off Ushant”, “Off les Casquets” and “Dover Strait/Pas de Calais”

4.	Electronic Chart Display and Information System (ECDIS)

At its 51st session, the Sub-Committee started to discuss the question of mandatory carriage of ECDIS on ships other than passenger vessels and high speed crafts, for which it is already compulsory. At its 52nd session, two FSA studies were produced by Denmark together with Norway and by Japan. Both concluded that the mandatory carriage requirement for ECDIS to cargo ships was justified as being cost-effective if the ships sailed in routes or sea areas where suitable scaled ENCs are available. However, the majority of the delegations had been of the view that ENC coverage was a necessary pre-requisite for the introduction of a mandatory carriage requirement of ECDIS, even if it does not mean a 100% ENC coverage. Therefore, the Sub-Committee decided not to recommend the mandatory carriage requirements at this stage.

At that session also, the Sub-Committee approved a draft MSC resolution on Adoption of the revised ECDIS performance standards and forwarded them to the Maritime Safety Committee which adopted it in December 2006 (Resolution MSC 232(82)). When adopting the Resolution, the MSC instructed the NAV Sub-Committee to assess whether a common layout of controls, names or symbols for controls and output on the display for each control could be appropriately included in the performance standards. This would ensure that officers transferring/serving on any particular ship were fully conversant with the basic operating procedures for ECDIS equipment. During this 53rd session, the Sub-Committee noted that IMO had already developed standards for common names and common output on the display in resolution MSC.191(79) and SN/Circ.243. The Sub-Committee also noted that the IEC had developed standards for symbols for controls. Moreover, future work connected with e-navigation was considering a common S-mode configuration for bridge equipment. Therefore, the Sub-Committee concluded that it was premature to again revise the ECDIS performance standards at this stage, but to await the outcome of these developments.

During this session also, the Sub-Committee received new information from IHO on the availability of ENCs and its aim at achieving adequate coverage, availability, consistency and quality of ENCs by 2010. However, many delegations pointed out the difficulties to achieve this objective and the problems of no adequate global ENC coverage around the coast of some developing countries and small islands States, and on the human element and training aspects and related issues. Those raising these concerns were of the opinion that a decision on a carriage requirement for ECDIS would therefore be premature at this stage, and called for postponing a decision thereon until these questions had been answered and existing problems including global ENC coverage issues had been solved.

The participants were equally divided on the matter and they finally agreed to propose to the Maritime Safety Committee to maintain the item on the Sub-Committee’s agenda for its 54th session for further discussions. In the meantime, the Chairman invited Members and Observers to provide further information on the matter, including updates on ENC coverage.

5.	Shipborne Automatic Identification System matters

The Sub-Committee noted the information provided by the IMO Secretariat on the revised version of Recommendation ITU-R M.1371-2 adopted by ITU-R Study Group 8 concerning the technical characteristics for AIS using time division multiple access in the VHF maritime mobile band.

The draft revision harmonizes the AIS publications of ITU, IALA and IEC and:

.1 	incorporates IALA’s Technical Clarifications to Recommendation ITU-R M.1371-1Edition 1.5 into the Recommendation;

.2 	changes the name of the Recommendation by removing the words “universal

Shipborne” from the title;

.3 	changes the binary messages in Annex 5 to resolve the conflict between

Recommendation ITU-R M.1371-2 and IMO document SN/Circ.236;

.4 	adds a new Annex 8 “AIS Messages” that removes the AIS messages from

Annexes 2 and 7 and consolidates them in one annex;

.5 	adds a new Annex 9 containing the list of abbreviations used in the Recommendation;

.6 	removes DSC polling from Annex 3;

.7 	changes recommends 1 to refer to the correct annexes;

.8 	changes recommends 3 to refer to IMO instead of IALA; and

.9 	includes editorial corrections to the document.

The Sub-Committee also noted with interest the information provided by IALA as the result of an IALA survey conducted in 2006 on AIS errors seen in VTS centres. The Sub-Committee took note that the greatest obstacle to attaining improved standards of accuracy in the transmission of AIS data was the continuing existence of the Minimum Keyboard Display (MKD). Feedback had clearly indicated that the true value of AIS was only apparent when presented on a fully integrated graphical display.

Germany and Sweden provided information describing the technical limitations for the use of AIS binary messages and presented the results of a study of the existing usage of the AIS VHF data link including the need for further work to develop guidelines for the use of AIS binary messages.

6.	Guidelines on the control of ships in a emergency

Following the “Prestige” accident, the Maritime Safety Committee requested the NAV and COMSAR Sub-Committees to develop guidelines covering the responsibilities of all parties involved in a maritime emergency, which would not create a chain of command but, if implemented by Member States as part of their emergency action plans, would clarify what the chain should be.

The Sub-Committee completed the work at this session and agreed to the draft MSC Circular on Guidelines on the control of ships in an emergency for submission to MSC 83 for approval. The guidelines would not change the responsibilities of the master, but they might avoid misunderstandings as to what a master’s role should be when coastal State laws would be enforced and what their effect would be on the master and others involved in an emergency.

7.	Date of the next session.

The 54th  session of the NAV Sub-Committee is tentatively scheduled to be held from 30 June to 4 July 2008.

The IMO Secretariat should receive documents intended for consideration by the Sub-Committee approximately as follows:

(a)	those containing more than 6 pages of  text (bulky documents) and those which 	require	 action or decision by the 30th of March 2008;

(b)	other documents by the 27th of April 2008.

The proposed agenda for the session is attached as annex.

8.	Recommended actions for IALA

It is recommended that:

8.1.	the Council

- note the discussion regarding e-navigation (item 2), including the tribute paid by the Secretary General to IALA’s role

- note the position of the Sub-Committee on the carriage of ECDIS (item 4)

- note the adoption of a draft MSC Circular on	Guidelines on the control of ships in a emergency (item 6)

8.2.		the e-Navigation Committee

- note the discussion regarding e-navigation (item 2)

- note the discussion on AIS information and mandatory ship reporting system (item 3.1)

- note the position of the Sub-Committee on the carriage of ECDIS (item 4)

- note the information provided on AIS (item 5)

- note the new agenda items 11 and 18 (Annex)

8.3.    the VTS Committee

- note the discussion regarding e-navigation (item 2)

- note the discussion on AIS information and mandatory ship reporting system (item 3.1)

- note the position of the Sub-Committee on the carriage of ECDIS (item 4)

- note the adoption of a draft MSC Circular on	Guidelines on the control of ships in a emergency (item 6)

- note the new agenda items 11 and 18 (Annex)

8.4.		the Aids to Navigation Management Committee

- note the discussion regarding e-navigation (item 2)

- note the discussion on AIS information and mandatory ship reporting system (item 3.1)

- note the position of the Sub-Committee on the carriage of ECDIS (item 4)

- note the new agenda items 11 and 18 (Annex)

* * *

J.Ch. Leclair

Accredited Representative of IALA to IMO,

10 August 2007.

### A N N E X

Sub-Committee on Safety of Navigation (NAV) – 54th session - Agenda proposed to the Maritime Safety Committee

Opening of the session

1	Adoption of the agenda

2	Decisions of other IMO bodies

3 	Routeing of ships, ship reporting and related matters

4	Revision of the performance standards for IBS

5	Amendments to the General Provisions on Ships’ Routeing

6	Carriage requirements for a bridge navigational watch alarm system

7	Review of COLREGs regarding the right of way of vessels over pleasure craft

8	Amendments to COLREGs Annex I related to colour specification of lights

9	ITU matters, including Radiocommunication ITU-R Study Group 8 matters

10	Code of conduct during demonstrations/campaigns against ships on high seas

11	Measures to minimize incorrect data transmissions by AIS equipment

12	World-wide radionavigation system (WWRNS)

13	Development of an e-navigation strategy

14 	Development of carriage requirements for ECDIS

15	Guidelines for uniform operating limitations of high-speed craft

16	Guidelines on the lay-out and ergonomic design of safety centers on passenger ships

17 	Review of vague expressions in SOLAS regulation V/22

18	Revision of the Guidance on the application of AIS binary messages

19	Improved safety of pilot transfer arrangements

20	Casualty analysis

21	Consideration of IACS unified interpretation

22	Work program and agenda for NAV 55

23	Election of Chairman and Vice-Chairman for 2009

24	Any other business

25	Report to the Maritime Safety Committee

__________