AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation A-124

on

the AIS Service

[Working towards Edition 2]

[Future]

Edition 1.0 issued December 2002

Edition 1.1 issued December 2004

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Recommendation on the AIS Service

(Recommendation A-124)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that IMO has concluded that AIS will improve the safety of navigation and the protection of the environment;

RECOGNISING FURTHER that the Association has recommended that National Members

- providing aids to navigation services, including VTS, consider the introduction of AIS facilities in their shore infrastructure (IALA Recommendation A-123);

- using AIS in marine aids to navigation consider IALA Recommendation A-126;

NOTING that National Members intending to introduce AIS facilities in their shore infrastructure need information on the operational, technical and life cycle management aspects of such facilities;

NOTING ALSO that IALA has contributed substantially to the development of AIS through collaboration with IMO, ITU and IEC;

CONSIDERING that there is an urgent need for shore based AIS infrastructure to facilitate the interchange of data with shipping;

CONSIDERING ALSO that

- IMO recently has defined the e-Navigation concept;

- the shore based AIS infrastructure of National Members have an important role in that concept;

CONSIDERING FURTHER the Association has recommended that National Members consider

- the e-Navigation architecture – the shore perspective (IALA recommendation eNAV 101);

- the common shore based e-Navigation system architecture (IALA recommendation [eNAV 201]);

- the e-Navigation generic engineering service model template (IALA recommendation [eNAV 210]);

- the shore based AIS infrastructure of National Members should be understood as an AIS Service within the framework of the e-Navigation architecture;

RECOMMENDS that National Members introducing an AIS Service into their shore infrastructure take into account the annex to this recommendation.

IALA Recommendation on the AIS Service

Table of Contents

Main Part

1	AIS Service as described by other IALA Recommendations 6

2	Service Model of the AIS Service 6

2.1	Parts to the description of the one AIS Service - Overview and Annexes 6

2.2	Capabilities of the AIS Service for the shore-based e-Nav system 8

2.3	Data model of the AIS Service 10

2.4	Structure model - the Layered Structure of the AIS Service 11

2.5	Distribution model for the AIS Service 13

2.6	Interaction and data storage model of the AIS Service 15

2.7	Interfacing model of the AIS Service 21

2.8	Internal time latency model of the AIS Service 21

2.9	Internal reliability model of the AIS Service 22

2.10	Test Model for the AIS Service 22

2.11	Functional Components of the AIS Service 22

2.12	Implementation, Installation and Maintenance 32

2.13	VDL Usage by AIS Service and VDL Management 32

3	References 32

Annexes

Deliverables of the AIS Service to the shore-based e-Navigation system:

Annex 1 – 	AIS Service Generic Use Cases (Basic AIS services)

Annex 2 – 	Data types and data objects of the AIS Service (data model)

Annexes related to AIS Service model:

Annex 3 – 	Distribution model for the AIS Service

Annex 4 – 	Interaction and data storage model of the AIS Service

Annex 5 – 	Interfacing model of the AIS Service

Annex 6 – 	Internal Time Latency model of the AIS Service

Annex 7 – 	Internal Reliability model of the AIS Service

Annex 8 – 	Test model for the AIS Service

Annexes related to functional components of AIS Service

Annex 9 – 	Functional description of the Logical AIS Shore Station

Annex 10 – 	Functional description of the Physical AIS Shore Station

Annex 11 – 	Functional description of the AIS Service Management

Annexes related to installation and life-cycle management issues

Annex 12 – 	Remote Shore Stations (RSS) considerations

Annex 13 – 	Recommendation regarding efficient operation and maintenance of the AIS Service

Annexes related to the VDL usage and the VDL management by AIS Service

Annex 14 – 	FATDMA planning and operation of an AIS Service

Annex 15 – 	Assigned mode operation of the AIS Service

Annex 16 – 	DGNSS broadcast via the AIS Service

Annex 17 –  	Channel management by an AIS Service

Annex 18 – 	General loading management by an AIS Service

# AIS Service as described by other IALA Recommendations

IALA Recommendation on “The Provision Of Shore Based Automatic Identification Systems (AIS)” (A-123; Edition 2007) encourages administrations to provide “an AIS shore infrastructure so that the full benefit of the system can be realised in terms of navigation safety and protection of the environment” (§1).

The same Recommendation identifies the AIS as a maritime, safety-related information service as follows (§4):

“From a VTS or, more generally speaking, from the point of view of a competent authority the AIS provides an information service for shore-based VTS, traffic management schemes, ship reporting systems and other shore-based safety-related services. This service consists of information delivery between ships and shore and vice versa. Thus the service of information exchange between ships and maritime, safety-related shore services, such as VTS, is one important part for the AIS. (...) Consequently, approaching the AIS from any shore-based application’s point of view, there will be an AIS Service delivered at a functional interface.”

The same Recommendation eventually identifies the place of the AIS Service within a hierarchy of other shore-based services (§5).

IALA Recommendation on “The Use of the Automatic Identification System (AIS) in Marine Aids to Navigation” (A-126, Edition 2007), while not expressively requiring a shore-based AIS Service for use of AIS in marine Aids to Navigation, implies the existance of a shore-based AIS Service for some of the applications described therein.

# Service Model of the AIS Service

## Parts to the description of the one AIS Service - Overview and Annexes

IALA Recommendation on Generic e-Navigation Service Engineering Model Template (eNAV-210, Edition [2008]) describes the generic engineering model for e-Nav services. The AIS Service is designed, operated and maintained as described in the above Recommendation. This chapter applies the generic engineering model template to the AIS Service (additional aspects are applied in following chapters and/or in appropriate Annexes). Therefore, the AIS Service is described by of the following parts of the generic e-Navigation service model, which complement each other (refer to Figure 1 on the following).

Figure 1: AIS Service model

The following sections only provide as much information as required for an overview or just state, what the particular model deals with in terms of the AIS Service. Additional information will be given in the Annexes:

- Annexes related to abstract modelling of the AIS Service:

  - Basic AIS Services: functional interface statements employing, amongst others, use case methodology: Annex 1

  - Data model: introduces abstract descriptions of the data provided by the AIS Service at its Basic AIS Services to the shore-based e-Navigation system, i.e. defines data types and data objects of the AIS Service: Annex 2

  - Distribution model: deals with issues arising from geographical distributed AIS Service layouts and with coverage planning as one major factor: Annex 3

  - Interaction and data storage model: introduces the dynamics of the interaction of individual components of the AIS Service (data flow dynamics, such as push or pull concepts) and deals with associated data storage issues in an abstract fashion: Annex 4

  - Interfacing model: Deals with data encoding for interfacing and with interface protocols proper between components of the AIS Service: Annex 5

  - Internal time latency model: Introduces absolute (real) time considerations, even quantitatively: Annex 6

  - Internal Reliability model: Provides an analytical framework for reliability considerations of the AIS Service: Annex 7

  - Test model: Provides a test strategy on service level: Annex 8

- Annexes related to functional components of AIS Service: This chapter briefly introduces the functional components of the AIS Service.

  - Functional description of the Logical AIS Shore Station (LSS): Annex 9

  - Functional description of the Physical AIS Shore Station (PSS): Annex 10

  - Functional description of the AIS Service Management (ASM): Annex 11

- Annexes related to installation and life-cycle management issues

  - Remote Shore Stations (RSS) Considerations: Annex 12

  - Recommendation regarding efficient operation and maintenance of the AIS Service: Annex 13

- Annexes related to the VDL usage and the VDL management by AIS Service

  - FATDMA planning and operation of an AIS Service: Annex 14

  - Assigned mode operation of the AIS Service: Annex 15

  - DGNSS broadcast via the AIS Service: Annex 16

  - Channel management by an AIS Service: Annex 17

  - General loading management by an AIS Service: Annex 18

## Capabilities of the AIS Service for the shore-based e-Nav system

### List of external Basic AIS Services (BAS)

The AIS Service is part of the shore-based e-Nav system. Hence it would be of primary interest to any other shore-based e-Nav service, what capabilities the AIS Service contributes to the shore-based e-Nav system. These capabilities can be defined as external Basic AIS Services (BAS). They all have in common, that they are justified by at least one interaction with a “requesting service” within the shore-based e-Nav system: The “requesting service” is any other service of the shore-based e-NAV system putting a request to the AIS Service.

The complete list of external Basic AIS Services and their categories is given below:

- External Basic AIS Service Category I:

- A_DYN: Receive dynamic data from shipboard sensors via Class A mobile AIS stations

- A_STAT: receive static data via Class A mobile AIS stations, which are eventually stemming from human interaction with the shipboard Class A mobile station and/or from shipboard mobile applications

- A_VOY: receive voyage related data via Class A mobile AIS stations, which are eventually stemming from human interaction with the shipboard Class A mobile station and/or from shipboard mobile applications

- B_DAT: receive data from Class B mobile AIS stations and possibly mobile applications

- SAR_DAT: receive data from SAR aircraft mobile AIS stations

- SART_DAT: receive data from AIS-SART mobile AIS stations

- ATON_DAT (receive): receive data from AtoN mobile AIS stations

- SAFE_AD / SAFE_BR (receive): receive safety related data from mobile applications via mobile AIS stations to application ashore

- TRANS_IAI / TRANS_RAI (receive): receive application specific data from mobile applications via mobile AIS stations to application ashore

- DGNSS_COR (receive): receive DGNSS corrections transmitted via adjacent AIS Services, other than own AIS Service

- External Basic AIS Service Category II:

- SAFE_AD / SAFE_BR (transmit): deliver safety related data from application ashore to mobile applications via mobile AIS stations

- TRANS_IAI / TRANS_RAI (transmit): deliver application specific data from application ashore to mobile applications via mobile AIS stations

- DGNSS_COR (transmit): broadcast DGNSS corrections via own AIS Service as received from own shore-based Differential Correction Service

- External Basic AIS Service Category III:

- ASGN_RATE/ASGN_SLOT: “soft” and “hard” assignment to mobiles by trigger event from requesting Service

- ATON_DAT (transmit): deliver synthetic and/or virtual AtoN data to mobile AIS stations as stipulated by requesting Service

- External Basic AIS Service Category IV:

- CH_MON (primary): monitors the AIS VDL and provides relevant data to requesting service

- MOB_PROFILE: monitors individual mobile AIS stations and their profile on the AIS VDL, and provides relevant data to requesting service

For each of the above external BAS one functional description has been developed. The full description of the external BAS consists of a standardized table describing the functions of the interacting components of the AIS Service roughly, of a figure, which maps it to the appropriate components, and of additional functional detail description as appropriate. More detailed information on the Basic AIS Services is available in Annex 1.

## Data model of the AIS Service

### Introduction

The above Basic AIS Services define the functionality provided by the AIS Service to the shore-based e-Nav system in an abstract manner. They are operations performed on certain data objects, or: they are f(x), to express it in mathematical terms. The data objects treated by the Basic AIS Services are the operands, i.e. the ‘x’, of these functions. To provide a complete picture of what the AIS Service delivers to the shore-based e-Nav system, it is necessary to consider those abstract data objects and their structure and mutual relationship. Eventually, it is the abstract data, which is only relevant for other services of the shore-based e-Nav system.

### Benefits of the Data Model

The Data Model of the AIS Service provides the well-structured and abstract (functional) description of the data, the AIS Service exchanges with requesting services of the common shore-based e-Nav system. In the Data Model all data objects are expressively described in one place together, with data attributes based on an appropriate data type definition.

The Data Model supports the concept of the Basic AIS Services (BAS) because data type and data object descriptions used by different BAS need only be described once.

Also, the Data Model shows the contribution of the different BAS to the overall knowledge the shore based e-Nav system may gain about a traffic object (mainly ships) by the AIS Service at all. [Hence, the Data Model shows the contributions of the AIS Service to the Universal Maritime Data Model (UMDM). It also assigns an appropriate branch of the Universal Data Object Identifier (U.DOI) to the data objects of the AIS Service.]

Due to its abstract description, the Data Model provides the designers of requesting services and / or applications using a variety of shore based e-Nav services precise information on what data the AIS Service may provide (receive) or expect (transmit), without the need to know e.g. the encoding details of Recommendation M.1371. Hence the data model helps to encapsulate the “science of the AIS” proper and confine it to the AIS Service on behalf of the shore-based e-Nav system as a whole.

Although, the Data Model does not contain encoding information, encoding is required on application level (i.e. regarding the BAS). The Data Model supports a variety of general purpose encoding techniques on application level, such as XML or SQL-like queries, but also more efficient and tailored encoding schemes such as IEC 61162-derived sentences (with abstract data content, though). Hence, the Data Model of the AIS Service provides flexibility in selecting an encoding technique as deemed appropriate by the competent authority. A portfolio of appropriate encoding techniques to be used with data objects of the AIS Service is given in the Interfacing Model (see below).

The Data Model of the AIS Service is given in Annex 2.

## Structure model - the Layered Structure of the AIS Service

### Definition of the layered structure of the AIS Service

The AIS Service consists of three main functional layers as follows (refer to Figure 2 on the following page):

- Logical Layer of the AIS Service

- Physical Layer of the AIS Service: comprises all Remote Shore Stations (RSS) of the AIS Service

- Service Management Layer of the AIS Service

The Physical Layer directly interacts with the AIS VDL. The AIS VDL as such does not belong to the AIS Service, although the AIS Service may influence it by its usage and/or management. (The AIS VDL is also outside the shore-based e-Nav system.)

For the description of Remote Shore Stations (RSS), the Physical Layer can be further subdivided into three layers as follows:

- Physical AIS Shore Station (PSS)

- AIS Fixed Stations Layer of the AIS Service

- RF Components of the AIS Service

These layers comprise all components necessary to implement and operate the AIS Service. Each layer comprises:

- service component, which provides the required functionality in terms of AIS-specific data processing;

- supporting components, which are exclusively used by the AIS Service, such as computers and local networking devices, i.e. the so called service owned infrastructure.

- (remote) Human Machine Interfaces (HMI) to allow for access to Technical Operation Personnel

Each layer is supported by on-site framework infrastructure, such as energy supply, which may be shared with other on-site e-Nav services.

Further details will be given in the section “Functional Components of the AIS Service” below.

Figure 2: Layered structure of AIS Service – Structure model of the AIS Service

Technical Operation Personnel (master control)

HMI

Logical Layer -

Logical AIS Shore Station (LSS)

HMI

The AIS Service

Net Data

Status

AIS Service

Management Layer

HMI

Physical AIS Shore Station (PSS)

HMI

Fixed AIS Stations Layer (Base stations, repeater stations)

HMI

RF components of the AIS Service

AIS VHF Data Link (VDL)

(Time Division Multiple Access)

Technical Operation Personnel

(there are tasks for each layer in principle)

Configuration

Physical

Layer /

Remote

Shore

Station

* Note: This symbolic representation does not imply any inference as to the amount of required personnel for the technical operation of the AIS service. This symbolic representation aims at indicating that human interaction with the largely automated AIS Service is required as the last resort - and in some cases possibly on a regular basis - on all layers of the AIS Service.

Boundary of shore-based e-Nav system

### Mapping of AIS Service capabilities to component functionalities and component requirements

To arrive at the required functionalities and at the requirements of the components internal to the AIS Service, it is necessary to map the Basic AIS Services (both external and internal) to the relevant individual components internal to the AIS Service. In accordance with Figure 2 and the above section there are the three main service components, namely Logical AIS Shore Station (LSS), Physical AIS Shore Station (PSS), and AIS Service Management (ASM) to be considered for the mapping. The AIS Base Stations, the AIS antennas, and also the AIS VDL are omitted from the consideration at this point since they are transparent in regard to the net information flow. They will re-appear when considering secondary issues of the AIS Service, such as component management proper or loading of the AIS VDL.

As an example, the Figure 3 shows the mapping of an external Basic AIS Service (Category-I) to the component functionalities of the above components.

Figure 3: Sample mapping of external BAS (Category I) to component functionality

The required component functionalities for all Basic AIS Services of the AIS Service are collected in those Annexes of this document which deal with the functional descriptions of the respective components (see below). They constitute the functional requirement description for that particular component.

## Distribution model for the AIS Service

So far the most fundamental capabilities of the AIS Service and its internal structure have been introduced. It is now necessary to consider the bearing of geographical topology on the design of the AIS Service. Figure 4 on the following page provides a generic geographic distribution setup of a shore-based e-Nav system and the place of the AIS Service.

Figure 4:  Generic geographic distribution setup of a shore-based e-Nav system and the place of the AIS Service

In most cases, due to coverage area requirements, the AIS Service of an administration spans a larger part of its coast or even the whole of its coast(s). The actual topology of the coastlines may vary from country to country. The actual number of Remote Shore Stations (RSS) needed depends on a variety of aspects, as does the number of sites, where Logical AIS Shore Stations would reside. In some cases, the AIS Service may shrink to an “one spot AIS Service”, e.g. when there is a geographically isolated administration in remote areas.

Despite the differences in individual geography and topology there are functional similarities regarding the distribution of components of the AIS Service, and also certain criteria to select a specific distribution configuration. The distribution model of the AIS Service describes some standard distribution configurations which would apply to most administrations, and also provides AIS-specific rationales and criteria.

The distribution model is essentially functional and generic. It needs to be applied (“tailored”) to the concrete geography and topology by every administration. To that end guidance is provided how to arrive at a proper individual distribution model taking into account the following aspects:

- coverage requirements and coverage planning

- coastal topology (concave, convex, line, one spot, ...)

- required functionality setup of an AIS Service, i.e. required Basic AIS Services

- distribution of receive / transmit functionalities

- maintenance and life-cycle considerations (rationalization)

- data transfer network considerations

- infrastructure requirements

The Distribution Model for the AIS Service is given in Annex 3.

## Interaction and data storage model of the AIS Service

### Introduction to and Justification for the Interaction Model of the AIS Service

There are three main reasons for introducing an Interaction Model:

- identify and generically describe the complex dynamic interactions between the various components of the AIS Service, taking into account their geographical distribution;

- provide a framework for the functionality description of the main functional components of the AIS Service, namely of the Physical Shore Station (PSS), the Logical Shore Station (LSS), and the AIS Service Management (ASM);

- provide guidance to a competent authority for proper and responsible operation of the components of the AIS Service.

#### Dynamic interactions within the AIS Service and the role of the functional components therein

So far, static aspects of the AIS Service have been considered. It is now necessary to introduce the mechanics of the internal working of the AIS Service.

The Interaction Model of the AIS Service shows the interaction between its relevant components. The interaction model combines the Basic AIS Service descriptions, the Structure Model and the Distribution Model and addresses issues associated with the the data flow mechanism in a distributed AIS Service setup. Hence, the Interaction model of the AIS Service weaves together the different strands which have been introduced so far.

#### Framework for the functionality description of the functional components

By doing so, the Interaction Model provides a framework for the functionality description of the main functional components of the AIS Service, namely of the Physical Shore Station (PSS), the Logical Shore Station (LSS), and the AIS Service Management (ASM).

This functionality description would allow for an eventual type approval applied to the PSS, the LSS, and the ASM software module, as far as the AIS-specific functionality is concerned.

The functionality of these functional components would consist of the sum of all functions needed to support all interaction scenarios identified within the Interaction Model of the AIS Service, which in turn would be derived from the service use cases of the AIS Service. Due to the complexity of the AIS Service many different interaction scenarios need to be considered. To reduce the number of interaction scenarios to be considered and thus reduce complexity, an appropriate methodology is introduced below.

#### Responsible deployment and operation of components of an AIS Service by the competent authority

Concerns have been expressed regarding the powerful functionality of AIS Base Stations and regarding their improper operation which may consume too much of the AIS VHF data link capacity. Therefore, guidance is required for competent authorities setting up and operating an AIS Service

- how to deploy and set up the powerful functionality provided for competent authorities by Recommendation ITU-R M.1371 in a responsible manner;

- how to manage and configure during run-time the operation of their AIS Service and of the AIS VHF data link in a responsible manner.

This is particularly true when considering the configuration and control of the two fundamental modes of AIS Base Stations, i.e. Dependent Mode operation and Independent Mode operation, by the Physical Shore Station (PSS).

The Interaction Model provides the comprehensive functional framework both

- for proper deployment and functional set-up of shore-based AIS capabilities by a competent authority and

- for responsible run-time configuration of an AIS Service by a competent authority.

While the role of the Interaction Model for the proper deployment and functional set-up of an AIS Service will be further developed here, its role for the responsible run-time configuration aspect is covered by several sections and Annexes below within the section “VDL Usage by AIS Service and VDL Management”.

Thus, IALA fulfils this part of its task to provide guidance to competent authorities setting up and operating an AIS Service, and also effectively answers the above concerns.

### Categories of external BAS

As a first step to arrive at an Interaction Model for the AIS Service, the external BAS are categorized in accordance with their interaction behaviour. Thereby the explanation for the BAS Categories in the above section is given:  The external BAS are arranged in Basic AIS Service Categories in accordance with the net information flow behaviour of the BAS. The categories simplify the further description of the various aspects of the BAS (for further justification details refer to IALA Recommendation on the generic e-Nav service model template, e-Nav 210). The Figure 5 shows the categories of the external BAS Categories.

Figure 5: External BAS Categories I - IV

External BAS Category I:				External BAS Category II:

External BAS Category III:			External BAS Category IV:

### Additional primary use cases of the AIS Service

Each Basic AIS Service constitutes a so called primary use case of the AIS Service, or a primary service use case. All primary use cases of the AIS Service taken together provide the summary capability statement the AIS Service from the perspective of the shore-based e-Nav system (compare Figure 6 on the following page).

Shore-based

e-Nav

system

Point of view

from ashore

Encapsulation of complexity

for shore-based e-Nav applications

AIS

Service

AIS VHF Data Link

(AIS VDL)

E-Nav

Appli-cation

E-Nav

Appli-cation

E-Nav

Appli-cation

Basic AIS Services / primary use cases of the AIS Service as its functional interface to the shore-based e-Nav system

Figure 6: Primary Use Cases of the AIS Service as comprehensive capability statement of the AIS Service

Most of the primary service use cases are external Basic AIS Services (BAS). The external BAS genuinely treat the AIS data as such. In addition, there are further primary use cases required, e.g. replay functionality.

Figure 7 provides an overview on all primary service use cases of the AIS Service, using the above BAS Categories and also indicating the relationship between them within the object oriented paradigm.

Figure 7: Complete overview on primary service use cases of the AIS Service

Note: The dotted lines indicate inheritance relationship.

### Secondary and tertiary use cases of the AIS Service

It should be noted, that there are also secondary and tertiary use case of the AIS Service, too.

Secondary service use cases are supportive in nature, but are essentially required. Tertiary service use cases only treat faulty conditions or optimize functionality.

Secondary use cases are not directly visible outside the AIS Service, neither on the AIS VDL nor within the shore-based e-Nav system, although their effect may be there.

Secondary service use cases mainly interact with the AIS Service Management (ASM) and thereby with the AIS Service Technical Operation Personnel.

Secondary service use cases may request the support of other services of the shore-based e-Nav system, i. e. rendering them “requested services” and turn the AIS Service into a “requesting service”.

The Internal Basic AIS Services are secondary service use cases of the AIS Service. They are also categorized into Internal BAS Categories in a similar fashion like the external BAS. Again, there are additional secondary AIS Service use cases besides the Internal Basic AIS Services. Examples are Recording of AIS Service data (pre-requisite to any replay), AIS Service Initialization, AIS Service Termination and Component Management of the AIS Service. For a full description of the internal BAS refer to Annex 1.

Figure 8 shows an overview on all secondary service use cases of the AIS Service, and Figure 8bis shows the Internal BAS Categories in overview.

Figure 8: Overview on secondary AIS Service use cases, including internal BAS

Note: The dotted lines indicate inheritance relationship.

Figure 8bis: Internal BAS Categories I - IV

Internal BAS Category I:				Internal BAS Category II:

Internal BAS Category III:				Internal BAS Category IV:

### Consequences for the Interaction Model

All the previous definitions are now used to describe the mechanics of the internal working of the AIS Service as explained in the introduction to this section: An efficient description of the Interaction Model is obtained by using the BAS Categories, i. e. there is only one interaction description needed per BAS Category.

Specifically, the Interaction Model deals with aspects like:

- mapping of different data flow mechanism to topology taking into account different BAS Categories;

- push/pull considerations on service use case level (as opposed to protocol mechanics; for this refer to the Interfacing Model);

- acknowledgement on service use case level (as opposed to protocol mechanics; for this refer to the Interfacing Model);

- networking of data storage entities within the AIS Service on service use case level;

- functional connections within the (distributed) AIS Service (required for AIS Service Management)

### The Data Storage Model of the AIS Service

So far the capabilities of the AIS Service, its internal structure, the impact of the geographical topology on the AIS Service, and the internal working of the AIS Service have been considered. Now it is necessary to consider the related aspects of internal data storage within the AIS Service.

The Data Storage Model of the AIS Service deals with storage entities of the AIS Service on an abstract level. It is relevant in particular due to the geographical distribution of its components. It deals with aspects like

- visibility of data objects on application level

- age of data objects on application level

- abstract rule bases for data processing in the AIS Service on application level (i. e. on BAS level)

- replication of data objects due to geographical distribution of node sites within the Multi-Node Distribution model

The Interaction Model and Data Storage models of the AIS Service are given in Annex 4.

## Interfacing model of the AIS Service

The Interfacing model of the AIS Service describes

- the relationship of the AIS Service within the shore-based e-Nav system architecture,

- the interfaces of the AIS Service to other services of the shore-based e-Nav system and

- the interfaces between the components of the AIS Service.

Items b) and c) take into account the following aspects:

- the application encoding methods; a portfolio of encoding techniques is recommended to suit the data objects of the AIS Data Model;

- the protocol stacks recommended for data transfer internal and external of the AIS Service.

All application encoding methods and protocol stacks recommended are in keeping with the open system philosophy of the shore-base e-Nav system architecture.

The Interfacing Model of the AIS Service is given in Annex 5

## Internal time latency model of the AIS Service

The Internal Time Latency Model of the AIS Service describes the time latency limits regarding the BAS (by external and internal BAS Category).

In particular the resulting permissible latency of AIS data delivery of the AIS Service operating within the shore-based e-Nav system is discussed. This is relevant, for example, when correlating AIS data with radar target data within the Value Added Processing Services of the shore-based e-Nav system.

From the BAS level, concrete timing requirements for components of the AIS Service and their mutual interconnection are derived.

The Internal Time Latency Model takes into account loading scenarios of the AIS VDL.

The Internal Time Latency Model of the AIS Service is given in Annex 6.

## Internal reliability model of the AIS Service

The Internal Reliability Model of the AIS Service analyses the internal reliability aspects of the AIS Service and describes how to measure internal reliability during run-time.

The Internal Reliability Model is based on the Structure Model, the Distribution Model and the Interaction Model of the AIS Service. Details are given in Annex 7.

## Test Model for the AIS Service

The Test Model for the AIS Service:

- describes what are good test strategies the AIS Service as a whole for Technical Operation Personnel, and

- provides guidance how to ascertain the proper function of an AIS Service after cold or warm start.

The Test Model does not generally deal with testing of individual components.

Details are given in Annex 8.

## Functional Components of the AIS Service

This section introduces the functional components of the AIS Service.

### Typical AIS Service physical setup

The Figure 9 on the following page shows a typical AIS Service physical setup. It is one possible translation of the more abstract layered structure depiction of the AIS Service in Figure 2 (see above) into one example setup using concrete physical components on concrete functional sites. Figure 9 shows a so-called One-Node AIS Service configuration setup. Figure 9 shows the functional components, the functional site designation, where these components reside, and also gives some indication regarding the interfacing of the AIS Service to its context within a shore-based e-Nav system. Figure 9 shows some important features of the AIS Service as follows:

- The setup of the Remote Shore Station (RSS), i.e. the components internal to it, will be described below in more detail. In Figure 9 it is sufficient to note, that there is RF equipment (symbolized by antenna on roof top of RSS), that allows the AIS Service to access the AIS VDL. The depiction of the RSS as a house also gives an indication of the relevance of shore-site framework infrastructure consideration.

- A Node site with the Logical AIS Shore Stations is shown, and how they are connected to the components of the other services of the shore-based e-Nav system at the same Node site. As an (typical) example, particularly the User Interaction Service of the shore-based e-Nav system is shown. The “other e-Nav services” may any other service of the shore-based e-Nav system.

- The components of the AIS Service Management (ASM) that reside at the Node site are shown. (It should be noted, that components of the ASM also reside at the Remote Shore Station; see below).

- Also the support of the AIS Service for Technical Operation Personnel by a Human Machine Interface (HMI) is shown.

- Note the concept of the functional sites in combination with the premises concept.

- All components at the premise N where the Node site, the Technical Operation Personnel site and the User site reside in the example are connected with LAN technology components. Only, when it comes to connecting the components of the AIS Service with its Remote Shore Stations on the Remote sites, functional links, i.e. WAN feeder links to be precise, will be used.

Cautionary note: Figure 9 should not be construed as normative or the only recommended way to set up an AIS Service. Figure 9 does claim however to be correct and consistent with the normative statements in this Recommendation in regard to what it shows.

Figure 9: Example of an AIS Service physical setup (One-Node AIS Service configuration within shore-based e-Nav System context)

### Overview over tasks of the functional components of the AIS Service

While Figure 9 describes one example of a workable AIS Service setup, it should be noted, that the available features and the available options in setting up an AIS Service can only precisely be captured by the more abstract descriptions provided in this Recommendation. Hence the following sections will continue the description of the AIS Service on a more abstract level, but will continue to give similar examples regarding the setup in more physical terms.

The functional components are defined by their individual set of tasks they are supposed to perform in order to provide the service use cases. The set of individual tasks of a functional component is identical with its component use cases, which were derived from the service use cases (see above). The Figure 10 on the following page shows the different tasks of the different functional components of the AIS Service.

Please note

- the different task orientation of functional components, i.e.

- the orientation of the Logical Layer towards the requesting / requested services of the shore-based e-Nav system and their requirements;

- the orientation of the Physical Layer towards the AIS VDL and its requirements;

- the need to have a service wide data storage which allows service-wide visibility and access of relevant (abstract) AIS data;

- the configuration and control capabilities of the AIS Service Management (ASM).

- that the instances are now implied, namely

- that the Physical Layer consists of all Remote Shore Stations (RSS) of the individual AIS Service of an individual administration;

- that the Logical Layer consists of all Logical AIS Shore Stations (LSS) set up as individual software processes of the individual AIS Service of an individual administration;

- that there are necessary functionalities, such as the service wide data storage, which are invisible to the net AIS data flow between the AIS VDL and the requesting services, and vice versa. These functionalities are supported by infrastructure components owned by the AIS Service (so called service owned infrastructure), namely by distributed data bases, local area networking, etc.

To specify concrete features and attributes of those component use cases, such as data processing capabilities, interaction capabilities, interfacing requirements, timing constraints, and reliability figures, the appropriate service models need to be considered as introduced above.

These models have discussed the AIS Service on the class level. In order to set up an AIS Service by an individual administration, it is necessary to eventually arrive at concrete components which materialize at concrete sites, i.e. at the instance level. This is where the Distribution model fits in.

All these different aspects need to be merged within the description of the functional components. This will be done in the appropriate Annexes, describing the particular functional component. In the following sections, a short introduction is provided.

Figure 10: Overview over tasks of the functional components of the AIS Servcie

This Figure 10 will be used and explained in several Annexes which elaborate certain aspects of this figure.

### Logical AIS Shore Station (LSS)

The software process of a LSS can run on any appropriate computer at any appropriate place. The options for selecting an appropriate configuration for setting up the instances of the LSS, and its consequences, are discussed in the Distribution Model of the AIS Service (see Annex 3).

An individual software process of an LSS running in an individual computer is called instance of a LSS. Hence, an instance of a LSS is a software process, which transforms the AIS data flow(s) associated with one or more Remote Shore Station(s) into a different AIS-related data flow associated with one (1) requesting service within the shore-based e-Nav system. The run-time configuration of any instance of LSS is managed by the AIS Service Management.

For each requesting service, the AIS Service sets up at least one instance of an LSS. An instance of LSS of the AIS Service communicates with one instance of a logical interface of the requesting service in an 1:1 relationship.

It is required that there are reliable functional connections between any instance of LSS and all associated AIS Shore Stations.

Details are described in Annex 9.

### Remote Shore Station (RSS)

The RSS is the most basic AIS-related entity, which can exist on its own in a real physical environment, as opposed to the Physical AIS Shore Station (PSS) or an AIS base station.

An RSS is physically fixed or is considered to be "fixed". An RSS may be theoretically mounted on a flying or floating platform, however. The latter cases are excluded from the scope of this document for simplicity's sake.

An RSS consists of at least the following components or functions:

- one Physical AIS Shore Station (PSS);

- one AIS base station (or one AIS repeater station);

- RF-domain equipment, at minimum simply a cable and a VHF antenna;

- an agent of the AIS Service Management at the Remote Site (ASM-RS) (generally another software process);

- service owned  infrastructure components on site, such as computer(s) to run the Physical AIS Shore Station and the software agents of the AIS Service Management, Local Area Networking components etc.;

- shared or framework on-site infrastructure, such as power supply, a means to protect the above component against environmental influence and damage, e.g. a shelter building or a housing case.

Thus, an RSS does not necessarily need to be considered large physically.

A RSS may generally have a UTC source of its own (there may be cases during which the AIS base station may be set up using only the synchronisation provided by the AIS VDL itself, i.e. UTC indirect or even slot synchronisation). This UTC source may be internal to the AIS base station such as a GNSS receiver, or external to the RSS (being a part of the on-site framework infrastructure), such as a solid-state (crystal oscillator) clock, also providing timing for the AIS base station(s) by an appropriate timing interface (such as IRIG / IEEE 1344).

Figure 11 on the following side illustrates these aspects. It zooms into Premise A given in Figure 9 above, while adding an overview of on-site framework infrastructure considerations (refer to Annexes 3 and 12 for further detail).

Figure 11: Typical setup of Remote Shore Station of the AIS Service (with on-site framework infrastructure and Technical Operation Personnel site; also typical)

Cautionary note: Figure 11 should not be construed as normative or the only recommended way to set up a Remote Shore Station of the AIS Service. Figure 11 does claim however to be correct and consistent with the normative statements in this Recommendation in regard to what it shows.

Conceptional co-location issues of RSS of the AIS Service with RSS of other shore-based e-Nav services at the same Remote Site, i.e. e.g. VHF Communications Service or DSC Service, are addressed in Annex 3 from the AIS Service’s point of view.

The Annex 12 on Remote Shore Stations (RSS) Considerations

- provides a detailed description of the RSS in total;

- provide guidance how to install all necessary components of a Remote Shore Station at the Remote Site in order to avoid interference;

- gives particular attention to the technical details of co-location with other e-Nav services operating at the same Remote Site.

#### Physical AIS Shore Station (PSS)

The Physical AIS Shore Station (PSS) performs the following main tasks:

- Pre-processing of AIS data in one or both directions (receive and/or transmit), depending on configuration

- Control of the Fixed AIS stations of the RSS depending on the configuration preset of the AIS Service Management

The PSS essentially is a software process running as an application software on a computer as a physical entity or as an integral firmware of a dedicated unit, such as a smart switch.

Details are described in Annex 10.

#### AIS Fixed Stations - Overview

The AIS Fixed Stations are the most basic AIS-related entity of any AIS shore infrastructure. Conceptually, they are a "bare bone" black-box like device defined by the functional description and interface definitions. The AIS Fixed Stations transform the presentation interface sentences, as received from the PSS, into actual AIS VDL message in RF for transmission via the RF components (transmit path), and vice versa (receive path).

There are three kind of AIS Fixed Stations, namely the AIS Base Station, the General Purpose AIS Base Station, and the AIS Simplex Repeater Station. Details for the AIS Fixed Stations are described in the appropriate IEC Standard (see references).

#### AIS Base Stations Dependent / Independent Mode

The AIS Base Station was designed for dependent only operation or independent operation. Both are under some control of a Physical Shore Station (PSS).

–	A dependent mode AIS base station accesses the AIS VHF data link (VDL) using the messages provided by the PSS. A dependent mode AIS base station operates in full dependency of the PSS, hence the name.

–	An independent mode AIS base station accesses the VDL using the messages provided by the PSS or by using internal control. When operated as an independent mode AIS Base Station the unit will be delegated certain autonomous functionality under the supervisory control of the PSS.

The PSS is responsible for AIS Base Station configuration, transmission scheduling, and processing of received information. Presentation Interface (PI) text sentences are used by the PSS to configure the AIS Base Station, schedule message transmissions, and output information. The AIS Base Station may also be used to monitor and improve the integrity of the AIS VDL.

- Further details are available in IALA Recommendation A-xxx on [“The Matrix”] and in IEC Standard 62320-1.

#### General Base Station (GBS)

The AIS Service in a littoral or port state is operated by the national competent authority for the whole of the national coverage area. This national competent authority has the need to control the AIS Service and the AIS VHF data link, by employing means like channel management, protected slot allocations, and assigned mode. The AIS Base Station allows for the full control of the AIS VHF data link.

However, there are other competent authorities responsible for local operations such as individual harbours, locks, marinas, off-shore structures. These regularly have an interest in monitoring the vessel traffic in their locally confined area of competence and exchange general operational information with ships in regard to their area of competence, e. g. docking scheduling, tugs operation.

The national competent authority is in charge of the full AIS Service functionality, including the control mechanisms. However, to allow local competent authorities to participate in and benefit from the AIS Service, there are two possible options for them:

- connect to the national AIS Service via a shore-based communication link,

- establish an own locally confined shore-based AIS infrastructure with general capacities.

Should the local competent authority be in need of tactical information on local ship movements or in situations where the national AIS Service does not provide coverage of that area, then the local competent authority could apply at the national competent authority to set up a General Base Station (GBS).

These General Base Stations do not exhibit any means of controlling the AIS Service and the AIS VHF data link and to broadcast any navigational signals. There is a requirement, though, that the operation of a GBS is co-ordinated with the national AIS Service in any case. In addition, national regulations may apply.

General overview on the capabilities and limitations of the GBS:

- The GBS is capable of receiving all AIS messages within its radio range. In the case of addressed AIS messages the GBS only receives messages addressed to itself.

- The GBS may transmit interrogation messages, binary and safety related messages. The GBS may also transmit acknowledgements to addressed messages.

- If the optional Aids-to-Navigation functionality is added then the GBS may also transmit AtoN-reports.

- The maximum permissible link utilization of a GBS is 20 slots per frame for all above messages except acknowledgements.

- The GBS is prohibited to transmit any controlling or navigational messages such as assigned mode commands, DGNSS broadcasts and to perform data link management and channel management. The GBS also does not provide secondary synchronisation support, i.e. it is not acting as a semaphore.

- GBS are subject to the channel management messages (TDMA and DSC Channel 70) of the national AIS Service, if performed by the national AIS Service.  Therefore, the GBS reacts like a mobile station in that it reacts to received Msg 22 and/or DSC-based AIS Channel Management command.

- Further details are available in IALA Recommendation A-xxx on [“The Matrix”] and in IEC Standard 62320-x.

#### RF domain equipment

RF domain equipment consists of the means to establish the VDL between the different AIS stations.  Antennas, cables and filters are components of the RF domain equipment.

### The AIS Service Management (ASM)

The AIS Service Management of a competent authority performs the following tasks:

- It acts as a management entity for the whole of the AIS Service (which will in most cases comprise more than one LSS, PSS, and AIS Fixed Station (refer to Distribution model, Annex 3)),

- it manages the internal BAS, including the management of the AIS VDL, and

- it is the last resort regarding faulty behaviour (tertiary service use cases).

Specifically, the ASM:

- invokes, initialises, configures and terminates all the instances of LSS and PSS at run-time;

- determines the functional connections between PSS and their associated LSS for them to use during run-time;

- determines the functional connections between the LSS and the requesting services’ logical interface associated with them, i.e. this top level acts as a "switch-board" for the data exchange relationships between the different processes;

- provides the Human Machine Interface for the Technical Operation Personnel to monitor the current status of the VDL and configure it accordingly by means of VDL management.

Due to the distributed nature of the AIS Service (compare Distribution Model of the AIS Service), the ASM consists of several distributed agents and of one centralized master functionality, all of which are functional sub-components of the ASM and are generally software processes. There are the following functional sub-components:

- ASM agent at the Remote Site, which represents the ASM at the Remote Shore Station of the AIS Service,

- ASM agent at the Node site, representing the ASM to the LSS,

- ASM master (regularly only one ASM master per AIS Service).

Details are described in Annex 11.

### Type approval considerations for PSS, LSS, and ASM software modules

When introducing the Interaction Model of the AIS Service above, the provision for type approval of the PSS, LSS and ASM software modules were given as a rationale. This section elaborates on that type approval aspect using the above introduction of the functionality descriptions for the PSS, the LSS, and the ASM.

Regarding type approval the following notions should be taken into account:

- Type approval for software modules should only apply to the AIS specific functionalities of the PSS, LSS, and ASM software modules. Ideally these AIS specific functionalities would be plug-in software modules. Type approval would not address or include infrastructure components owned by the AIS Service, such as computers, LAN components etc.

- The description of the AIS specific functionalities of PSS, LSS, and ASM in Annexes 9, 10, and 11 would be precise in terms of functionality while not being prescriptive regarding implementation method by a manufacturer. Thus, the functional components PSS, LSS, and ASM would be modelled as “black boxes”, rendering open interface definitions.

- The following aspects and the associated degree of impact should guide the selection of AIS specific functionality of these functional components subject to type approval:

| Aspect | Degree of impact |  |
| --- | --- | --- |
|  | HIGHER | LOWER |
| Local impact of AIS VDL related run-time configurations | PSS (for both modes of AIS Base stations) | ASM |
| Coastal-wide impact of AIS VDL related run-time configuration | ASM (in particular ASM-Master functionality) | PSS |
| Quality of data delivery to/from shore-based e-Nav system | LSS + PSS | ASM |

## Implementation, Installation and Maintenance

Details concerning the efficient technical operation and maintenance of the AIS Service are described in Annex 13.

## VDL Usage by AIS Service and VDL Management

There are the following aspects of VDL usage by the AIS Service:

### FATDMA planning and operation of an AIS Service

Details are described in Annex 14.

### Assigned mode operation of an AIS Service

Details are described in Annex 15.

### DGNSS broadcast via an AIS Service

The AIS Service does not contain a source for DGNSS correction data to be broadcast. Rather it acts as a “modem” to the DGNSS Correction Service of the shore-based e-Nav system architecture, hence “DGNSS broadcast via an AIS Service”: The AIS Service transmits DGNSS corrections received from the shore-based DGNSS Correction Service, while wrapping it in an AIS VDL message and also taking into account the specifics of the AIS VDL and also the relationship to other aspects of the AIS VDL operation such as FATDMA configuration and Channel management. Hence, the AIS Service encapsulates these specifics and thus alleviates the DGNSS Correction Service of any such AIS-specific consideration.

In Annex 16 the support of the AIS Service to the DGNSS Correction Service of the shore-based e-Nav system architecture is described. Also the guidelines for DGNSS corrections broadcast via an AIS Service are discussed.

### Channel management by an AIS Service

Details are described in Annex 17.

### Channel loading management by an AIS Service

Details are described in Annex 18.

# References

- IALA Recommendation A-123 on “The Provision Of Shore Based Automatic Identification Systems (AIS)” (A-123; Edition 2007)

- IALA Recommendation A-126 on “The Use of the Automatic Identification System (AIS) in Marine Aids to Navigation” (A-126, Edition 2007)

- IALA Recommendation A-xxx on [“The Matrix”] (future)

- IALA Recommendation eNAV-101 on “The e-Navigation architecture – the shore perspective”;

- IALA Recommendation eNAV-201 on “The common shore-based e-Navigation system architecture” (future)

- IALA Recommendation eNAV-210 on “The generic e-Navigation service engineering model template” (future)

(for further details in particular on the mechanisms refer to Annex 5)

The term BAS Categories will be explained in a section below. Here suffice it to use the term BAS Category to structure the list of External BAS into groups.

It should be noted, that there are internal Basic AIS Services as well, which will be introduced below. Their description can also be found in Annex 1.

The AIS Service is an e-Nav service from the “Data Collection and Data Transfer Services” group of the shore-based e-Nav system architecture. Therefore, it comprises a Physical Layer. Refer to IALA Recommendations e-Nav 201 and IALA Recommendation e-Nav 210.

For example: Replay, which is a primary service use case, is only possible if there has been done data recording, which is a secondary service use case.

Refer to Distribution model in Annex 3 for further details

Refer again to the Distribution model, Annex 3, for further explanation regarding the sites.

For further detail refer to the Interfacing Model, Annex 5.

For details regarding the setup and installation of an RSS also in infrastructure terms refer to the Installation guidelines for RSS, Annex 12.

The term “class” is used as defined by the object-oriented paradigm.

As is the case when mounted on an AtoN; the point here is, that the Physical AIS Shore Station has a fixed geographical position. If it is mounted on an AtoN the area in which the AtoN is allowed to swing is very small compared to the coverage area of this Physical AIS Shore Station.