e-Navigation Committee							e-NAV 5/7/5

5th session									15 Sept 2008

The Effect of GPS Jamming on Safe Navigation at Sea

# Introduction

In April 2008, the General Lighthouse Authorities of the United Kingdom and Ireland (GLA) collaborated with the UK Ministry of Defence’s Defence Science and Technology Laboratory (DSTL) to understand the effect of GPS jamming on safe navigation at sea.

# Background & Motivation

The US Global Positioning System (GPS) is currently the primary source of Position, Navigation and Timing (PNT) information in maritime applications, whether stand-alone or augmented with additional systems.  In the future, together with other Global Navigation Satellite Systems (GNSS - e.g. Galileo) it will underpin e-Navigation, the International Maritime Organisation’s future digital maritime architecture.  All GNSS signals are very weak when measured at the Earths surface and are vulnerable to intentional and unintentional interference and this may result in the denial of service over large geographical areas.  These trials were set up to understand the effect of GPS jamming on safe navigation at sea in terms of shore-based Vessel Traffic Services (VTS), aids to navigation (AtoNs), ships’ systems and people.  The study also explored the effect of GPS jamming on Enhanced Loran (eLoran) – a high-power, low-frequency, terrestrial back-up system.

# Trial Overview

DSTL deployed a 1.5W power GPS jammer at Flamborough Head on the East Coast of the United Kingdom and used both directional and omni-directional antennas.  The primary jammer coverage (main lobe) is illustrated in Figure 1.  Secondary coverage existing either side of the main lobe (side lobes) is not illustrated.

Figure 1 - Coverage area of the GPS jamming unit at 25m above ground level on maximum power of 1.58W ERP. (Image courtesy of DSTL)

The MoD issued formal notices regarding the peacetime use of GPS jamming units prior to the trials to ensure the safety of mariners.  The GLAs also issued a Notice to Mariners stating that the Flamborough Head DGPS reference station would be unreliable during the trials.

During the trials, the GLA vessel NLV Pole Star steered a 10 nm linear course between two waypoints that dissected the main lobe and side lobes of the GPS jammer as well as allowing GPS receivers to reacquire satellites beyond the jamming area.

The crew of the Pole Star was fully briefed prior to the trial and expected on-board GPS-enabled systems to fail.  It was prepared to use parallel indexing using radar when GPS was denied.

# Results

GPS is vulnerable to service denial by intentional interference using low-power jammers. These results can be extended to GPS service denial by unintentional interference including: spurious harmonics from active TV antennas, damaged GPS antenna cables and ionospheric effects (e.g. due to the eleven-year sun-spot cycle).

GPS service denial has a significant impact on maritime safety:

- On shore – the marine picture presented to Vessel Traffic Services (VTS) will be confused as AIS information with erroneous positions and high-velocities conflicts with the radar information. It is unclear how VTS operators would respond during a real event.

- AtoNs – DGPS reference stations can be jammed resulting in the absence of DGPS corrections and integrity information broadcast to users over a very large geographical area; the automatic identification system (AIS) used as an AtoN may broadcast incorrect information; and synchronised lights may not be synchronised, thus having an adverse impact on visual conspicuity.

- On ships – navigation, situational awareness, chart stabilisation and DSC emergency communications will be lost if they are based on GPS.  It is unclear how integrated bridge systems will react when the autopilot is being used – the results are potentially extremely hazardous.  Continuation of navigational safety is dependent on mariners’ abilities to recognise GPS service denial and to operate effectively using alternative techniques (e.g. radar parallel-indexing). Increased use of Electronic Chart Display Information Systems (ECDIS) will increase the attendant risks.

- On people – People are conditioned to expect excellent GPS performance. As a result, when ships’ crews or shore staff fail to recognise that GPS service denial is occurring and/or there is a loss of familiarity with alternative methods of navigation or situational awareness, there may be significant impact on safety and security. In this trial, despite the fact that the Pole Star’s crew was forewarned, problems were experienced with the ECDIS. Moreover, the number of alarms that can sound on the bridge can be distracting. Moving to other navigation techniques can cause an increase in bridge workload.

Finally, eLoran was unaffected by GPS jamming and demonstrated an accuracy of 8.1m (95%) which is comparable to stand-alone, single-frequency GPS. Consequently, eLoran can be used to detect erroneous positions and high velocities that may be experienced during GPS service denial. Moreover, when GPS is unavailable, eLoran can provide a PNT input to all maritime systems. In the future e-Navigation environment, the combination of GPS, Galileo and eLoran will provide robust and resilient PNT in order to reduce the impact of human error and to improve the safety, security and protection of the marine environment.