3.2.3 Radar Aids to Navigation

Radar aids to navigation are devices that are meant to work with the radar installed on ships and other vessels.

The IMO carriage requirements contained in Chapter V of the SOLAS Convention 1974 (as amended) Regulation 19, state:

- all ships of 300 gross tonnage and upwards to carry a 9 GHz radar, and;

- all ships of 3,000 gross tonnage and upwards to be fitted with a 3 GHz radar or, where considered appropriate by the Administration, a second 9 GHz radar.

Some administrations may impose other carriage requirements.

IMO Resolution MSC.192(79) Adoption of the Revised Performance Standards for Radar Equipment 06 December 2004 states that 9 GHz radars should be capable of detecting radar beacons and should be capable of detecting SARTs and radar target enhancers.

9 GHz radars are also extensively carried by vessels not covered by SOLAS or local regulation. Because of this high rate of carriage, radar aids to navigation, especially in the 9 GHz band, are especially useful.

3.2.3.1 New Technology Radars

Previous to IMO Resolution MSC.192(79) 06 December 2004, IMO radar performance standards stated that 3 GHz radars should be capable of detecting radar beacons. Resolution MSC.192(79) removed this requirement.

There are a number of factors that were considered in this decision. Among them are:

- Pressure from ITU and other organizations to reduce the bandwidth needed for 3 GHz radars and to reduce the amount of spurious out of band emissions.

- Demand by the mariner for higher performance radars, especially with small object detection and reduction of sea clutter. Higher performance 3 GHz radars have been available for many years. But, because of the techniques used by these radars, they would not detect racons, could not be used to satisfy carriage requirements and were rarely installed.

- Reduction of cost for sophisticated radars because of technology improvements in signal processing and solid-state transmitters.

With the removal of the racon detection requirement, ships owners are free to use higher performing radars, often referred to New Technology (NT) radars.

3.2.3.2 Radar Reflectors

A radar reflector is a passive device designed to return the incident radar pulses of electromagnetic energy back towards the source and thereby to enhance the response on the radar display. By design, a radar reflector attempts to minimise the absorption and random scattering effects. Most radar reflectors are designed for use with 9 GHz radars.

3.2.3.2.1 Applications

A radar reflector is generally installed as a supplementary device at sites that would also be marked with a light. The main objectives of its use are to enhance:

- target detection at long ranges (for example, for landfall navigation);

- target detection in areas of sea or rain clutter;

- radar conspicuity of aids to navigation to reduce the risk of collision damage.

3.2.3.2.2 Radar Cross Section (RCS)

The performance of a radar reflector can be defined in terms of its effective radar cross section (RCS). This is a value determined by comparing the strength of radar signals returned by the radar reflector with the equivalent return from a radar reflective sphere.

Example: A radar reflector quoted as having an RCS of 30 m² would be equivalent to a radar reflecting sphere of diameter:

D = 30×4π = 6.2 metres

3.2.3.2.3 Technical Considerations

The range at which a radar reflector target can be detected is dependent on the heights of the radar antenna and the reflector and the output power of the radar. There are analogies to the geographical range of visual marks. The radar performance of corner cluster reflectors may vary considerably from one make to another, despite being of similar physical size. This arises from differing design philosophies; some that favour the fabrication process and others that try to optimise the polar distribution of radar reflections.

Use of small radar reflectors can also be subject to multipath fading effects. Please see the document IALA Guideline 1010 on racon range performance for a discussion on multipath fading.

Most radar reflectors are designed for use by 9 GHz radars. The reflectors are also usable with 3 GHz radars; however, the effective radar cross section is about an order of magnitude less.

3.2.3.3 Radar Target Enhancers

A Radar Target Enhancer (RTE) is a device that amplifies and returns the pulse from a ship’s radar to give an enhanced image on the radar screen. Unlike a racon, the returned signal from an RTE is not coded. The RTE was designed primarily for buoy applications, but could also be used on small vessels that might normally carry a passive radar reflector. A paper on RTE trials presented at the 1998 IALA Conference noted that the RTE tested had an effective radar cross section (RCS) of about 100 square metres, compared with an RCS of 20 to 30 square metres for passive radar reflectors typically fitted to buoys.

To date, commercially available RTEs only operate in the 9 GHz band.

RTE use is subject to multipath fading effects. Please see the document IALA Guideline 1010 on racon range performance for a discussion on multipath fading.

3.2.3.4 Radar Beacon (Racon)

Racons are receiver/transmitter devices operating in the maritime radar frequency bands (9 and 3 GHz) that enhance the detection and identification of certain radar targets.

A racon responds to the presence of a ship’s radar by sending a characteristic pulse train. The response appears as a coded mark (or “paint”) on the ship’s radar display that highlights the range and bearing of the racon. The display paint can be fixed to a specified length or can be dependent on the radar range setting and uses a Morse character for identification.

Figure 1 – Example of a racon and a radar display with and without the racon character.

3.2.3.4.1 Applications

A racon is generally considered to be a supplementary aid to navigation installed at sites that would also be marked with a light. As noted in means that a large number of vessels are capable of making use of a racon.

A racon can be used for:

- ranging and identification of positions in ice conditions or on inconspicuous coastlines;

- identification of aids to navigation, both seaborne and land based;

- landfall identification;

- indicating centre and turning point in precautionary areas or Traffic Separation Scheme (TSS)

- marking hazards;

- indicating navigable spans under bridges;

- leading lines.

3.2.3.4.2 Frequency-Agile Racon

A frequency-agile RACON responds on the frequency on which it is interrogated and the response could be re-painted on each radar sweep. The purpose of frequency agility is to provide a signal to the radar that is within the receiver bandwidth of the radar.

However, to avoid masking other features on the radar screen the racon response is usually switched on and off on a preset cycle.

3.2.3.4.3 Signal Characteristics

Racons operate in the 9 GHz band with horizontal polarisation, and/or in the 3 GHz band with horizontal and optionally vertical polarisation.

Table 1 – Preferred terminology for the description of racon operating frequencies.

| Preferred Terminology | Alternatives |  |  |
| --- | --- | --- | --- |
| 9 GHz | 9300 -9500 MHz | X - band | 3 cm |
| 3 GHz | 2900 -3100 MHz | S - band | 10 cm |

3.2.3.4.4 Performance Criteria

The availability of a racon is the principal measure of performance determined by IALA.

In the absence of any specific considerations IALA recommends that the availability of a Racon should be at least 99.6%

Refer to IALA publications:

IALA Guideline 1010 on racon range performance;

IALA Recommendation R-101 on maritime radar beacons (Racons)’

IALA Recommendation O-113 for the marking of fixed bridges over navigable

waters.

3.2.3.4.5 Technical Considerations

There are a number of technical considerations:

- The angular accuracy of the bearing between the ship and racon depends entirely on the interrogating radar while the accuracy of the range measurement depends on both the radar and racon;

- When racons are used in leading line applications, an alignment accuracy of about 0.3 degrees can be expected;

- When the ship is very close to the racon, side-lobes from the radar antenna can trigger the racon. The resulting multiple responses on the radar display may be a distraction and can mask other targets. Side-lobe suppression techniques are standard features of frequency agile racons.

3.2.3.4.6 Use with New Technology Radars

All currently available and installed racons are designed for use with high power pulse radars. In comparison, NT radars use low power transmissions with long pulses. Because of the low received signal strength and long pulse at the racon, current racons are not likely to detect NT radars. Further, it is unlikely that current racons transmit a response usable by NT radars.

NT radars are installed new when needed and currently installed 3 GHz high power pulse radars are not expected to be replaced in the near future. Racons with 3 GHz capability continue to be useful. At some point there may be a requirement to replace all old radars. At this time, 3 GHz pulse capability will not be needed in racons.

IALA is encouraging CIRM and radar and racon manufacturers to form a working group to find ways for racons to be usable with NT radars.

“Active Radar Target Enhancers For Buoy Signature Enhancement”; Commissioners of Irish Lights.