Rapporteurs Report

Development in non-radio navigation systems

(Task M2)

1. Background

This report, prepared for the ENAV Committee, provides information about ongoing developments in the field of non-radionavigation systems. Furthermore developments of INS (Integrated Navigations Systems) which combine radio navigation and non-radionavigation systems will be reported. Most of the information is based on published material, conference proceedings and contacts with manufacturers in this area.

2. Results from previous Reports

Different previous reports informed about the integration of GNSS and INS. In eNAV2/5.2  a report was presented which informed about different coupling methods between the GNNS  and the IMU sensor (loosely coupled, tightly coupled or deeply coupled).  Another encouraging method to enhance the performance of an integrated positioning system based on GNSS and MEMS (Micro Electro Mechanical System), is the use of “VirtuaLites”,[1].

3. VirtuaLites

The VirtuaLites concept was developed such that the integration of a low cost MEMS technology together with GNSS could be done in a way that only minimal or no modification needs to occur within the existing GNSS receiver.

Principle

In order to combine GNSS measurements with MEMS position information, it is necessary to transform in a first step the MEMS measurements into GNSS like measurements (creation of virtual satellites) . The stepwise placement, construction and data processing  is the key strategy in this method. Especially the placement of the virtual satellites can follow different basic principles:

a) 	The constructed virtual satellites are used to replace any blocked GNSS satellite
b) 	The constructed virtual satellites are placed in the same position as the received 
  	visible GNSS satellites This has the effect  of providing a second range 
 	measurement to each visible GNNS satellite

c)  	The constructed virtual satellites are used for the placement  in the local ENU (East North Up) axis to construct a three virtuaLite system in an orthogonal orbit.

After the placement of the virtuaLites, the creation of the measurements is a straightforward matter that  requires the calculation of the Euclidean distance between the placed virtual satellites and the MEMS inertial system estimated receiver position. The processing of the virtuaLites will occur using existing GPS software, however, depending on the method used to place the virtuaLites, some pre-processing may be required of the combined measurements.

M. Hoppe, September 2008

[1]  Cole, A., Wang, J., Dempster, A.G., & Rizos, C., 2007. VirtuaLite, a new method for GPS 
      INS integration for the agricultural environment. Proceedings of ION GNSS 2007, DC ROM.