| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5B/TEMP/160 Subject: Question ITU-R 232/5 | Annex 1 to Document 5B/296-E |
|  | 1 June 2009 |
|  | English only |
| Annex 1 to Working Party 5B Chairman’s Report |  |
| working document towards a preliminary draft revision of recommendation itu-r m.1371-3 |  |

# 1	Discussion

## 1.1	Document 5B/260

Annex 5 of Document 5B/175 (the Chairman’s Report of the last WP 5B meeting) is a “Preliminary draft new Report ITU-R M.[SAT-AIS] - Improved satellite detection of AIS” which has now been approved by WP5B. This addresses further studies and the need to revise Recommendation ITU-R M.1371-3. Consequentially, Annex 4 to Recommendation ITU-R M.1371-3 should be revised as shown in Annex 1.

## 1.2	Document 5B/88

The International Committee of the Red Cross (ICRC) noted that Resolution 18 (Rev.WRC-07) relates to the procedure for identifying and announcing the position of ships and aircraft of States not parties to an armed conflict. However, if Resolution 18 (Rev.WRC-07) is to be deleted at the next WRC-11, there would be a need to adapt the current texts which would not make sense any longer. Therefore it is proposed to use the terms contained in the title of Resolution 18 (Rev.WRC-07), namely "ships and aircraft of States not parties to an armed conflict". This term is well known and understood in international law, in particular in the situations where there is a need for correct identification of neutral vessels. Consequentially, Recommendation ITU-R M.1371-3, Table 50 of Annex 8 should be revised as shown in Annex 1.

## 1.3	Document 5B/186

The IMO Sub-Committee on Safety of Navigation (NAV), at its fifty-fourth session (30 June to 4 July 2008), considered the impact of Resolution MEPC.118(52) upon existing AIS shipboard installations. As a consequence, with the entry into force of Resolution MEPC.118(52), the reference to hazard or pollutant categories A, B, C and D have changed to hazard or pollutant categories X, Y, Z and OS, and therefore Recommendation ITU-R M.1371-3, Table 45 of Annex 8 should be revised as shown in Annex 1.

## 1.4	Document 5B/190

IMO thanked ITU-R Working Party 5B for the liaison statement concerning the proposed text message within message 14 to identify the modes of the AIS SART active and test, as well as the navigational status to be used by this device when activated. The Sub-Committee on Radiocommunications and Search and Rescue (COMSAR), at its thirteenth session (19 to 23 January 2009), noted, in relation to the proposed text message within message 14, that ITU-R WP 5B finds both suggestions acceptable.  The Sub-Committee has decided to stay with its initial position that the messages should be as simple as possible; therefore the following wording should be used in Recommendation ITU-R M.1371-3 Annex 8 for Message 14:

1	For the active SART, use the text “SART ACTIVE”.

2	For the test mode, use the text “SART TEST”.

The COMSAR Sub-Committee also considered the ITU-R WP 5B suggestion relating to the navigational status in messages 1, 2 and 3.  The Sub Committee believes that the reserved number 14 should be activated for the use by AIS-SART. The message used should define the navigational status as “AIS-SART” when the device is activated.  The Sub-Committee further agreed that when the device is under test the navigational status should be set to 15 “NOT DEFINED”. Consequentially, Recommendation ITU-R M.1371-3 Annex 8 should be revised as shown in Annex 1.

# 2	Actions required

1	IALA is requested to incorporate the attached Annex 1 into its preliminary draft revision of Recommendation ITU-R M.1371-3 and to upgrade the IALA technical clarifications to Recommendation ITU-R M.1371-3 accordingly;

2	IALA is requested to provide a new preliminary draft revision of Recommendation ITU-R M.1371-3 along with an appropriate reply liaison statement for consideration at the next ITU-R WP 5B meeting;

3	IEC TC 80 is invited to consider the actions taken by ITU-R WP 5B and IALA for the relevant test standards; and

4	IMO is invited to consider the actions taken by ITU-R WP 5B and IALA.

Annex 1

Working document toward a preliminary draft 
revision of Recommendation ITU-R M.1371-3

Consequential revisions to

Annex 4

Long-range applications

# 1	General

Long range applications should be by interface to other equipment and by broadcast.

# 2	Long range applications by interface to other equipment

Class A shipborne mobile equipment should provide a two-way interface for equipment which provides for long-range communications. This interface should comply with IEC 61162 series.

Applications for long-range communications should consider that:

–	The long-range application of AIS must operate in parallel with the VDL. Long-range operation will not be continuous. The system will not be designed for constructing and maintaining real-time traffic images for a large area. Position updates will be in the order of 2-4 times per hour (maximum). Some applications require an update of just twice a day. It can be stated that long-range application forms hardly any workload to the communication system or the transponder and will not interfere with the normal VDL operation.

–	The long-range operational mode will be on interrogation basis only for geographical areas. Base stations shall interrogate AIS systems, initially by geographical area, followed by addressed interrogation. Only AIS information will be included in the reply; e.g. position and static and voyage-related data.

–	The communication system for long-range AIS is not defined in this Recommendation. -

Example configuration:

Operation with Inmarsat-C.

The general set-up of the long-range configuration is in Fig. 25.

Because of the lack of IEC 61162-2 interfaces on long-range communication systems, the configuration shown in Fig. 26 can be used as an interim solution.

# 3	Long Range Applications by Broadcast

Long range AIS receiving systems may receive long range AIS broadcast messages, provided these messages are appropriately structured and transmitted to suit the receiving systems.

## 3.1	Packet bit structure for the AIS long range broadcast message

Long range AIS receiving systems require suitable buffering in order to preserve the integrity of the AIS message in the AIS slot boundaries. Table 1 shows a modified packet bit structure that is designed to support reception of AIS messages by satellites with orbital altitudes up to 1000 km.

TABLE 1

Modified packet bit structure for long range AIS message reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 24 | Standard |
| Start flag | 8 | Standard |
| Data field | 96 | Data field is 168 bits for other single-slot AIS messages. This field is shortened by 72 bits to support the long range receiving system buffer |
| CRC | 16 | Standard |
| End flag | 8 | Standard |
| Long range AIS receiving system buffer | 96 | Bit stuffing = 4 bits Synch jitter (mobile station) = 3 bits Synch jitter (mobile/satellite) = 1 bit Propagation time delay difference = 87 bits Spare = 1 bit |
| Total | 256 | Standard (NOTE – Only 160 bits are used in the 17 millisecond transmission) |

## 3.2	Long range AIS broadcast message

The long range AIS broadcast message data field is shown in Table 2.

TABLE 2

Data field for long range broadcast message; Message 27

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message ; always 27 |
| Repeat indicator | 2 | Always 3 |
| User ID | 30 | MMSI number |
| Position accuracy | 1 | As defined for Message 1 |
| RAIM flag | 1 | As defined for Message 1 |
| Navigational status | 4 | As defined for Message 1 |
| Longitude | 18 | Longitude in 1/10 min (180º, East = positive, West = negative |
| Latitude | 17 | Latitude in 1/10 min (90º, North = positive, South = negative |
| SOG | 6 | Knots (0-62); 63 = not available = default |
| COG | 9 | Degrees (0-359); 511 = not available = default |
| Status of current GNSS position | 1 | 0 = Position is the current GNSS position; 1 = Reported position is not the current GNSS position = default |
| Spare | 1 | Set to zero, to preserve byte boundaries |
| Total number of bits | 96 |  |

NOTE - There is no time stamp in this message. The receiving system is expected to provide the time stamp when this message is received.

## 3.3	Transmission method for the long range AIS broadcast message

The long range AIS broadcast message should be transmitted by the AIS Class A stations at the current power setting using the transmission interval, access scheme, AIS shore station qualifier and designated frequencies specified below. The AIS Class A should perform the long range AIS broadcast as a transmit-only function.

### 3.3.1	Transmission interval

- The nominal transmission interval for the long range AIS broadcast message should be 3 minutes.

### 3.3.2	Access scheme

The access scheme for transmitting the long range AIS broadcast message should be RATDMA. The AIS should consider only the two frequencies in the AIS VDL for qualifying the candidate slots for the transmission of the long range AIS message. The AIS VDL is comprised only of channel A (by default AIS 1) and channel B (by default AIS 2). The long range AIS broadcast on the long range frequencies should not be considered part of the AIS VDL for the purpose of scheduling the RATDMA transmissions of the long range AIS broadcast messages.

### 3.3.3	AIS shore station qualifier

- Transmission of the long range AIS broadcast message should be suppressed when the AIS Class A station is in range of an AIS base station. This should be implemented by reset of the 3-minute timer for the nominal transmission interval upon receipt of a Message 4 (the AIS base station message).

### 3.3.4	Transmitting the long range broadcast message

The long range AIS broadcast message should be transmitted on two channels. The transmissions should alternate between these two channels such that each channel is used once every 6 minutes.

Consequential revisions to

Annex 8

AIS Messages

## 3.1	Messages 1, 2, 3: Position reports

The position report should be output periodically by mobile stations.

TABLE 45

| Parameter |  | Number of bits |  | Description |
| --- | --- | --- | --- | --- |
| Message ID |  | 6 |  | Identifier for this message 1, 2 or 3 |
| Repeat indicator |  | 2 |  | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| User ID |  | 30 |  | MMSI number |
| Navigational status |  | 4 |  | 0 = under way using engine, 1 = at anchor, 2 = not under command, 3 = restricted manoeuvrability, 4 = constrained by her draught, 5 = moored, 6 = aground, 7 = engaged in fishing, 8 = under way sailing, 9 = reserved for future amendment of navigational status for ships carrying DG, HS, or MP, or IMO hazard or pollutant category Z, high speed craft (HSC), 10 = reserved for future amendment of navigational status for ships carrying dangerous goods (DG), harmful substances (HS) or marine pollutants (MP), or IMO hazard or pollutant category X, wing in grand (WIG); 11-13 = reserved for future use, 14 = AIS-SART (active), 15 = not defined = default (also used by the AIS-SART under test) |
| Rate of turn ROTAIS | 8 |  | 0 to +126 = turning right at up to 708° per min or higher 0 to –126 = turning left at up to 708° per min or higher Values between 0 and 708° per min coded by ROTAIS = 4.733 SQRT(ROTsensor) degrees per min where ROTsensor is the Rate of Turn as input by an external Rate of Turn Indicator (TI). ROTAIS is rounded to the nearest integer value. +127 = turning right at more than 5º per 30 s (No TI available) –127 = turning left at more than 5º per 30 s (No TI available) –128 (80 hex) indicates no turn information available (default). ROT data should not be derived from COG information. |  |
| SOG | 10 |  | Speed over ground in 1/10 knot steps (0-102.2 knots) 1 023 = not available, 1 022 = 102.2 knots or higher |  |
| Position accuracy | 1 |  | The position accuracy (PA) flag should be determined in accordance with Table |  |
| Longitude | 28 |  | 1 = high (= 10 m) 0 = low (= 10 m) 0 = default Longitude in 1/10 000 min (180º, East = positive (as per 2’s complement), West = negative (as per 2’s complement). 181= (6791AC0h) = not available = default) |  |
| Latitude | 27 |  | Latitude in 1/10 000 min (±90°, North = positive (as per 2’s complement), South = negative (as per 2’s complement). 91° (3412140h) = not available = default) |  |
| COG | 12 |  | Course over ground in 1/10 = (0-3599). 3600 (E10h) = not available = default. 3 601-4 095 should not be used |  |
| True heading | 9 |  | Degrees (0-359) (511 indicates not available = default) |  |
| Time stamp |  | 6 |  | UTC second when the report was generated by the electronic position system (EPFS) (0-59, or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if electronic position fixing system operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) |
| special manoeuvre indicator |  | 2 |  | 0 = not available = default 1 = not engaged in special manoeuvre 2 = engaged in special manoeuvre (i.e.: regional passing arrangement on Inland Waterway) |
| Spare |  | 3 |  | Not used. Should be set to zero. Reserved for future use. |
| RAIM-flag |  | 1 |  | Receiver autonomous integrity monitoring (RAIM) flag of electronic position fixing device; 0 = RAIM not in use = default; 1 = RAIM in use. See Table |
| Communication state |  | 19 |  | See Table 46 |
| Number of bits |  | 168 |  |  |

### 3.3.2	Type of ship

TABLE 50

| Identifiers to be used by ships to report their type |  |  |  |  |
| --- | --- | --- | --- | --- |
| Identifier No. | Special craft |  |  |  |
| 50 | Pilot vessel |  |  |  |
| 51 | Search and rescue vessels |  |  |  |
| 52 | Tugs |  |  |  |
| 53 | Port tenders |  |  |  |
| 54 | Vessels with anti-pollution facilities or equipment |  |  |  |
| 55 | Law enforcement vessels |  |  |  |
| 56 | Spare – for assignments to local vessels |  |  |  |
| 57 | Spare – for assignments to local vessels |  |  |  |
| 58 | Medical transports (as defined in the 1949 Geneva Conventions and Additional Protocols) |  |  |  |
| 59 | Ships and aircraft of States not parties to an armed conflict |  |  |  |
| Other ships |  |  |  |  |
| First digit(1) |  | Second digit(1) | First digit(1) | Second digit(1) |
| 1 – Reserved for future use |  | 0 – All ships of this type | – | 0 – Fishing |
| 2 – WIG |  | 1 – Carrying DG, HS, or MP, IMO hazard or pollutant category X | – | 1 – Towing |
| 3 – See right column |  | 2 – Carrying DG, HS, or MP, IMO hazard or pollutant category Y | 3 – Vessel | 2 – Towing and length of the tow exceeds 200 m or breadth exceeds 25 m |
| 4 – HSC |  | 3 – Carrying DG, HS, or MP, IMO hazard or pollutant category Z | – | 3 – Engaged in dredging or underwater operations |
| 5 – See above |  | 4 – Carrying DG, HS, or MP, IMO hazard or pollutant category OS | – | 4 – Engaged in diving operations |
|  |  | 5 – Reserved for future use | – | 5 – Engaged in military operations |
| 6 – Passenger ships |  | 6 – Reserved for future use | – | 6 – Sailing |
| 7 – Cargo ships |  | 7 – Reserved for future use | – | 7 – Pleasure craft |
| 8 – Tanker(s) |  | 8 – Reserved for future use | – | 8 – Reserved for future use |
| 9 – Other types of ship |  | 9 – No additional information | – | 9 – Reserved for future use |
| DG: Dangerous goods HS: Harmful substances MP: Marine pollutants (1) The identifier should be constructed by selecting the appropriate first and second digits. |  |  |  |  |

## 3.12	Message 14: Safety related broadcast message

The safety related broadcast message could be variable in length, based on the amount of safety related text. The length should vary between 1 and 5 slots.

TABLE 60

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 14; always 14. |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. See § 4.6.1, Annex 2; 0-3; 0 = default; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station of message |
| Spare | 2 | Not used. Should be set to zero. Reserved for future use |
| Safety related text | Maximum 968 | 6-bit ASCII as defined in Table 44 |
| Maximum number of bits | Maximum 1 008 | Occupies 1 to 5 slots subject to the length of text. For Class B mobile AIS stations the length of the message should not exceed 2 slots |

Additional bit stuffing will be required for this message type. For details refer to transport layer, § 5.2.1, Annex 2.

Table 61 gives the number of 6-bit ASCII characters, so that the whole message fits into a given number of slots. It is recommended that any application minimizes the use of slots by limiting the number of characters to the numbers given, if possible:

TABLE 61

| Number of slots | Maximum 6-bit ASCII characters |
| --- | --- |
| 1 | 16 |
| 2 | 53 |
| 3 | 90 |
| 4 | 128 |
| 5 | 161 |

These numbers also take bit stuffing into account.

The AIS-SART should use Message 14, and the safety related text should be:

1.	For the active SART, the text should be “SART ACTIVE”.

2.	For the test mode, the text should be “SART TEST”.