Maritime frequency allocations

The table below sets out the various current maritime frequency allocations as allocated by ITU Radio Regulation. Due to the size, footnotes to the allocation in the ITU Radio Regulation are not included. Please refer to Article 5 in the ITU Radio Regulation, Edition of 2008.

| Frequency | Region 1 Allocation | Region 2 Allocation | Region 3 Allocation | Status of Maritime Use |
| --- | --- | --- | --- | --- |
| 90 – 110 kHz | RADIONAVIGATION 5.62 Fixed 5.64 |  |  | No constraints |
| 285 – 315 kHz | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 5.72 5.74 | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 |  | No constraints |
| 315 – 325 kHz | AERONAUTICAL RADIONAVIGATION Maritime radionavigation (radiobeacons) 5.73 5.72 5.75 | MARITIME RADIONAVIGATION (radiobeacons) 5.73 Aeronautical radionavigation | AERONAUTICAL RADIONAVIGATION MARITIME RADIONAVIGATION (radiobeacons) 5.73 | Secondary in Region 1 |
| 415 – 435 kHz | MARITIME MOBILE 5.79 AERONAUTICAL RADIONAVIGATION 5.72 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  | No constraints |
| 435 – 495 kHz | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.72 5.82 | MARITIME MOBILE 5.79 5.79A Aeronautical radionavigation 5.80 5.77 5.78 5.82 |  | No constraints |
| 495 – 505 kHz | MOBILE 5.82A 5.82B |  |  | No constraints |
| 505 – 510 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION 5.72 | MARITIME MOBILE 5.79 | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile Land mobile | No constraints |
| 510 – 525 kHz | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION 5.72 | MOBILE 5.79A 5.84 AERONAUTICAL RADIONAVIGATION | MARITIME MOBILE 5.79 5.79A 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile Land mobile | No constraints |
| 1606.5 – 1625 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 | BROADCASTING 5.89 5.90 | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 1635 – 1705 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | FIXED MOBILE BROADCASTING 5.89 Radiolocation 5.90 | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 1705 – 1800 kHz | FIXED MARITIME MOBILE 5.90 LAND MOBILE 5.92 5.96 | FIXED MOBILE RADIOLOCATION AERONAUTICAL RADIONAVIGATION | FIXED MOBILE RADIOLOCATION RADIONAVIGATION 5.91 | Region 1 only and subject to restrictions on permitted coverage area |
| 2045 – 2065 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | FIXED MOBILE |  | Region 1 only |
| 2065 – 2107 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | MARITIME MOBILE 5.105 5.106 |  | No constraints |
| 2107 – 2160 kHz | FIXED MARITIME MOBILE LAND MOBILE 5.92 | FIXED MOBILE |  | Region 1 only |
| 2170 – 2173.5 kHz | MARITIME MOBILE |  |  | No constraints |
| 2190.5 – 2194 kHz | MARITIME MOBILE |  |  | No constraints |
| 2625 – 2650 kHz | MARITIME MOBILE MARITIME RADIONAVIGATION 5.92 | FIXED MOBILE |  | Region 1 only |
| 4000 – 4063 kHz | FIXED MARITIME MOBILE 5.127 5.126 |  |  | No constraints |
| 4063 – 4438 kHz | MARITIME MOBILE 5.79A 5.109 5.110 5.130 5.131 5.132 5.128 |  |  | No constraints |
| 6200 – 6525 kHz | MARITIME MOBILE 5.109 5.110 5.130 5.132 5.137 |  |  | No constraints |
| 8100 – 8195 kHz | FIXED MARITIME MOBILE |  |  | No constraints |
| 8195 – 8815 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 5.111 |  |  | No constraints |
| 12230 – 13200 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  | No constraints |
| 16360 – 17410 kHz | MARITIME MOBILE 5.109 5.110 5.132 5.145 |  |  | No constraints |
| 18780 – 18900 kHz | MARITIME MOBILE |  |  | No constraints |
| 19680 – 19800 kHz | MARITIME MOBILE 5.132 |  |  | No constraints |
| 22000 – 22855 kHz | MARITIME MOBILE 5.132 5.156 |  |  | No constraints |
| 25070 – 25210 kHz | MARITIME MOBILE |  |  | No constraints |
| 26100 – 26175 kHz | MARITIME MOBILE 5.132 |  |  | No constraints |
| 156 – 156.4875 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 | FIXED MOBILE 5.225 5.226 |  | Operation protected by footnote 5.226 |
| 156.4875 MHz – 156.5625 MHz | MARITIME MOBILE (distress and calling via DSC) 5.111 5.226 5.227 |  |  | No constraints |
| 156.5625 – 156.7625 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 | FIXED MOBILE 5.225 5.226 |  | Operation protected by footnote 5.226 |
| 156.7625 MHz – 156.8375 MHz | MARITIME MOBILE (distress and calling) 5.111 5.226 |  |  | No constraints |
| 156.8375 - 162.050 MHz | FIXED MOBILE except aeronautical mobile (R) 5.226 5.227A 5.229 | FIXED MOBILE 5.226 5.227A 5.230 5.231 5.232 |  | Operation protected by footnote 5.226 |
| 406 – 406.1 MHz | MOBILE-SATELLITE (Earth-to-space) 5.266 5.267 |  |  | No constraints |
| 456 – 459 MHz | FIXED MOBILE 5.286AA 5.271 5.287 5.288 |  |  | Footnote 5.287 specifies maritime mobile use |
| 460 – 470 MHz | FIXED MOBILE 5.286AA Meteorological-satellite (space-to-Earth) 5.287 5.288 5.289 5.290 |  |  | Footnote 5.287 specifies maritime mobile use |
| 1525 – 1530 MHz | SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile except aeronautical mobile 5.349 5.341 5.342 5.350 5.351 5.352A 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 | SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile 5.349 5.341 5.351 5.352A 5.354 | Complex |
| 1530 – 1535 MHz | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile except aeronautical mobile 5.341 5.342 5.351 5.354 | SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 |  | Complex |
| 1535 – 1559 MHz | MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A |  |  | Complex |
| 1616 – 1626.5 MHz | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) 5.208B 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 | MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) 5.208B Radiodetermination-satellite (Earth-to-space) 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 | Complex |
| 1626.5 – 1646 MHz | MOBILE-SATELLITE (Earth-to-space) 5.351A 5.341 5.351 5.353A 5.354 5.355 5.357A 5.359 5.362A 5.374 5.375 5.376 |  |  | Complex |
| 2900 – 3100 MHz | RADIOLOCATION 5.424A RADIONAVIGATION 5.426 5.425 5.427 |  |  | No constraints |
| 9200 – 9300 MHz | RADIOLOCATION MARITIME RADIONAVIGATION 5.472 5.473 5.474 |  |  | No constraints |
| 9300 – 9500 MHz | RADIONAVIGATION EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION 5.427 5.474 5.475 5.475A 5.475B 5.476A |  |  | No constraints |