e-Navigation Committee					e-NAV7/13/1

7th Session							21 Sept 2009

Racon Trials with NT Radar

Introduction

This input has been provided by the Research & Radio Navigation Directorate of the General Lighthouse Authorities of the UK and Ireland (GLA). It summarises the results of trials carried out by the GLAs on the performance of radar beacons (racons) with New Technology (NT) radar. The full report can be obtained by contacting nick.ward@gla-rrnav.org.

Background

From 1 July 2008, IMO MSC79 resolution 192(79) removed the requirement for S-band radar to trigger racons.  This has encouraged manufacturers to develop new types of radar; so called New Technology (NT) radar.

The GLAs have conducted racon trials with an NT radar fitted to the Commissioners of Irish Lights (CIL) tender Granuaile.  The trials were conducted in the Dublin Bay and Codling areas of the Irish Sea, where a number of fixed and floating CIL assets fitted with racons were used.

Summary of results

Trial results showed that:

- the radar did trigger racons but at a significantly reduced range;

- there were gaps in the racon response;

- short proportional responses can be obscured in clutter;

- some racons responded with a portion of the “paint” in front of the actual position of the racon;

- one racon gave a response which was offset in azimuth and range from the actual racon position.

Recommendations

The full report proposes that consideration be given to modifying existing racons to improve their response to NT radars.  It is suggested that such modifications should be conducted in consultation with racon manufacturers.  Further trials should also be arranged to clarify the results and test the effects of any modifications to existing racons.

Actions proposed

The Committee is invited to note the results and consider whether there is sufficient awareness of the potential effect of NT radars on racon performance and to assess the strategic options set out in the report. The Committee is also invited to review the recommendations and any coordinated measures that might be appropriate.