e-NAV User Requirements

# Introduction

The International Convention on Safety of Life at Sea (SOLAS) identifies the carriage requirements for shipborne navigational systems and equipment in Chapter V Safety of Navigation in Regulation 19.  As such, this equipment can be considered to form the basis of the shipboard user requirements for e-navigation.  Similarly, Chapter IV Radiocommunications identifies the carriage requirements for GMDSS.  The following list of equipment includes references to IMO performance standards for this equipment.  This list is extensive and the performance standards voluminous, so all of those requirements cannot be replicated here.  However, an attempt was made to capture the most basic requirements (i.e., the data elements output by the equipment) of each below.  The same approach should be done from the shoreside perspective.

If the shipboard user requirements behind these carriage requirements are supported by e-navigation, then there will be the same benefit to mariners.  Better information from added sensors will serve to enhance the benefit.  Through the e-navigation integration of this equipment, one can anticipate a potential reduction in equipment and hence, a cost savings to ship owners and operators.

# SOLAS Navigation and Communications Equipment

## Magnetic compasses. A.382(X) and A.694(17)

Magnetic heading/azimuth in degrees

Directional error not to exceed 0.5 degrees

## Transmitting magnetic heading devices. Formerly called electromagnetic compass - A.694(17) and MSC.86(70) Annex 2

Using geomagnetic field, obtain information about ship’s true heading (by magnetic or electromagnetic compass) and generate a suitable output signal in order to transmit this information to other equipment

Indicate any variation or deviation applied to compensate the heading

Static accuracy within +/- 1 degree.

Dynamic accuracy within +/- 1.5 degrees

## Gyrocompasses. A.424(XI) and A.694(17)

Geographic (true) heading in degrees

Settle point error not to exceed +/- 0.75 x secant latitude (up to 60 degrees latitude)

## Speed and distance indicating devices. Resolutions A.694(17) and A.824(19) as amended by MSC.96(72)

Speed presented in either analog or digital form with digital increments not to exceed 0.1 kts.  Analog displays graduated at least every 0.5 kts.

Distance displayed in digital form from 0 to not less than 9999.9 nm and increments not to exceed 0.1 nm.  Mode selection and mode indication should be provided if capable of being operated in either “speed through water” or “speed over ground” mode.

For digital display, speed error not to exceed 2% of speed or 0.2 kts, whichever is greater (when operating free from shallow water effect).

For analog display, speed error not to exceed 2.5% of speed or 0.25 kts, whichever is greater (when operating free from shallow water effect).

For output data transmission, speed error not to exceed 2% of speed or 0.2 kts, whichever is greater (when operating free from shallow water effect).

Errors in distance not to exceed 2% of distance run by ship in 1 hr or 0.2 nm in each hour, whichever is greater.

## Rate of turn indicators. A.526(13) and A.694(17)

Indicate rates of turn to port and starboard

Be self-contained or form part of, or derive information from, other appropriate equipment

Accurate to 0.5 degree/minute plus 5% of indicated rate of turn.

## Echo (depth) sounding equipment. A.224(VII) as amended by MSC.74(69) Annex 4, and A.694(17)

Depth of water between 2 metres – 200 metres

Accurate to +/- 0.5 metre on 20 metre range scale

+/- 5 metres on 200 m range scale or +/- 2.5% of indicated depth, whichever is greater

## Heading control systems. A.342(IX) as amended by MSC.64(67), Annex 3, and A.694(17)

Enable a ship to keep a preset heading with minimum operation of the ship’s steering gear

Connected to a suitable heading source.

Connected to a suitable source of speed information.

Off-heading alarm

## Auto-tracking aids. A.694(17), and MSC 64(67), Annex 4, Appendix 1

Obtain information about automatically plotted targets

Detection; Manual acquisition; Automatically track, process, simultaneously display and continuously update information on at least 10 targets

Select targets and display relevant symbol and target range, bearing, range and time of CPA, calculated true course and speed of target

## Track control system. A.694(17), and MSC 74(69), Annex 2

Keep a ship automatically on a pre-planned track over ground under various conditions within the limits of ship’s maneuverability

Steer the ship from current position to a single waypoint or along a sequence of waypoints

Requires an input from an Electronic Position Fixing System

Alarm on loss or reduction of power, position monitor deviation, heading monitor deviation, loss of position, cross track error, course difference, and low speed.

Continuously display mode of steering; sources of position, heading, speed; status and failure of sensors; track course and actual heading; actual position, cross track distance and speed; TO and NEXT waypoints; time and distance TO waypoint; next track course; and selected track identification.

## Automatic radar plotting aids. ARPA - A.694(17) and A.823(19)

Detection

Acquisition – manual or automatic

Tracking – automatically track, process, simultaneously display and continuously update at least 20 targets with range, bearing, range and time at CPA, true course and speed of target

## Electronic plotting aids. A.694(17) and MSC 64(67), Annex 4, Appendix 2

For small ships fitted with either gyro or transmitting marine electromagnetic compass.

Plot a minimum of 10 targets on a radar display

Display on demand plot number and time since last plot, range and bearing of target, range and time at CPA, true course and speed or target.

## Chart facilities for shipborne radar. Resolutions A.694(17), and A.817(19) as amended by IMO Resolution MSC.64(67), Annex 5

Selected parts of the System Electronic Navigation Chart (SENC) information may be displayed such that radar information is not masked, obscured or degraded.

SENC information should at least include coastlines, own ship's safety contour, dangers to navigation and fixed and floating aids to navigation.

## Electronic chart display and information system. ECDIS - Resolutions A.694(17), and A.817(19) as amended by MSC.86(70), Annex 4

Display all system electronic navigational chart information (display base, standard display, all other information)

Facilitate updating electronic navigational chart

Provide an indication whether the information is disp0layed at a larger scale than contained in SENC or own ship’s position is covered by an ENC at a larger scale than provided by display.

Radar information (radar image and ARPA information) should not degrade SENC information

Perform route planning, route monitoring, and voyage recording

Adequate back-up required

## Electronic chart display and information system back-up equipment. A.694(17) and A.817(19) as amended by MSC.64(67), Annex 5 and MSC.232(82), Annex 24

Ability for a safe take-over of ECDIS functions

Means to provide for safe navigation for the remaining part of voyage in case of ECDIS failure including chart presentation, route planning, route monitoring, display information, chart information, updating, scale, and voyage recording.

## Raster chart display systems. RCDS - A.694(17) and A.817(19) as amended by MSC.64(67), Annex 5 and MSC.232(82), Annex 24

When relevant chart information is not available in appropriate form, some ECDIS equipment may operate in RCDS mode.

Route planning, monitoring, and voyage recording

Alarms on route deviation; approach to critical point, line, area or mariner entered feature; positioning system failure; different geodetic datum; malfunction of RCDS mode.

Indication on ECDIS operating in raster mode; information under-scale or over-scale; larger scale RNC available

## Global positioning system equipment. GPS - A.694(17) and A.819(19)

Capable of receiving GPS SPS signals on L1 signal and C/A code

Static accuracy within 100 m (95%), 10m (95%) in differential mode

Indication within 5 s if either HDOP is exceeded or a new position not calculated for more than 2 s.

Warning on loss of position

Indication DGPS corrections being applied

## Global navigation satellite system equipment. GLONASS - A.694(17), and MSC.53(66)

Capable of receiving SPS signals of GLONASS on L1 signal and C code

Static accuracy within 35m (95%), 10m (95%) in differential mode

Indication within 5 s if either HDOP is exceeded or a new position not calculated for more than 2 s.

Warning on loss of position

Indication DGLONASS corrections being applied

## Differential global positioning system equipment. DGPS - A.694(17) and MSC.64(67), Annex 2

Operate in 283.5 – 315 KHz in region 1, 285 – 325 KHz in regions 2 and 3.

Provide means of automatically and manually selecting frequency.

Make data available not more than 100 ms after reception.

Be capable of acquiring signal in less than 45 sec.

Have at least one serial data output.

Alarm if no DGPS message is received.

## Differential global navigation satellite system (DGLONASS) equipment. DGLONASS - A.694(17) and MSC.64(67), Annex 2

Operate in 283.5 – 315 KHz in region 1, 285 – 325 KHz in regions 2 and 3.

Provide means of automatically and manually selecting frequency.

Make data available not more than 100 ms after reception.

Be capable of acquiring signal in less than 45 sec.

Have at least one serial data output.

Alarm if no DGPS message is received.

## Combined global positioning system and global navigation satellite system (GPS/GLONASS) receiver equipment. A.694(17) and MSC.74(69), Annex 1

Capable of receiving GPS SPS and range code signals in GLONASS in a combined receiver

Static accuracy within 35m (95%), 10m (95%) in differential mode

Indication within 5 s if either HDOP is exceeded or a new position not calculated for more than 1 s.

Warning on loss of position

Indication DGPS and DGLONASS corrections being applied

## Loran-C equipment. A.694(17) and A.818(19)

Latitude in degrees (2 digits), minutes, 100ths of minutes N or S

Longitude in degrees (3 digits), minutes, 100ths of minutes E or W

Coordinates in WGS-84

Time difference better than 0.3 microsec

## Chayka equipment. A.694(17) and A.818(19)

Latitude in degrees (2 digits), minutes, 100ths of minutes N or S

Longitude in degrees (3 digits), minutes, 100ths of minutes E or W

Coordinates in WGS-84

Time difference better than 0.3 microsec

## Integrated bridge systems. A.694(17) and MSC.64(67), Annex 1

Centralized access to sensor information or command/control

Support systems performing two or more of: passage execution; communications; machinery control; loading, discharging and cargo control; and safety and security.

Provide functional integration

## Integrated navigational systems. A.694(17) and MSC.86(70), Annex 3

Provide ‘added value’ to the functions and information needed by OOW to plan, monitor, or control the progress of the ship, to include basic functions, integrity monitoring, data exchange, integration, configuration control, display of information, and alarm management

Basic INS functions - combine, process, evaluate all sensors and distribute information, to include as a minimum: position, speed, heading, and time (for INS A). For INS B add depth where available.  For INS C add automatic control of heading, track, or speed.

## Voyage data recorders. A.694(17), A.861(20), and MSC.214(81)

Continuously record: date and time, ship’s position, speed, heading, bridge audio, communication links, radar data post-display selection, echo sounder, main alarms, rudder order and response, engine order and response, hull openings status, WTD and fire door status, accelerations and hull stresses, and wind speed and direction.

Recording medium in a protective capsule, capable of being accessed, secure from tampering, maximizing probability of survival and recovery, highly visible color and marking, and fitted with an appropriate device to aid location.

Provide an interface for downloading data to external computer

## Simplified voyage data recorders. - A.694(17), A.861(20), MSC.163(78)

Continuously record: date and time, ship’s position, speed, heading, bridge audio, communication links, radar data post-display selection, and AIS data.

Final recording medium installed in a protective capsule of either fixed or float-free type, capable of being accessed following incident, secure from tampering, maintain recorded data at least 2 years following termination of recording, highly visible color and marking, and fitted with an appropriate device to aid location.

Provide an interface for downloading data to external computer

## Shipborne automatic identification systems. A.694(17) and MSC.74(69), Annex 3

Ship-to-ship mode for collision avoidance; Littoral states obtain information about ship and cargo; as a VTS tool for traffic management

Provide information autonomously and continuously to a competent authority and ships without involvement of ship’s personnel

Receive and process information from other sources

Respond to high priority and safety related calls with a minimum of delay

Provide positional and maneuvering information at a data rate adequate to facilitate accurate tracking

Provide static, dynamic, voyage-related, and short safety-related messages

## Class B Shipborne automatic identification systems (AIS-B). Resolution A.694(17)

## Radar reflectors. Resolution A.384(X) and MSC.164(78)

Radar cross section of at least 7.5 m2 in X-band (9 GHz, 3cm wavelength) and 0.5 m2 in S-band (3 GHz, 10 cm wavelength).

## Sound reception systems. A.694(17) and MSC.86(70), Annex 1

Receive sound signals from all directions in audio band 70 – 820 Hz

Reproduce incoming sound signals acoustically inside the bridge

Indicate approximate direction of incoming sound signals to determine at least whether the sound signal is forward or abaft the beam and from which side of the ship it is being detected.

Suppress unwanted background noise and allow reception of meaningful sounds

## Daylight signaling lamps. A.694(17) and MSC.95(72)

Convey information between ships or between ship and shore by light signals

Visible 2 nm with atmospheric transmission of 0.8 or luminous intensity of 60,000 candela

Indication of operational status

Capable of being operated by personnel wearing gloves

## Gyrocompasses for high speed craft. A.694(17) and A.821(19)

Geographic (true) heading in degrees

Settle point error not to exceed +/- 0.75 x secant latitude (up to 60 degrees latitude)

Error not to exceed +/- 1 degree

Heading transmission system

Operate at speeds between 30-70 kts, max. rate of turn 20 degrees/sec, and at latitudes between 70N – 70S

## Heading control systems for high-speed craft (formerly automatic pilots). A.342(IX) as amended by MSC 64(67), Annex 3, A.694(17), and A.822(19)

Enable a ship to keep a preset heading within +/- 2 degrees with minimum operation of the ship’s steering gear

Connected to a suitable heading source.

Connected to a suitable source of speed information.

## Night vision equipment for high-speed craft. A.694(17) and MSC.94(72)

Detect at night hazards to navigation including unlit boats, floating logs, oil drums, containers, buoys, ice, hazardous waves, and whales.

Display information pictorially in real time

Detect (standard target) at a minimum of 600 m under mean starlight conditions without clouds and without moon

Capable of panning at minimum 30 degrees/sec

Indicate heading in field of view

Radar equipment MSC.64(67), Annex 4

Provide an indication in relation to the ship of the position of other surface craft and obstructions and buoys, shorelines, and navigational marks.

Electronic variable range marker

Electronic Bearing Line (EBL)

Indication of own ship heading

Detect and display radar beacons

## Global Maritime Distress and Safety System (GMDSS) A.694(17), General requirements for shipborne radio equipment forming part of the global maritime distress and safety system (GMDSS) and for electronic navigational aids (See table below)

General: VHF – Voice Channels 6, 13, 16, DSC Channel 70; radar transponder operating in 9 GHz band; NAVTEX receiver; MSI radio facility; 406 MHz EPIRB;

Additional requirements for:

Sea area A1 (within range of shore-based VHF-FM/DSC coast station, typically 20 nm from shore) – VHF-DSC

Sea areas A1 and A2 (within range of shore-based MF/DSC coast station, typically 100 nm from shore) – MF-DSC on 2187.5 KHz and voice on 2182 KHz

Sea areas A1, A2, and A3 (within Inmarsat satellite coverage, between 70N – 70S) – Inmarsat or HF-DSC

Sea areas A1, A2, A3, and A4 )Polar regions) – Polar orbiting system 406 MHz EPIRB and Inmarsat

# Annex 1: Existing SOLAS V mandatory carriage requirements

| No. | Facilities | Note | 0 to 150 GT | 150 to 300 GT | 300 to 500 GT | 500 to 1600 GT | 1600 to 3000 | 3000 to 10K | 10K to 50K | 50K GT < |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1.2 | Magnetic Compass |  |  |  |  |  |  |  |  |  |
| 2.1.4 | Paper chart or ECDIS |  |  |  |  |  |  |  |  |  |
| 2.1.5 | Back-up ECDIS | ECDIS in 2.1.4 |  |  |  |  |  |  |  |  |
| 2.1.6 | GNSS(GPS, DGPS) |  |  |  |  |  |  |  |  |  |
| 2.1.7 | Radar Reflectors | If possible |  |  |  |  |  |  |  |  |
| 2.1.9 | Communication for heading information | In case of emergency steering |  |  |  |  |  |  |  |  |
| 2.2.1 | Spare Magnetic Compass |  | Passenger ship |  |  |  |  |  |  |  |
| 2.3.1 | Echo Sounder |  |  |  |  |  |  |  |  |  |
| 2.3.2 | X band radar |  | Passenger ship | Passenger ship |  |  |  |  |  |  |
| 2.3.3 | Electronic Plotting Aids | EPA | Passenger ship | Passenger ship |  |  |  |  |  |  |
| 2.3.5 | THD (Transmitting Heading Device) |  | Passenger ship | Passenger ship |  |  |  |  |  |  |
| 2.4 | AIS (V1.0) |  | Passenger ship | Passenger ship | Passenger ship |  |  |  |  |  |
| 2.3.4 | SDME (through Water) |  | Passenger ship | Passenger ship |  |  |  |  |  |  |

SOLAS-V Carriage Requirement (2/2)

| No. | Facilities | Note | 0 to 150 GT | 150 to 300 GT | 300 to 500 GT | 500 to 1600 GT | 1600 to 3000 | 3000 to 10K | 10K to 50K | 50K GT < |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.5.1 | GYRO Compass |  |  |  |  |  |  |  |  |  |
| 2.5.5 | ATA (Automatic Tracking Aid) |  |  |  |  |  |  |  |  |  |
| 2.6 | 2nd Compass |  |  |  |  |  |  |  |  |  |
| 2.7.1 | S band 2nd Radar |  |  |  |  |  |  |  |  |  |
| 2.7.2 | 2nd ATA |  |  |  |  |  |  |  |  |  |
| 2.8.1 | ARPA |  |  |  |  |  |  |  |  |  |
| 2.8.2 | Track Control System |  |  |  |  |  |  |  |  |  |
| 2.9.2 | SDME (Speed over Ground) |  |  |  |  |  |  |  |  |  |
| 20 | VDR, S-VDR |  | Passenger ship | Passenger ship | Passenger ship | Passenger ship | Passenger ship |  |  |  |
| SOLAS-V 19-1 | LRIT (Long Range Identification & Tracking | On or after 31 Dec. 2008 |  |  |  |  |  |  |  |  |
| SOLAS-V | BNWAS (Bridge Navigational Watch Alarm System) | On or after 1st July 2011 |  |  |  |  |  |  |  |  |

# Annex 2: Existing SOLAS IV mandatory carriage requirements (example)

| Area | Ship (TG) | VHF + DSC | MF + DSC | MF/HF + DSC + NBDP | Inmarsat | SART | EPIRB | NAVTEX | EGC | TWO-WAY | TWO-WAY AIR | VHF DSC WKR | MF DSC WKR | MF/HF DSC WKR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 300-500 | 1 (*1) |  |  |  | 1 | 1 | 1 | [1] | 2 |  | 1 |  |  |
|  | 500 and Over | 1 (*1) |  |  |  | 2 | 1 | 1 | [1] | 3 |  | 1 |  |  |
|  | Passenger | 1 (*1) |  |  |  | 2 | 2 | 1 | [1] | 3 | 1 | 1 |  |  |
| A2 | 300-500 | 1 (*1) | [1] (*1) |  |  | 1 | 1 | 1 | [1] | 2 |  | 1 | 1 |  |
|  | 500 and Over | 1 (*1) | [1] (*1) |  |  | 2 | 1 | 1 | [1] | 3 |  | 1 | 1 |  |
|  | Passenger | 1 (*1) | [1] (*1) |  |  | 2 | 2 | 1 | [1] | 3 | 1 | 1 | 1 |  |
| A3 | 300-500 | 1 (*1) |  | [1] (*1) | [1] (*1) | 1 | 1 | 1 | [1] | 2 |  | 1 | [1] | [1] (*1) |
|  | 500 and Over | 1 (*1) |  | [1] (*1) | [1] (*1) | 2 | 1 | 1 | [1] | 3 |  | 1 | [1] | [1] (*1) |
|  | Passenger | 1 (*1) |  | [1] (*1) | [1] (*1) | 2 | 2 | 1 | [1] | 3 | 1 | 1 | [1] | [1] (*1) |
| A4 | 300-500 | 1 (*1) |  | 1 (*1) |  | 1 | 1 | 1 | [1] | 2 |  | 1 |  | 1 (*1) |
|  | 500 and Over | 1 (*1) |  | 1 (*1) |  | 2 | 1 | 1 | [1] | 3 |  | 1 |  | 1 (*1) |
|  | Passenger | 1 (*1) |  | 1 (*1) |  | 2 | 2 | 1 | [1] | 3 | 1 | 1 |  | 1 (*1) |

(*1) Duplication is required ; 
[] Brackets can be omitted by the terms and conditions.

EGC: Enhanced Group Call	TWO-WAY AIR: TWO-WAY VHF Air band	WKR: Watch keeping receiver