# Development of new characteristics

The following tables summarise the status of the various ITU technical characteristics relating to maritime communication applications both for those already existing as well as for those foreseen in e-Navigation. They outline where existing characteristics are likely to need updating or else where new characteristics need to be developed. If changes are required they will take place through the appropriate ITU Study Groups.

ITU Study Groups (SG) are responsible for developing draft recommendations to be provided to the periodic World Radio Conferences (WRC). Study groups are established and assigned study questions by a Radiocommunication Assembly to prepare draft Recommendations for approval by ITU Member States. In the case of e-Navigation two SG’s are likely to be involved in the development of material to be agreed at WRC, namely SG4 and SG5.

SG4 relates to satellite services, covering systems and networks for the following service:

- Fixed-satellite service,

- Mobile-satellite service,

- Broadcasting-satellite service and

- Radiodetermination-satellite service.

SG5 relates to terrestrial services, covering systems and networks for the following services:

- Fixed,

- Mobile,

- Radiodetermination,

- Amateur and

- Amateur-satellite.

Below the SG’s are subgroups, such as Working Parties (WP) and Task Groups (TG) that are established to study the questions assigned to the different Study Groups. For example, SG4 has the following WP’s:

- Working Party 4A - Efficient orbit/spectrum utilization for FSS and BSS

- Working Party 4B - Systems, air interfaces, performance and availability objectives for FSS, BSS and MSS, including IP-based applications and satellite news gathering

- Working Party 4C - Efficient orbit/spectrum utilization for MSS and RDSS

SG5 has the following WP’s:

- Working Party 5A - Land mobile service excluding IMT; amateur and amateur-satellite service

- Working Party 5B - Maritime mobile service including Global Maritime Distress and Safety System (GMDSS); aeronautical mobile service and radiodetermination service

- Working Party 5C - Fixed wireless systems; HF systems in the Fixed and Land Mobile Services

- Working Party 5D - IMT Systems

- Joint Task Group 5-6 - Studies on the use of the band 790-862 MHz by mobile applications and by other services

For maritime e-Navigation applications any necessary revision to, or creation of, technical characteristics will take place in one of two WP’s – SG4 WP4C and SG5 WP5B.

| Application |  | Current characteristics |  | Amendments | Mechanism |
| --- | --- | --- | --- | --- | --- |
| DSC | VHF | ITU-R M.493-12 | Digital selective-calling system for use in the maritime mobile service. | The draft revision of ITU-R M.493-2 was approved by SG5 in May 2009 to harmonise the functionality for non-SOLAS ships with IMO requirements. For other Recommendations, no amendments are foreseen for the use of VHF DSC within the e-Navigation concept as currently envisaged. However, if the VHF voice channels are changed to make them more spectrally efficient (e.g. see VHF voice below) there may be a need to revise the VHF DSC characteristics. | N/A |
|  |  | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | ITU-R M.693 | Technical characteristics of VHF emergency position-indicating radio beacons using digital selective calling (DSC VHF EPIRB) |  |  |
|  |  | ITU-R M.689-2 | International Maritime VHF Radiotelephone Systems with Automatic Facilities Based on DSC Signalling Format |  |  |
|  |  | ITU-R M.821-1 | Optional Expansion of the Digital Selective-Calling System for Use in the Maritime Mobile Service |  |  |
|  | MF/HF | ITU-R M.493-12 | Digital selective-calling system for use in the maritime mobile service. | The draft revision of ITU-R M.493-2 was approved by SG5 in May 2009 to harmonise the functionality for non-SOLAS ships with IMO requirements. For other Recommendations,no amendments are foreseen for the use of HF/MF DSC within the e-Navigation concept as currently envisaged. N/A | N/A |
|  |  | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service |  |  |
|  |  | ITU-R M.1173 | Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz |  |  |
|  |  | ITU-R M.1082-1 | International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  |  | ITU-R M.476-5 | Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
|  | Satellite | ITU-R M.541-9 | Operational Procedures for the Use of Digital Selective-Calling Equipment in the Maritime Mobile Service | DSC over satellite is not currently defined explicitly through a technical characteristic. However, it could be appropriate to develop a complementary characteristic to those available for VHF and HF. | This task should be coordinated through WP4C/SG4 under existing question 227/4. |
| LRIT | HF |  |  | The use of HF and Satellite data links to support the LRIT application should not need a new technical characteristic as they are simply applications supported by existing data links. However, there may be merit in amending the existing recommendations in relation to HF and satellite communications to explicitly recognise the use of LRIT. | The subject is on the WRC-11 agenda under Item 1.10. However, there is unlikely to be any directly relevant outcomes. The work would therefore have to be addressed through WP4C/SG4 Questions 84-4/4 (Iridium), 87-4/4 (satellite) coordinating with WP5B/SG5 for HF. |
|  | Satellite |  |  |  |  |
| AIS |  | ITU-R M.1371-3 | Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile band | There are a number of potential changes to AIS under e-Navigation that would require revision to the existing technical characteristics. Firstly, the possible need for a dedicated channel to enable the detection of AIS transmissions from space. Secondly the allocation of additional channels to provide extra capacity for e.g. environmental or security related messaging. Thirdly the possible need to define new AIS messages such as those relating to channel management. Fourthly, the possible use of the AIS data link as a candidate technology for future maritime VHF data communications could also be a possibility. | WRC-11 agenda item 1.10 is considering the expanded use of VHF for port safety and security. Under this item the allocation of a 3rd AIS channel for detection from space is being considered. As a result of this item there could be a need to revise technical characteristics. This task would likely fall to WP5B/SG5 under existing questions 232/5 and 96-2/5. |
| VHF Comms | Voice | ITU-R M.1084-4 | Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service | Under e-Navigation a general push to make more efficient use of the spectrum available for maritime voice communications may take place as existing channels are increasingly allocated to digital services. The existing characteristics (e.g. M.1084) propose possible interim solutions such as the use of 12.5kHz channel spacing. It may be desirable to develop a new technical characteristic to describe a permanent solutions for e.g. 12.5 kHz or 8.33 kHz voice channels in the future communication environment. | This task should be coordinated through WP5B/SG5 possibly under existing question 96-2/5. |
|  |  | ITU-R M.489-2 | Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz |  |  |
|  | Data | ITU-R M.1842-1 | Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels | A new technical characteristic will be required to support the use of VHF channels for digital data transmission – assuming that AIS is not selected as the maritime domain datalink of choice. The current characteristic for VHF data exchange refers to the specific narrowband solutions implemented and proposed by Telenor in Norway and US respectively. Also, it refers to the wideband solutions using two or four 25 kHz channels providing 153.6 kbps and 307.2 kbps respectively proposed by US based on the ETSI standards. If this solution is accepted by the maritime community this characteristic could be amended. However, in the more likely scenario of a new datalink technology being proposed a new characteristic will be required. | This task should be coordinated through WP5B/SG5 possibly under existing question 96-2/5. |
|  | SAR | ITU-R M.489-2 | Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz | No e-Nav related comms change is anticipated for the use of 121.5 MHz for short range SAR purposes. However, were the VHF channels to be revised the SAR associated characteristic (M.489) would in any event be updated. | N/A |
| HF Comms | Voice | ITU-R M.1173 | Technical characteristics of single-sideband transmitters used in the maritime mobile service for radiotelephony in the bands between 1 606.5 kHz (1 605 kHz Region 2) and 4 000 kHz and between 4 000 kHz and 27 500 kHz | No e-Nav related comms change is envisaged for HF voice services. However, if there is a broad encroachment of digital techniques into the band there may be a need to revise existing voice characteristics to accommodate data. | N/A |
|  |  | ITU-R M.1082-1 | International maritime MF/HF radiotelephone system with automatic facilities based on DSC signalling format |  |  |
|  | Data | ITU-R M.1081 | Automatic HF facsimile and data system for maritime mobile users | There are a number of existing technical characteristics relating to data exchange in the HF band that include both outmoded legacy technologies as well as candidate future technologies. There needs to be either an update to M.1798, or else a new characteristic to describe the selected HF digital data scheme to be used under e-Navigation. Currently M.1798 and F.1821 describes multi-channel HF data links such as the system currently deployed by Globe Wireless for commercial services (email, etc.). | The HF channelling requirements are being addressed within WRC-11 agenda item 1.9. To solve the agenda, a new characteristics in relation to the e-Navigation preferred HF datalink will be developed. See WP5B/SG5 and questions 145-2/5 and 158-1/5. |
|  |  | ITU-R M.1798 | Characteristics of HF radio equipment for the exchange of digital data and electronic mail in the maritime mobile service |  |  |
|  |  | ITU-R M.625-3 | Direct-printing telegraph equipment employing automatic identification in the maritime mobile service |  |  |
|  |  | ITU-R M.627-1 | Technical characteristics for HF maritime radio equipment using narrow-band phase-shift keying (NBPSK) telegraphy |  |  |
|  |  | ITU-R F.1821 | Characteristics of advanced digital high frequency (HF) radiocommunication systems |  |  |
| NAVTEX / NBDP | MIO | ITU-R M.625-3 | Direct-Printing Telegraph Equipment Employing Automatic Identification in the Maritime Mobile Service | If under e-Navigation the NAVTEX data is carried across different communication links there may be a need to provide a new characteristic to accommodate it. However, if it is intended to include NAVTEX data along with other safety related information e.g. Maritime Information Objects, etc. then it may be more appropriate for a specific characteristic to encompass all maritime safety information. | This task should be coordinated through WP5B/SG5 possibly under existing question 98-5. |
|  |  | ITU-R M.540-2 | Operational and Technical Characteristics for an Automated Direct- Printing Telegraph System for Promulgation of Navigational and Meteorological Warnings and Urgent Information to Ships - Section 8B - Maritime Mobile Service; Telegraphy and Related Subjects. |  |  |
|  |  | ITU-R M.688 | Technical characteristics for a high frequency direct-printing telegraph system for promulgation of high seas and NAVTEX-type maritime safety information |  |  |
|  |  | ITU-R M.492-6 | Operational procedures for the use of direct-printing telegraph equipment in the maritime mobile service |  |  |
|  |  | ITU-R M.476-5 | Direct-Printing Telegraph Equipment in the Maritime Mobile Service |  |  |
| On board Comms | UHF | ITU-R M.1174-2 | Technical characteristics of equipment used for on-board vessel communications in the bands between 450 and 470 MHz | There is a need to update the existing characteristics to support the implementation of digital PMR. | This task should be coordinated through WP5B/SG5 most likely under a new question. |
|  | WiFi | Rep. ITU-R F.2086 | Technical and operational characteristics and applications of broadband wireless access in the fixed service | If WiFi stations are to be adopted for on-board data communications there may be a need for a new characteristic associated with is use in the maritime mobile domain. | This task should be coordinated through WP5B/SG5 most likely under a new question. |
| Satellite Comms | Broadband | ITU-R S.1709 | Technical characteristics of air interfaces for global broadband satellite systems | Broadband over satellite is an existing application of current satellite systems and does not warrant a specific characteristic for the maritime domain | N/A |
|  | MSI | ITU-R S.1709 | Technical characteristics of air interfaces for global broadband satellite systems | The transmission of MSI over broadband may warrant a characteristic to define the data layers. | This task should be coordinated through WP4C/SG4 under existing question 87-4/4. |
| DGNSS |  | ITU-R M.823-3 | Technical characteristics of differential transmissions for global navigation satellite systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3 | The current characteristic M.823-3 will ultimately need to be updated to accommodate new developments in GNSS such as Galileo and COMPASS. | This task should be coordinated through WP5B/SG5. |
| Port Services | WiMax / 2G / 3G / 4G |  |  | The maritime community will make use of technologies developed and standardised for other applications in port. No maritime specific characteristics are required. | N/A |
| Inter-ship Broadband | WiMax | ITU-R M.1801 | Radio interface standards for broadband wireless access systems, including mobile and nomadic applications, in the mobile service operating below 6 GHz | Under e-Navigation ad-hoc mesh WiMax networks could be deployed to provide broadband coastal networks. This would require a specific form of implementation of WiMax that would necessitate a specific technical characteristic. | This task should be coordinated through WP5B/SG5 possibly under existing question 212-3/5. |
| Loran | eLoran data channel | ITU-R M.589-3 | Technical characteristics of methods of data transmission and interference protection for radionavigation services in the frequency bands between 70 and 130 kHz | The existing technical characteristic will need to be revised to accommodate the definitive 9th pulse, Eurofix and/or new modulation schemes selected for the eLoran data channel. | This task should be coordinated through WP5B/SG5 under a new question. |
| Radar | SART | ITU-R M.628-4 | Technical Characteristics for Search and Rescue Radar Transponders | Relates to 9 GHz SART, so no change required. | N/A |
|  | Racon | ITU-R M.824-3 | Technical parameters of radar beacons (Racons) | No e-Nav related comms change. | N/A |
|  | Radio determination | ITU-R M.1460 | Technical and operat ional characteristics and protection criteria of radiodetermination radars in the 2 900-3 100 MHz band | No e-Nav related comms change. | N/A |
| EPIRB | VHF | ITU-R M.690-1 | Technical characteristics of emergency position-indicating radio beacons (EPIRBs) operating on the carrier frequencies of 121.5 MHz and 243 MHz | No e-Nav related comms change. | N/A |
|  | AIS | ITU-R M.1371-x | Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile band | New annex to M.1371-3 will be required for the EPIRB-AIS which will be introduced by IMO. |  |
|  | Satellite (COSPAS / SARSAT) | ITU-R M.633-3 | Transmission characteristics of a satellite emergency position-indicating radio beacon (satellite EPIRB) system operating through a satellite system in the 406 MHz band | No e-Nav related comms change. | N/A |
|  | Satellite | ITU-R M.632-3 | Transmission Characteristics of a Satellite Emergency Position Indicating Radio Beacon (Satellite EPIRB) System Operating Through Geostationary Satellites in the 1.6 GHz Band | No e-Nav related comms change. | N/A |