e-NAV9 Input paper

Agenda item	08 (WG 2)

Task Number	01, 02 and 03

Author	Nick Ward

This input has been provided by the General Lighthouse Authorities as a possible contribution to an IALA study on resilient position-fixing.

Options for Resilient Position-Fixing

Purpose of Document

The IMO Maritime Safety Committee has stated:

“e-Navigation systems should be resilient and take into account issues of data validity, plausibility and integrity for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems should be considered.”

(IMO MSC 85/26)

Discussions in the e-Navigation Working Group at NAV 56 in July 2010 identified redundant positioning as an aspect of the Gap Analysis to be considered further at NAV 57. This document condenses the results of a comprehensive study of alternative positioning solutions, carried out in the UK, which could be used to inform the discussions at NAV 57.

Vulnerability of satellite navigation

It is recognised that Global Navigation Satellite Systems (GNSS) will be the primary position input for e-Navigation. However, GNSS on vessels is vulnerable to space-vehicle failures, solar disturbances, unintentional radio interference and, increasingly, deliberate jamming. Some examples are given here.

No marine accidents to date have been attributed to loss of GNSS. However, the growing use of ECDIS increases the reliance on positional data from GNSS and therefore the probability of a related accident.

Problems in GNSS reception are relatively common in a number of areas, such as the Golfo di Genova, the Golfo di Napoli and in the Golfe du Lion. The problems in the three named areas are known to be caused by interference from certain high power TV transmitters. Problems have also been reported in areas where high power military radars are in use.

On 1 January 2004, the failure of an atomic clock in a satellite caused ships’ GPS receivers in European waters to display serious position errors. There was no warning before incorrect positions appeared that were plausible, misleading and dangerous. Later these errors built up to between 10km and 40km. They affected all vessels that did not take advantage of the integrity service provided by lighthouse authorities’ IALA DGPS beacons, or other augmentation services. This type of failure should be effectively resolved by the inclusion of RAIM, when fitted, which has been implemented on some new equipment since 2002.

On 6 December 2006, a solar flare caused the Sun to send out radio noise so intense that GPS receivers stopped working across the entire sunlit side of the earth. Such solar cycle events are expected to be more frequent for the next few years. They will affect all GNSS including Galileo, China’s new Compass system and Russia’s revitalised GLONASS.

The principal vulnerabilities of GNSS result from the extremely weak signals from the satellites that reach the user’s receiver on earth. A satellite transmits no more power than a car headlight, yet must illuminate nearly half the earth’s surface from 20,000km away. Natural noise or man-made interference can easily swamp the received signal. When electronic equipment radiates interference accidentally, all GNSS users in range can be affected. There have been several documented examples of this type of accidental interference.

Now GPS is increasingly being jammed intentionally. Low-power transmitter devices are on retail sale for well under £100. They are deployed by hackers and also by drivers who wish to prevent their employers from monitoring their activities. Criminals have been convicted for using such jammers to overcome GPS security systems and so steal vehicles and their loads. Devices in the hands of the public include GPS jammers with ranges of tens of kilometres, mobile telephone blockers and jammers that will attack the next-generation of GPS, plus Galileo and all known future GNSS. How effective will such devices be at sea?

In 2007, vessels in the harbour of San Diego suddenly lost GPS service for two hours, when a jammer used for naval trials was accidentally left on. In the city, more than 100 mobile telephone sites failed. The communications of first responders to the incident and other emergency services were affected.

In 2010 GPS was lost along 300 km of coastline, in an Asian country, for significant periods over several days, as a result of a jamming signal from an adjacent country.

Recently, the impact of low-level GPS jamming has been explored under controlled conditions off the coast of the UK. A jammer of less than one thousandth the power of a mobile phone, on board a ship, easily and frequently induced false GPS positions and velocities with no alarm indications. The ship’s position on the chart display would silently begin to depart from the truth, in ways that bridge officers would possibly not notice. This is a very dangerous phenomenon.

When the jamming power was increased slightly, multiple alarms sounded in various parts of the bridge, causing confusion. The following systems all failed concurrently: the main Electronic Chart Display and with it the ship’s autopilot; the precise differential GPS system; the heli-deck stabilisation system; and the Digital Selective Calling (DSC) and GMDSS (Global Maritime Distress Safety System). In addition, the principal navigation aids on which officers depend for navigation in the absence of GPS also showed alarms: the ship’s radar; the gyrocompass; as did the Automatic Identification System. For a small bridge team in poor visibility, safely maintaining navigation whilst dealing with multiple alarms, and thereafter continuing the voyage without these systems, would have been challenging if not impossible.

A Range of Approaches to Resilient PNT

Resilience can be defined as “…the ability to withstand or recover quickly from difficult conditions….” Systems may be made resilient by building-in, or otherwise providing, standby capacity or by switching to alternative operational procedures.

GNSS itself will become more resilient as multiple satellite constellations are deployed, each with its independent monitoring and control. Augmentation systems (Space-Based or Ground-Based Differential GNSS) or Receiver Autonomous Integrity Monitoring (RAIM) can enhance integrity, ensuring that Position, Navigation and Timing (PNT) information is correct, or immediately warning the user when it is not. However, these approaches do not provide a solution when solar noise, or man-made interference or jamming, affect all GNSS signals and the augmentations themselves may also be affected.

Alternative systems with dissimilar failure modes can provide integrity and protect against these hazards. Radar systems are not susceptible to solar activity or jamming at GNSS frequencies, although they can be degraded by multipath and clutter effects.  Some maritime radar equipment also uses information derived from GNSS. Low-frequency terrestrial systems such as eLoran, with their higher signal levels and larger antennas, are much more resilient to interference and jamming. Like satellite systems, they are occasionally affected by solar activity although not at the same time, since the causes are different. The data channel of eLoran warns the user of such events.

Onboard inertial navigation systems provide independent measurements of a ship’s motion. Inertial systems with the long-term stability to act as a backup for a useful period are much too costly for civil use, though developments in solid-state sensors and digital processing will eventually reduce prices. Another potential solution in the longer term is geo-magnetic mapping.

Resilience of PNT information on ships requires dissimilar systems, optimally combined. The study on which this paper is based considered all potentially available solutions and arrived at a ‘short-list’ of four options, selected according to the judgement of a pool of independent experts. This short-list identifies combinations of GNSS with various complementary technologies or dissimilar systems. The number of physical AtoN required to support each option was determined based on the navigational requirements.

Options identified

Options for the future of maritime Aids-to-Navigation within an e-Navigation environment, have been developed for a study in the UK, through a series of workshops with prominent national experts in the fields of maritime systems and navigation.

Four options were identified, which were credible within the timescale for the introduction of e-Navigation:

- Option 1 – Do minimum;

- Option 2 – Maximise the current infrastructure;

- Option 3 – Hardening of GNSS;

- Option 4 – eLoran implementation.

Option 1 ‘Do Minimum’

The first option represents a broad continuation of the current provision, using existing approaches and technologies to mark hazards, channels and traffic separation schemes. In this option IMO’s e-Navigation concept is assumed to be implemented as expected by 2018-2020 and is predicated upon existing technologies. It is expected that the majority of SOLAS vessels will be equipped with standards-compliant ECDIS by 2018 and will over time install NT radar systems and multi-constellation GNSS receivers, as their existing equipment needs replacing.

Option 2 ‘Maximise Current Infrastructure’

The second option recognises the current trends in maritime radar equipage and assumes an expansion or enhancement of the physical AtoN infrastructure to support a fallback mode of radar positioning. Again it is assumed that IMO’s e-Navigation concept is implemented by 2018-2020, all SOLAS vessels equipped with ECDIS by 2018 and over time, multi-constellation GNSS receivers. In this option NT radar systems will be installed by 2018 and will be capable of supporting vessel absolute positioning in the coastal and harbour environment, providing latitude and longitude output to the ECDIS for use in the event of a loss of GPS position. This would require some cooperative action on the part of radar manufacturers, possibly stimulated by regulatory changes.

Recognising the dependence of users upon GNSS-based navigation, some initiatives would be taken by shore-based authorities to mitigate the risk as the mariner navigates closer to danger due to misplaced confidence in vulnerable GNSS.

- The first would be to deploy an enhanced infrastructure of radar AtoN (corner reflectors or target enhancers) at appropriate points around the coastline, to support NT radar positioning as a key component of integrated navigation that provides resilient PNT for e-Navigation.

- The second activity would be to extend the introduction of new physical AtoN such as synchronised lights and to take measures to enhance conspicuity of lights.

As welI as additional buoys to mark expanding offshore installations for renewable energy, marking of new channels, hazards and traffic separation schemes may be necessary.

Option 3 ‘Hardened GNSS’

The third option represents a concerted effort to reduce the risks of complete dependence upon GNSS by the maritime community for the implementation of e-Navigation. Actions would be undertaken to protect the GNSS infrastructure against the effects of interference (so-called ‘hardening’). This would include installation of multi-constellation GNSS receivers by 2020, incorporating more robust testing for the effects of interference, with alerting when interference is detected.

Recognising this dependence on GNSS based navigation, the infrastructure would need upgrading, enhancing the capabilities of IALA DGPS to support multiple GNSS systems and additional frequencies. There would also need to be support for the standardisation of more robust user GNSS receiver equipment and its safe integration into future bridge systems.

This option would require action from maritime safety authorities and telecommunications regulators to ensure adequate means to deter, detect and respond to GNSS interference events. Reversionary procedures would need to be developed by maritime authorities, VTS operators and shipping organisations, through IMO, for mariners in the event of potentially hazardous GNSS interference.

Option 4 ‘eLoran in the maritime domain’

The fourth option represents the provision of an alternative position-fixing system by maritime authorities, complementing GNSS to ensure resilient PNT.

The IMO’s e-Navigation concept would be implemented as expected by 2018-2020 predicated upon robust PNT sourced from GNSS and eLoran. All SOLAS vessels will be equipped with standards-compliant ECDIS by 2018 and eLoran/GNSS receivers by 2020.

eLoran implementation activities would lead to Full Operational Capability (FOC) in 2018. FOC would ensure a resilient PNT service for harbour and coastal operations around all coastlines where ‘the volume of traffic justifies and the degree of risk requires’ (SOLAS Chapter V).

As a consequence of the reliability of PNT and the resulting robustness of e-Navigation, it would be possible for shore-side authorities to rationalise their lights infrastructure. Passage lights around the coastline not marking hazards could be withdrawn and sold or transferred. The lights for withdrawal would be based on sound navigational principles. Remaining lights would be replaced using low cost technology, independently mounted, or mounted on existing structures if a day mark is required. These changes could result in substantial income and savings.

Economic appraisal of options

The approach taken to the economic appraisal was to perform a discounted cash flow analysis of the costs and benefits of each individual option. Comparisons were then made to the ‘do minimum’ option (Option 1). This required the identification and valuation of all of the relevant costs and benefits that fall to all of the maritime stakeholders within each option.

A conservative approach was taken to evaluate the economic case so that the results were unbiased and credible for all options. In this connection, the main economic benefits were assumed to be the possibility of reducing the provision of physical AtoN (lighthouses and buoys) as a result of confidence in the resiliency of electronic position-fixing. Estimates of savings associated with the removal of lights incorporate the conservative assumption that the overall cost of provision of lights (capital, operational and overhead costs) would be subject to efficiency savings (2.5% p.a.) irrespective of the option pursued.

In some of the options, the primary benefits arise from avoided expenditure. These costs and benefits were valued over the transition to, and the minimum lifetime of, the eLoran service (or alternative). The timing of the costs and benefits was established and the resulting figures discounted.

Subsequently the Net Present Value (NPV) of each option was ascertained. This enabled comparison with the ‘do minimum’ option. A number of parameters (costs, benefits and timing) were varied in order to understand the sensitivity of the results to specific assumptions.

General assumptions

It was assumed within a number of options that the maritime users equip with new capabilities in order to receive the benefits of e-Navigation. It was assumed that only those costs incurred specifically due to each option were to be valued. For example, ECDIS equipage was assumed across all scenarios (including the ‘do minimum’ option) and was therefore not valued.

Widespread fitting of enhanced equipment was assumed, in order to enable the full benefits of e-Navigation. In practice this would mean that most vessels transiting the waters in which e-Navigation is implemented as well as those making port calls, would have the required equipment. The appraisal has not attributed the costs of equipage of all international maritime users in this study, which was carried out for a limited region.

In addition, the full cost of new equipment was not attributed. Instead, the analysis used the cost delta between the basic capability that the vessel owner would ordinarily equip with and that of the more advanced receiver. The cost delta per receiver was estimated to be £200 for multi-constellation GNSS and £500 for eLoran/GNSS on SOLAS vessels and £100 and £250 respectively on non-SOLAS vessels. This figure was assumed to decrease over time until in 10 years’ time there would effectively be no cost delta as the new receiver becomes the de facto standard. This model is based on experience of the cost profile of GPS receivers as new capabilities are introduced.

Preferred option on economic grounds

The economic analysis of the four options developed within the business case process resulted in only one option (Option 4) that produced a positive NPV over the duration of the analysis. Furthermore, neither of the other options resulted in a better NPV than the ‘do minimum’ option.

The preferred option on purely economic grounds would be Option 4 (Maritime eLoran). The benefit-to-cost ratio of the preferred option was in excess of 2:1.

Sensitivity analysis of Option 4 showed that the benefits are robust to significant changes in its underpinning assumptions. It is most sensitive to changes in the number and timing of the lighthouse rationalisation programme. It is not particularly sensitive to variations in the cost of implementing eLoran, either the core system or the necessary vessel equipage. In addition, the likely extension of the system life beyond 10 years greatly enhances the overall benefits of the option.

Additional commercial benefits

- In addition to the core service of eLoran transmissions, three further areas (included within Option 4) could provide important benefits:

- Maritime-specific Differential Loran (DLoran) services, establishing and operating DLoran reference stations, and distributing DLoran information for transmission over the eLoran data channel could be provided. A precedent for the DLoran service has been set by existing DGPS services, provided with the support of commercial providers.

- Additional Secondary Factor (ASF) surveys could be conducted by commercial contractors and ASF data collection and processing could be provided commercially.

- Availability of affordable combined GNSS/eLoran receivers for equipage of ships is likely to accelerate as the eLoran service is established and market demand increases. An increasing market volume is envisaged, driven by demand from all classes of vessels, not just the SOLAS fleet. A competitive market for eLoran equipment should exist by the mainstream equipage of vessels around 2018–2020.

Complement to GNSS to achieve resilient PNT

The ideal maritime complement to GNSS would:

- enable resilient PNT for use by critical infrastructure applications including maritime transport.

- be readily integrated with GNSS at chip-level.

- support interference detection and mitigation.

- maintain the user’s concept of operations with a seamless transition to a complement when GNSS is lost.

- have the potential to be deployed world-wide.

- support maritime general navigation applications.

- be independent of GNSS.

- be dissimilar in terms of failure modes.

- provide similar levels of performance to GNSS.

Only eLoran seems likely to meet these requirements in the timescale envisaged for the introduction of e-Navigation.

Conclusions

On the basis of the economic analysis, the net financial benefits and ongoing savings of implementing eLoran in the UK case would be substantial. Even allowing for uncertainty, there is persuasive analytical support for extending this case to other regions.

It has been shown that when all the building blocks of e-Navigation are in place, with fully resilient PNT it may be possible to significantly reduce expenditure on conventional AtoN, although removal of lights would need to be based on sound navigational principles, ensuring retention of those lights that safeguard the mariner’s local situational awareness.