| From: VTS Committee | e-NAV9/10/7 Formerly VTS32/output/14 |
| --- | --- |
| To: e-Navigation Committee | 11 March 2011 |

Liaison Note

Specify VTS and other VTM related user needs in relation to the allocation of the radio frequency spectrum, for further delivery to the IMO and ITU

# Introduction

During VTS31, the VTS Committee commenced work on Task 17 regarding the VTS/VTM radio frequency spectrum user needs.

The VTS Committee produced technical content related to this topic at Annex A. Considering the importance of the subject and the intended recipients (IMO and ITU), the e-Navigation Committee is requested to consider the user needs identified in Annex A.

# Action requested

The e-Navigation Committee is requested to:

- consider the identified user needs

- where necessary, extend and finalise the paper

- submit the paper to IMO and ITU such that it can be taken into consideration before the next WRC

- VTS / VTM related user needs in relation to the allocation of the radio frequency spectrum, for further delivery to the IMO and ITU

11 March 2011

# Introduction

General

Due to the increased demand for radio frequency spectrum, some countries are promoting the more efficient use through the re-allocation of spectrum together with new pricing regimes and lower power levels. The existing provision of VTS relies on the use of allocated spectrum for the following services/bands:

- VHF – short range voice, DSC and AIS communications

- MF – medium range voice, telex, DSC, Differential GNSS and Maritime Safety Information (Navtex)

- HF – long range voice, telex  and DSC

- UHF including Satcom – global data, voice, LRIT, GNSS, Satellite AIS

- Radar – safety and navigation

The necessity and sufficiency of the existing VTS and e-Navigation allocations is discussed in the following paragraphs

# Discussion of existing VTS and e-Navigation Spectrum allocations

## VHF

The VHF band is primarily used for shore/ship and ship/ship voice communications, DSC, AIS, radio direction finding and the Automatic Transmission Identification System (ATIS) for inland waterways.  In addition voice communications between adjacent VTS centres, between VTS centres and allied services, shore/ship and ship/ship communications in coastal areas utilise the VHF band.

Currently there is a shortage of International Maritime Mobile (IMM) VHF simplex channels specified for ship shore use.  The resulting interference can impact on navigational safety.  A number of VTS stakeholders consider that the current allocations have an unnecessarily large number of channels devoted to duplex use for which a solution may be to reassign some duplex circuits to simplex.  In the longer term a more radical approach should be considered e.g. to use digital technologies such as trunked networks to achieve greater spectrum efficiency.  This approach, however, is not expected to free up band width for other non-maritime users.

DSC is used to support distress and calling within GMDSS.  The carriage of GMDSS equipment is mandated under SOLAS legislation as a safety-related system.

AIS was conceived as a collision avoidance system that is also an automatic means of identifying shipping and is heavily relied upon  within the VTS solution for control and monitoring of vessels. AIS Channels 1 and 2 are internationally mandated.  AIS is used for the transmission of safety-related messages.

The ATIS system is used predominantly on inland European waterways to identify vessels.  It makes use of existing VHF channels.

## MF

The MF band is used for medium range voice communications (shore/ship and ship/ship communications), telex, DSC, Differential GNSS and Maritime Safety Information (Navtex) transmissions.

These essential safety related functions contribute to the safe passage of vessels within the VTS area.

## HF

The HF band is used for long range voice, telex and DSC ship/shore and ship/ship communications.

## UHF including Satcom

Satellite communications are used for global voice, data (including telex, LRIT and Maritime Safety Information) and GNSS (GPS, GLONASS and Galileo) positioning.  Examples include the provision of pre-arrival information to the VTS authority to aid berth planning.

Shore-based microwave links connect a remote radar site to a VTS centre when required (due to topography).  Allocation of bandwidth for this purpose is on the basis of regulator and local approvals.

## Radar

Radar is the primary sensor technology for detection and positioning of vessels to aid safe navigation within a VTS area. Typically X-band is favoured for VTS applications although some site conditions favour the use of S-Band (e.g. in areas susceptible to heavy precipitation).

Current marine radar spectrum allocations appear to operate with minimal problems relating to emissions affecting adjacent spectrum users or susceptibility arising from adjacent spectrum transmissions.  However planned changes to the spectra adjacent to the existing marine bands are expected to introduce new challenges to the existing marine radar technologies.  In addition the likely increase in density of radar emissions in areas of heavy traffic creates new challenges to VTS and maritime radar solutions.

New pulsed technology radars in both X and S band offer reduced transmission bandwidth allowing significant reductions in unwanted interference between radars (on land and shipboard) and with other users in adjacent allocated bandwidth.

Today’s VTS X-band radars mainly operate in the 9.225 – 9.5 GHz region. Increased use of the full band allocated 8.8 – 9.5 GHz (excluding some sub-bands reserved for Air traffic and Meteorological use) may be required in order to avoid congestion and interference.

The Ka – Ku band is also in use for VTS radar applications for vessel management and to support infrastructure protection (unwanted intruder detection).

Short range millimetre wave radar is used for berthing systems etc.

# Future VTS Requirements

The maritime industry (not just VTS) has identified additional spectrum requirements which may influence the decisions being made for bandwidth allocation in the future.  Some of these are described below.

- Reliable voice (and data) communications in VTS polar areas where existing types of communication are not always available / applicable. Satellite communications or other technologies could be considered.

- The adoption of digital voice communication techniques within the existing VHF spectrum maritime allocation is necessary to counter existing problems experienced in high usage areas, however adoption of such new technology across the whole maritime industry requires careful planning to maintain an acceptable level of service in all VTS locations. It is not envisaged that this will affect other non-maritime spectrum users.

- Increased requirements placed on VTS authorities throughout the world may require additional shore/shore data links to support distributed VTS facilities in a given area. This should continue to be managed at national (as opposed to global) levels. Special attention should be given to spectra used and capacity of microwave links for future ISPS/VTM/e-Navigation applications. In addition, consideration needs to be given to the adoption of mobile technology (WiFi / WiMax / 3G or dedicated bandwidth) in aid of VTS operations such as the use of portable devices to display the overall VTS picture (to pilot / harbourmaster, back-up VTS centre etc.).

- There is the possibility of a terrestrial back-up to existing satellite based navigation systems such as eLoran which builds upon the historic long wave system – further information should be sought from the e-Navigation community.

- National security objectives and enforcement of laws (territorial fishing for example) may see the adoption of long range radars to facilitate early vessel detection from coastal sites. Information relating to suitable frequency spectra for such technology needs to be sought from more appropriate bodies.

- HF transmission of security related messages from the VTS authorities to vessels.