AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No 1060

On

Recapitalisation of DGNSS

Edition 2

March 2011

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr    Internet - www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1	Introduction	4

2	Replacement Options	4

3	Progress to date	4

4	Guidance	4

5	References	5

ANNEX I	Status of DGNSS Recapitalisation within IALA Members (EARLY 2011)	1

Guidance on the recapitalisation of DGNSS

# Introduction

IALA has recommended that members operating the beacon DGNSS service, which is the international standard for maritime augmentation, should consider modernisation to ensure that levels of service can be maintained and future requirements can be met (Ref 1).  This recommendation took into account various potential alternatives, including Satellite Based Augmentation Services (SBAS) and the Automatic Identification System (AIS).

These Guidelines should be viewed in the context of the development by IALA of a World Wide Radio Navigation Plan (WWRNP) in support of e-Navigation.  One key concept in this Plan is the separation of the generation of correction data from the means of transmission, to facilitate broadcasting by a variety of methods.  This could lead to the integration of terrestrial systems (DGNSS beacons, eLoran, AIS) to provide shared data channels and common correction sources, as well as additional ranging signals, contributing to a redundant position-fixing solution, complementary to, but independent of GNSS.

Recapitalisation of DGNSS should be considered in the context of position-fixing requirements for e-Navigation, noting that DGNSS is likely to be an essential part of the WWRNP.

# Replacement Options

The simplest recapitalisation option may be replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM), but this may limit the potential for future development.  Other possibilities include:

- Software Reference Stations and Integrity Monitors (RSIM);

- Virtual Reference Station (including the option of incorporating land survey network data;

- SBAS Integration.

These options are explored further in IALA Recommendation R-135.

# Progress to date

IALA members are currently at different stages of developing varying solutions based on their individual requirements, timelines, geographical coverage area, size of network and budgetary constraints (See Annex A).  All of these solutions will meet the needs of existing users.

# Guidance

IALA members are encouraged to:

- Recapitalise existing DGNSS systems prior to their obsolescence, noting the advices given in R-135 and R-121;

- Consider the options for replacement systems, with reference to the documents listed below;

- Share information regarding studies carried out and specifications of replacement solutions, and report progress to IALA;

- Investigate potential future developments to be compatible with the development of shore based e-Navigation architecture;

- Investigate techniques for adding value to transmissions, such as transmission of safety related information and/or backup positioning capabilities.

# References

- IALA Recommendation R-135 on The Future of GNSS

- DGNSS Modernisation: Recapitalisation of the GLA’s DGPS Network (A.Grant, N.Ward, M.Pattinson, M.Dumville), NAV07, Royal Institute of Navigation, 2007

- Invitation to Tender For the Recapitalisation of the Differential Global Positioning Service (DGPS) operated by the General Lighthouse Authorities of the United Kingdom and Ireland, March 2008

- German Federal Waterways and Shipping Administration.  Contribution to the IALA World Wide Radio Navigation plan, IALA e-NAV 4, February 2008.

- Swedish Maritime Administration. Report of VRS Workshop held in Malmö February 2010. Input to IALA e-NAV 8. September 2010.

- German Federal Waterways and Shipping Administration. Installation of a VRS-Test Pilot for the transmission of GNSS corrections using MF radio beacons and 
AIS base stations. Input to IALA e-NAV 8. September 2010.

- Status of DGNSS Recapitalisation within IALA Members (EARLY 2011)

- The US has implemented software RSIM equipment across all USCG and Inland Waterways transmitter sites. Trials of High-accuracy Nationwide DGPS are continuing.

- The UK and Ireland conducted a study of replacement options which concluded that the lowest risk option for the GLAs would be hardware replacement, although the flexibility provided by the software option would better meet emerging requirements and should not be ruled out (Ref 2).  The GLAs have tendered for new RSIM equipment, which may be fulfilled by either hardware or software solution (Ref 3).

- Germany is planning to implement a first realisation of VRS, based on the separation of generating differential corrections from signal transmission, which could use MF beacon and/or AIS carriers.  Germany is also planning a feasibility study to investigate the addition of Ranging signals (R-Mode) on MF beacon and AIS carriers (Ref 4).

- Australia is investigating software RSIM solutions, and adding another 2 stations to their service.

- Malaysia has recapitalised their service and increased the number of stations in 2008.

- Vietnam implemented a service in 2009.

- Poland has replaced their RSIM equipment with an L1/L2 hardware solution, remote control and plans to transmit VRS data via MF beacons in the future.

- Sweden is currently studying options for software RSIM solutions and  plan to implement a VRS station in 2011, subject to budget.

- China is expanding coverage with the construction of three additional stations in 2008.

- The Netherlands have a requirement to replace equipment in 2011, and are considering both hardware and VRS solutions.

- Denmark installed replacement hardware in 2009.

- Finland plans to implement a VRS station in 2011.

- Other administrations, including those of Canada, France, Portugal, Norway and South Africa are currently studying options for recapitalisation.

- The number of administrations providing DGNSS services is continuing to increase. coverage and monitoring facilities.