e-NAV9	Input paper

Agenda item	8 (WG2), 7 (WG1), 9 (WG3), 10 (WG4), 11 (WG5), 12 (WG6)

Task Number	as appropriate

Author(s)	J.-H. Oltmann, E. Engler, R. Ziebold, M. Hoppe

Development of a Maritime Integrated PNT System

ADDENDUM.1: Graphical representation of Integrated PNT System and the PNT Unit as a future INS Module

# Summary

This document illustrates the main document entitled “Development of Maritime Integrated PNT System” by a picture (attached): “A picture says more than a thousand words.”

While the attached drawing describes the Position, Navigation, Timing (PNT) domain, it should be noted, that the architectural principles indicated in the picture as applied to the PNT domain may be transferred similarly to other envisaged INS modules. Some examples, relevant to IALA e-Navigation Committee, are given, such as the COM Module or the AIS (Next Generation) Module.

# Explanation of the Picture attached

The attached picture shows in an architectural fashion the arrangement of the different entities described in the main document. The attached picture thereby graphically defines the entities:

- The overarching Integrated PNT System: This is shown to the right hand side of the picture and the starting point for digesting the picture. The Integrated PNT System clearly spans both space-borne (GNSS!), shipboard (PNT Module of the INS), and shore-based (terrestrial augmentation and terrestrial backups) entities.

- The shipboard Integrated Navigation System (INS) as the architectural framework for “plug-in” modules such as the envisaged PNT Module, the envisaged COM Module, the envisaged AIS (Next Generation) Module, and so forth. This is shown in “zoom-in” fashion for the shipboard side.

  - It should be noted that the “Alarm Management Module” is already defined in the present IMO INS Performance Standards (PS). Each and every other module must have a data output to the “Alarm Management” Module, indicated by the “A” output.

  - It should be noted, that a dedicated “Presentation Surface Module” (working title) would be required to act as the Human-Machine-Interface of the INS at large and to present data from all modules to the mariner in a harmonized (and maybe integrated) fashion.

- The architecture of the PNT Module as such: This part of the picture shows – again in “zoom-in” fashion – the functional architecture internal to the PNT Module, and in particular the so-called PNT (processing) Unit. A functional “data bus” is shown, which symbolizes the connectivity of the PNT (processing) Unit to different “front end” entities/devices such as receivers. The different “front end” entities/devices may be considered more “modem-like”, i.e. providing “more raw” data, than the present

Some important features should be noted throughout the picture:

- The bluish bars at the interface position of the entities symbolize the “product specifications” which govern the functionality of the associated entity and which define the functional interface of that entity, i.e. they define what functionality and data the entity is expected to deliver at its functional interface.

- Since IMO has asserted “governance”, it is most likely that this “product specification” may be – on the higher most level – represented in IMO PS:s which in turn reference the appropriate “product specifications” of appropriate international organisations dealing with more detail.

- It should be noted, that the architecture shown in the picture is to be construed functional; it does not prescribe or preclude any specific technical implementation as long as it does fulfil the envisaged functionality.

- There may be an architecture governing the relationship of the different modules within the INS: This is not shown expressively in this picture, but implied by the “special” INS Modules for “Alarm Management” and “Presentation Surface”. Also, the internal functional architecture of a module, in particular the distinction between “front-end” and “processing” entities which are inter-connected by the functional data bus give rise to that notion.

- The envisaged architectural setup may result in the revision of existing IMO PS:s and of existing “product specificiations” of relevant international organisations and/or in the creation of new IMO PS and/or “product specifications”, as appropriate. Hence, the envisaged architectural setup has relevance for the present “gap analysis” at IMO e-Nav Correspondence Group.

# References

The attached drawing is in keeping with the architectural principles regarding “products” as described in the input documents for the “e-Navigation architecture” from intersessional work of WG 5 (agenda item 11 refers). These documents should be consulted for a broader view of the context.

# Action requested of the Committee

The Committee is requested to consider the architectural principles depicted in the attachment:

- as applicable to the PNT domain;

- as applicable to other functional domains, as indicated by examples in the drawing, such as COM, AIS (NG) etc. The Committee may wish to identify a tentative list of envisaged “INS modules” which would cover the needs of interaction of IALA members (and other shore-based stakeholders) with defined and supportive shipboard equipment;

- as relevant input for the IALA contribution to the “gap analysis” of the IMO e-Navigation process;

- as relevant input for any IALA contribution on the proposed integrated PNT System to IMO.

- The Committee is requested to absorb the architectural principles in their work, where applicable and as appropriate.

- Attachment: