| From: EEP | e-NAV9/10/4 Formerly EEP15/output/5 |
| --- | --- |
| To: ANM, e-NAV | 8 Oct 2010 |

Liaison Note

Visible Light Communications Technology

# Introduction

The EEP committee, working group 4 (Light and Vision), was asked to consider an input to EEP15 (EEP15/11/5).  This input from the Japan Coast Guard (JCG) was a paper entitled ‘Simulation for Establishment of Visible Light Communications Technology.’

# Study of Visible Light Communications Technology

JCG are part of a consortium (Visible Light Communications Consortium (VLCC)) which has undertaken to study data communications using the medium of light.  The consortium consists of JCG, Toshiba, Casio and NEC.

The data communications considered in the paper employ the use of the actual AtoN signal light to transmit data.  This is achieved by the modulation of the AtoN light at frequencies higher than the flicker fusion frequency of human vision.  This means that the light “flickers,” but the flickering is not perceptible to human vision.  This is similar to Fibre-Optic Communication, but without the fibre!

Some benefits of this system include:

- There are no frequency band regulations for visible light

- The source of the transmitted data is easily located for use

- Secure (difficult to access data without specialist equipment)

- Any light (domestic, workplace, etc) could be used to set up a network

- Harmless to the human body and to electronic devices

The studies have looked at data gathering from both empirical and simulated sources, but the thrust of the input document is on the simulation of wave action on the ability of a vessel to communicate using the medium of light.

The paper shows that there is sufficient evidence that communicating in this way is possible under “slight” sea-state conditions.  The VLCC intends to carry out further investigations into the affect of roll, pitch and yaw on data communications via light.

Of, probably, more important note is the preamble to the JCG paper which notes previous VLCC studies which consider the data rates of “long-distance” light communications.  Results as follows:

- Over a distance of 2km, data rates of 1,022bs-1 were achieved

- Over a distance of 1km, data rates of 1,200bs-1 were achieved

To achieve these results, both transmitter and receiver were permanently fixed to shore-based structures.  Further consultation with the author discovered that these results had only been achieved once and not repeated.

Circulating water tank simulation test results showed that communications were possible over a distance of 390m for the purposes of monitoring AtoN (the original purpose of the study); however, the probability of satisfactory inter-ship communication was quite low.

# Action requested

The ANM & e-NAV Committees are requested to note the information provided and specifically:

- The ANM Committee is requested to consider whether visible light communications technology would be of future use?

- The e-NAV Committee is requested to consider whether visible light communications technology would be a relevant medium for use in the e-Navigation communications architecture?