AISM Association Internationale de Signalisation Maritime     IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

Information paper on

IALA Recommendation

e-NAV-140

On

Shore-based

e-Navigation Architecture

Edition 2

[December 2011]

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01    Fax: +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

Information Paper on the draft
IALA Recommendation e-NAV 140 on the e-Navigation Architecture – the shore-based perspective (Ed 2)

Shore-based e-Navigation Architecture

# Introduction

This document describes in short the e-Navigation architecture with focus on a shore-based perspective. The document will describe step by step how the current e-Navigation Architecture was derived from the high level concept.

The document introduces important principles of the e-Navigation architecture. Further the document establishes relevant principles for a shore-based system architecture harmonized for e-Navigation.  A more elaborate explanation of the architecture is given in the e-NAV 140 document.

# The overarching e-Navigation architecture

## The most generic architecture of the e-Navigation – “the three sides of the coin”

Taking into account the very definition of e-Navigation a rudimentary but still meaningful overarching e-Navigation architecture can be devised. It is represented as the “three sides of the coin” and illustrated in Figure 1. This Figure is a graphical representation of the IMO statement of the “vision of e-Navigation” (IMO MSC85/25, Annex 20, section 4, refers).

To enable true e-Navigation all sides must be present, and operating.

- Rudimentary representation of overarching e-Navigation architecture – the “three sides of the coin”: shipboard, shore-based and links in-between

“harmonized collection, integration, exchange, presentation and analysis of maritime information

onboard”

“harmonized collection, integration, exchange, presentation and analysis of maritime information ashore”

### The overarching e-Navigation architecture

A more elaborate representation of the overarching e-Navigation architecture is shown in Figure 2.  The shipboard entities, the physical link(s) and the shore-based entities are included in this representation of the overarching e-Navigation architecture.

- Overarching e-Navigation Architecture as defined by IMO NAV e-Navigaton Correspondence Group (NAV56/8, para 29, April 2010)

IMO has identified a “common data structure”. In Figure 2 this is introduced as the core of e-Navigation. The scope of this “common data structure” is confined to the maritime domain, hence Common Maritime Data Structure (CMDS). The CMDS will serve as a common reference for all implementers of e-Navigation and thereby accommodate for a certain of harmonization.

### Information-flow-oriented graphical representation of the overarching e-Navigation architecture

Figure 2 does not yet convey the full notion of the e-Navigation concept, however: When considering the e-Navigation architecture, one should think in terms of information/data flow, application interactions, and user interfaces. It is therefore necessary to refine that figure into an information/data flow oriented graphical representation.

Figure 3 shows the principal of an information/data flow in the e-Navigation architecture while the structural details of both the technical shipboard and shore-based e-Navigation system architectures are not yet shown. This brings into focus the “operational service” level and the “Functional links used by Technical services” and the “Physical links used by Technical services”.

- The e-Navigation architecture – simplified presentation

Figure 3 further develops Figure 2 by:

- mapping the fundamental distinction between information and data domains.

- explaining the relationship between the user requested information items.

- putting the concepts of Operational Services, Technical Services as well as Functional and Physical Links into a hierarchical perspective.

- identifying the place of the IMO introduced concept of “Maritime Service Portfolio”.

- unfolding the relationship of shore-to-shore data exchange.

The complete e-Navigation architecture is shown in Figure 4 overleaf. This introduces two additional important features of the overarching e-Navigation architecture:

- The Common Maritime Data Structure (CMDS) spans the whole of the horizontal axis.

- The World Wide Radionavigation System (WWRNS).

- The overarching e-Navigation architecture – complete presentation

### The IMO governed Common Maritime Data Structure (CMDS) at the core of e-Navigation

The scope of the “common data structure” is confined to the maritime domain. Hence, the CMDS will serve as a common reference for all implementers and thereby accommodate for a certain of harmonization.

The functional relationships of the Common Maritime Data Structure are introduced in Figure 5 overleaf:

- The CMDS can represent any maritime entity, can be extended by the addition of new entities, and is accessible to any stakeholder or implementer.

- The CMDS is an abstract representation of parts of the maritime domain.  Specifically, it represents the entities and relationships among the entities that exist in this domain but does not represent processes.

- The CMDS contains no details about physical representation of the entities within it.  However, the CMDS can be used to guide the development of databases and interfaces.

- The CMDS is flexible and extendable for meeting future requirements.  New entities can be added by any stakeholder through a process known as registration.

Figure 5 illustrates how the IMO CMDS will be created – based on user requirements – and how it will influence the components of the e-Navigation architecture when used by (software) engineers.

IMO

governed

Common Maritime Data Structure (CMDS)

Ship

technology environment harmonized for e-Navigation

Shore-based

systems harmonized for e-Navigation

User requirements

User requirements

Templates + methodology

Templates + methodology

Encoding for

Physical Links

- Scope and Impact of the proposed IMO governed Common Maritime Data Structure (CMDS)

### The co-operation between IHO and IALA regarding IMO’s intended Common Maritime Data Structure

Recognizing the need for a registry, IALA and IHO are currently exploring the possibilities of using the GI Registry, operated by IHO. It is considered to be flexible and extendable for meeting future requirements.  New entities can be added by any stakeholder through a process known as registration.  Once registered, an entity is available to all stakeholders; there is open, public access. IALA will manage a separate domain within the GI Registry called IALA Domain.

Figure 6 shows the GI Registry including the IALA domain.

- The representation of the IHO GI Registry, including the IALA domain.

# the Common Shore-Based System Architecture

The shore-based systems of IALA National Members are built to serve similar purposes and to perform similar tasks in particular in the realms of Aids-to-Navigation and VTS. Therefore, already today they have similar architectures. By standardizing the architecture of IALA National Members’ shore-based systems benefits could be gained.

An additional impact is created by the globalization of the system architecture by the e-Navigation concept. E-Navigation will require as a key strategic element internationally harmonized system architecture.

Therefore, it follows that IALA National Members’ shore-based systems should have a common architecture, which would comply with the e-Navigation concept, i.e. the Common Shore-based System Architecture (CSSA). It is introduced in this section and illustrated by Figure 7 overleaf.

In Figure 7 the following are shown:

- Interfaces between entities.

- Technical systems symbolised by cubes (black boxes);

- Human-Machine-Interfaces (HMI), and Machine-Machine-Interfaces (M2M).

The CSSA is further detailed in a dedicated [future] IALA Recommendation.

- The e-Navigation architecture with focus on the shore-side (simplified representation; i.e. without CMDS and WWRNS)

# the IALA e-Navigation Stack

The IALA e-Navigation Stack is an internationally harmonized/unified layer modelling for the shore-side, specifically developed for e-Navigation. The idea of the stack is to represent the different working areas of IALA regarding e-Navigation in a layered model. It follows the philosophy of the Open System layered model, providing a seamless framework for IALA’s work for the shore side starting top from User Requirements down to the shore-based system architecture and the specific technical e-Navigation services.

This section shows the place of the “IALA e-Navigation stack” within the overarching e-Navigation architecture. Table 1 provides an overview of the “IALA e-Navigation Stack”.

| Acronym of layer | Name of layer | Name of sub-layer |
| --- | --- | --- |
| Maritime Transportation Processes (incl. Logistics Chain) |  | N/A |
| IMO User Needs |  | N/A |
| ISHR | IALA Stakeholders Harmonized User Requirements |  |
| IALA UOPS | Unified Operational Presentation Surface |  |
| IHDM | IALA’s contribution to the IMO governed Common Maritime Data Structure (CMDS) | Data properties definitions |
|  |  | Data objects definitions |
| IALA MDEF | IALA Maritime Data Exchange Format | Encoding-free sentence definitions |
|  |  | Technology specific encoded sentences |
| IALA CSSA | IALA Common Shore-Based System Architecture Technology-specific level | Generic part of CSSA |
|  |  | Technology-specific part of CSSA |
| National / regional adaptation in their appropriate procurement documentation |  |  |
| Contractor / industry implementation |  |  |

- Simplified representation of the “IALA e-Navigation stack” (highlighted by double lines) within its context

## The context of the IALA e-Navigation stack within the wider context of IMO’s e-Navigation strategy

Figure 8 shows the place of the IALA e-Navigation stack within the overarching e-Navigation architecture in general. It also shows, to which part of the overarching e-Navigation architecture the layers correspond.

- The place of the IALA e-Navigation stack and its layers within the overarching e-Navigation architecture

## The relationship between the IALA e-Navigation Stack and the IHO GI Registry

As introduced previously a register is needed to host data objects, portrayal aspects etc. The relationship between this register and the previously introduced e-Navigation stack is depicted in
Figure 9. This figure shows which part of the IALA e-Navigation stack is incorporated in the IHO’s GI Registry (based on IHO S-100/S-99).

- The relationship between  the IHO GI Registry and the IALA e-Navigation stack

## The relationship of the concept of “products” with the IALA e-Navigation stack

A “product” is understood as a technical or operational service provided by an IALA member to its user(s), in particular to the mariners and to the maritime community at large.

The notion of “products” is beneficial for the future work of IALA and of IALA membership as it helps focus IALA’s and IALA membership’s efforts, ties them in with the emerging Maritime Service Portfolio concept, and ties them in with business process and business use case considerations.

A “product” is composed by a variety of contributions of the different layers of the IALA e-Navigation stack in accordance with a rule base given by a “product specification”. The “product specifications” can be kept within dedicated documents (as is use in IALA); they could also be kept in a register (as is the case with the IHO GI Registry). By the same token, different “product specifications” can draw upon the same contributions of the layers.

Figure 6 shows the relative position of “products”.

## The contribution of IALA’s “products” to the Maritime Service Portfolio

A “Maritime Service Portfolio” defines and describes the set of operational and technical services and their level of service provided by an IALA member (and other stakeholders) in a given sea area, waterway, or port, as appropriate.

Hence, a “Maritime Service Portfolio” can also be construed as a set of “products” provided by an IALA member (and other stakeholders) in a given sea area, waterway, or port, as appropriate.

# Closing remark

The aim of this document was to give the reader an insight in the way the e-Navigation architecture was derived from the (by IMO) stated goals. To arrive at the point that the e-Navigation Architecture can be implemented a lot of decisions have to be taken and a paradigm shift is needed in order to achieve the set goals.